3GPP TSG-RAN WG4 Meeting #114							        	 		R4-2502067

Athens, GR, 17th – 21st February 2025

Title: 	Measurement results for PC1.5 intra-band contiguous ULCA

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.1.1.2	Intra-band contiguous and non-contiguous UL CA with PC1.5	[NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In R4#112bis we provided a preliminary set of measurement results [1] for PC1.5 contiguous intra-band ULCA. In this contribution, we provide further measurements, albeit incomplete, which does not enable to conclude on the MPR values.

# Discussion

## 2.1 Measurement results

The measured data is provided in the annex and is the raw data without correction by re-referencing to the PC2 specified back-off. It also only contains data with equal PSD at the input (note that PSD at the output may not be equal due to the PA bandwidth).

As such, the results are incomplete since the same measurements for 2Tx PC2 are needed to allow the establishment of the delta back-off, and some measurements with PSD imbalance would allow the assessment of the margin needed for up to a 6dB imbalance.

Those measurements were planned for this meeting, but due to the fires in the Los Angeles area the measurements have seen many interruptions. The results already obtained needed more than one week of full-time measurements without interruptions.

Furthermore, for back-off above 6dB, additional measurements are needed with lower voltage on the PA to mimic what happens for ET and APT PAs at moderate back-off. This means that back-offs in the 6-7.5dB range in the current set of measurements probably need a 7-9dB back-off.

The measurement is for two CCs of 80MHz with 2Tx and PAs calibrated for 31dB ACLR for 20MHz QPSK DFT-s-OFDM 100RB0 waveforms at 1dB MPR and assuming 3dB post PA losses and 10dB antenna coupling. Note that the measurements only look at out-of-band requirements and thus the inner back-off may need to be checked for IBE and EVM (per CC).

The measurements were only performed when allocation was present in each CC since in R19 we agreed that the single CC MPR applies when there are RB allocated in only one CC.

Due to outside circumstances, we were not able to perform all the measurements we thought were needed to set the proper MPR. We are open to discuss tentative values in this meeting to be further verified by additional measurements.

## 2.2 Observations and deducted range from measurements.

The current measurement levels allow the following observations:

- For contiguous inner allocations, the worst back-off is 0.9dB for CP-OFDM and 0.3dB for DFT-s-OFDM. These measurements did not verify EVM and IBE.

  - Allocations <77MHz BW do not need any back-off (for OOB emissions)

  - Allocations >77MHz are ACLR limited. Since PC1.5 and PC2 ACLR are the same and relative, it can be anticipated that the same MPR as PC2 2Tx should apply.

- For contiguous outer allocations, the worst back-off is 4.5dB for CP-OFDM and 3.8dB for DFT-s-OFDM and are ACLR limited.

- For non-contiguous inner allocations, the worst back-off is 2.9dB for CP-OFDM and 2.2dB for DFT-s-OFDM. These are limited by IMD5 falling in the -10dBm/MHz SEM region.

  - Allocation with RBBW >2MHz do not need back-off, for OOB emissions.

  - It is possible that 15kHz SCS 1RB+1RB would require slightly higher MPR and needs to be verified.

- For non-contiguous outer1 allocations, the worst back-off is 6.5dB for CP-OFDM and 6.0dB for DFT-s-OFDM. These are limited by IMD3 falling in the -13dBm/MHz SEM region.

  - With approximately 6dB back-off, the back-off should be verified at a lower PA supply voltage to account for the APT or ET operation in real products. One can anticipate needing a 1-2dB higher back-off.

  - It is possible that 15kHz SCS 1RB+1RB would require slightly higher MPR and needs to be verified.

- For non-contiguous outer2 allocations, the worst back-off is 7.5dB for CP-OFDM and 6.6dB for DFT-s-OFDM. These are limited by IMD3 falling in the -13dBm/MHz SEM region.

  - With >6dB back-off, the back-off should be verified at a lower PA supply voltage to account for the APT or ET operation in real products. One can anticipate needing a 2-3dB higher back-off.

  - It is possible that 15kHz SCS 1RB+1RB would require slightly higher MPR and needs to be verified.

  - Also, the outer2 allocation benefit from the PA roll-off due to the large BW for the IMDs (IMD3 has 3x160MHz=480MHz range and IMD5 has a 5x160MHz=800MHz range) and thus partially attenuates them. The same outer2 should be verified for the lowest class C aggregated bandwidth of 105MHz.

With the observations above we can only anticipate the range to be considered for MPR for the different allocations. This requires thorough verifications of the points expressed in the above observations.

Proposal: Until we have done the following verifications, we will only accept tentative values for PC1.5 contiguous intra-band ULCA MPR:

- Recalibrate our PC1.5 measurements with 2Tx PC2 measurements so that we can provide delta MPR values.

- For the cases with back-off >6dB, re-measure at a lower PA voltage representative of the true PA operation in APT or ET.

- Verify a few 1RB+1RB cases with 15kHz SCS for non-contiguous allocations.

- Verify non-contiguous outer 2 allocations for the smallest class C BW of 110MHz to ensure that PA BW does not attenuate IMDs for the largest class C BW.

- Assess margin needed for up to 6dB PSD imbalance (at PA input).

# Conclusion

In this contribution, we further provided a larger batch of measurements for PC1.5 contiguous intra-band ULCA, unfortunately our measurements have been subject to too many interruptions outside our control and are still too partial to allow a solid MPR proposal.

Proposal: Until we have done the following verifications, we will only accept tentative values for PC1.5 contiguous intra-band ULCA MPR:

- Recalibrate our PC1.5 measurements with 2Tx PC2 measurements so that we can provide delta MPR values.

- For the cases with back-off >6dB, re-measure at a lower PA voltage representative of the true PA operation in APT or ET.

- Verify a few 1RB+1RB cases with 15kHz SCS for non-contiguous allocations.

- Verify non-contiguous outer 2 allocations for the smallest class C BW of 110MHz to ensure that PA BW does not attenuate IMDs for the largest class C BW.

- Assess margin needed for up to 6dB PSD imbalance (at PA input).

# References

[1] R4-2415644 Preliminary input on 2Tx PC1.5 contiguous ULCA MPR based on measurements, Skyworks Solutions Inc., R4#112bis

[2] R4-2420329 WF on HPUE RF requirements, Samsung, R4#113

# Annex Raw measured data

The tables provide the following information:

- The first 5 column provide:

  - Allocation type with CC1 for contiguous inner, CCO for contiguous outer, NCI for non-contiguous inner, NO1 for non-contiguous outer1 and NO2 for non-contiguous outer2.

  - Number of RB (#RB1) and RBstart (RB#1) for CC1 then number of RB  (#RB1) and RBstart for CC2 (RB#1)

- The next 5 columns provide the RB BW, TXBW and gap between each CC allocation: RBBW is the total RB allocation BW, RBBW1 is the RB BW for CC1, RBBW2 is the RB BW for CC2, TXBW is the transmission BW (the bandwidth between the lowest and highest RB) and the gap is the BW for the gap between the highest RB in CC1 and the lowest RB in CC2.

- The next 4 columns are the CP-OFDM measurement results with:

  - For a positive value, BO is for the back off needed to meet all emissions requirements. A negative value provides the smallest margin at Pmax

  - The limit column provides which emission requirement is dictating the back-off (or has the lowest margin): ac is ACLR, s4 is the spurious region at -30dBm/MHz, s3 is the SEM region at -25dBm/MHz, s2 is the SEM region at -13dBm/MHz, s1 is the SEM region at -10dBm/MHz and s0 is the SEM limit in the first MHz.

  - The source column is the source of the emissions: floor is for the noise floor of the setup (only when there is already margin a Pmax), i3/i5 is for IMD3/5 respectively of the allocations in each CC.

  - Finally, the PSD column provides the PSD imbalance between the two CCs at the output (while the input waveforms are with equal PSD)

- The last 4 columns are the DFT-s-OFDM measurement results with the same parameters.

Contiguous allocations measurements (25 allocations measured)

| Waveform type and allocation |  |  |  |  |  |  |  |  |  | CP-OFDM |  |  |  | DFT-s-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| type | RB#1 | #RB1 | RB#2 | #RB2 | RBBW | RBW1 | RBW2 | TBW | Gap | BO | limit | source | PSDout | BO | limit | source | PSDout |
| CCI | 002 | 215 | 002 | 000 | 1.44 | 0.72 | 0.72 | 3.30 | 1.86 | -12.24 | s4 | floor | 0.00 | -12.27 | s4 | floor | 0.10 |
| CCI | 004 | 213 | 004 | 000 | 2.88 | 1.44 | 1.44 | 4.74 | 1.86 | -12.24 | s4 | floor | 0.40 | -12.21 | s4 | floor | 0.20 |
| CCI | 008 | 209 | 008 | 000 | 5.76 | 2.88 | 2.88 | 7.62 | 1.86 | -12.23 | s4 | floor | 0.10 | -12.18 | s4 | floor | 0.10 |
| CCI | 016 | 201 | 016 | 000 | 11.52 | 5.76 | 5.76 | 13.38 | 1.86 | -12.20 | s4 | floor | 0.20 | -12.27 | s4 | floor | 0.30 |
| CCI | 027 | 190 | 027 | 000 | 19.44 | 9.72 | 9.72 | 21.30 | 1.86 | -12.21 | s4 | floor | 0.10 | -12.16 | s4 | floor | 0.30 |
| CCI | 054 | 163 | 054 | 000 | 38.88 | 19.44 | 19.44 | 40.74 | 1.86 | -8.17 | ac | i5 | 0.50 | -12.02 | ac | i5 | 0.30 |
| CCI | 108 | 109 | 054 | 000 | 58.32 | 38.88 | 19.44 | 60.18 | 1.86 | -4.26 | ac | i3 | 4.30 | -3.86 | ac | i3 | 0.70 |
| CCI | 108 | 109 | 108 | 000 | 77.76 | 38.88 | 38.88 | 79.62 | 1.86 | 0.90 | ac | i3 | 0.60 | 0.30 | ac | i3 | 0.60 |
| CCO | 109 | 108 | 109 | 000 | 78.48 | 39.24 | 39.24 | 80.34 | 1.86 | 1.00 | ac | i3 | 0.60 |  |  |  |  |
| CCO | 120 | 097 | 129 | 000 | 89.64 | 43.20 | 46.44 | 91.50 | 1.86 | 2.60 | ac | i3 | 0.70 |  |  |  |  |
| CCO | 216 | 001 | 108 | 000 | 116.64 | 77.76 | 38.88 | 118.50 | 1.86 | 3.40 | ac | i3 | 0.60 | 2.10 | ac | i3 | 0.60 |
| CCO | 216 | 001 | 216 | 000 | 155.52 | 77.76 | 77.76 | 157.38 | 1.86 | 4.50 | ac | i3 | 0.90 | 3.80 | ac | i3 | 1.00 |
| CCO | 217 | 000 | 217 | 000 | 156.24 | 78.12 | 78.12 | 158.10 | 1.86 | 4.60 | ac | i3 | 0.90 |  |  |  |  |

Non-contiguous allocations measurements (153 allocations measured)

| Waveform type and allocation |  |  |  |  |  |  |  |  |  | CP-OFDM |  |  |  | DFT-s-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| type | RB#1 | #RB1 | RB#2 | #RB2 | RBBW | RBW1 | RBW2 | TBW | Gap | BO | limit | source | PSDout | BO | limit | source | PSDout |
| NCI | 001 | 146 | 001 | 070 | 0.72 | 0.36 | 0.36 | 52.98 | 52.26 | 2.90 | s1 | i5 | 0.90 | 2.20 | S1 | i5 | 0.80 |
| NCI | 002 | 147 | 001 | 071 | 1.08 | 0.72 | 0.36 | 52.98 | 51.90 | -1.71 | s2 | i5 | 1.00 | -4.42 | s2 | i5 | 0.60 |
| NCI | 002 | 147 | 002 | 070 | 1.44 | 0.72 | 0.72 | 52.98 | 51.54 | 0.30 | s2 | i5 | 0.80 | -3.55 | s2 | i5 | 1.10 |
| NCI | 004 | 149 | 002 | 071 | 2.16 | 1.44 | 0.72 | 52.62 | 50.46 | -2.80 | s2 | i5 | 0.80 | -7.94 | s2 | i5 | 0.90 |
| NCI | 004 | 149 | 004 | 069 | 2.88 | 1.44 | 1.44 | 52.62 | 49.74 | -2.17 | s2 | i5 | 0.70 | -5.99 | s2 | i5 | 0.80 |
| NCI | 008 | 151 | 004 | 070 | 4.32 | 2.88 | 1.44 | 52.26 | 47.94 | -5.54 | s2 | i5 | 0.80 | -8.40 | ac | i5 | 1.40 |
| NCI | 008 | 151 | 008 | 066 | 5.76 | 2.88 | 2.88 | 52.26 | 46.50 | -2.66 | s2 | i5 | 0.90 | -4.74 | s2 | i5 | 0.90 |
| NCI | 016 | 157 | 008 | 069 | 8.64 | 5.76 | 2.88 | 51.18 | 42.54 | -5.60 | ac | i5 | 0.70 | -7.53 | ac | i5 | 0.80 |
| NCI | 016 | 157 | 016 | 060 | 11.52 | 5.76 | 5.76 | 50.82 | 39.30 | -3.47 | ac | i5 | 0.70 | -4.50 | ac | i5 | 0.80 |
| NCI | 027 | 164 | 008 | 072 | 12.60 | 9.72 | 2.88 | 49.74 | 37.14 | -7.72 | ac | i5 | 0.70 | -8.40 | ac | i5 | 1.10 |
| NCI | 027 | 164 | 027 | 053 | 19.44 | 9.72 | 9.72 | 49.74 | 30.30 | -4.55 | ac | i5 | 0.60 | -4.67 | ac | i5 | 0.80 |
| NCI | 054 | 163 | 001 | 079 | 19.80 | 19.44 | 0.36 | 50.10 | 30.30 | -12.17 | s4 | floor | -3.90 | -12.27 | s4 | floor | 0.80 |
| NCI | 054 | 163 | 054 | 026 | 38.88 | 19.44 | 19.44 | 50.10 | 11.22 | -0.63 | ac | i5 | 0.60 | -7.52 | ac | i5 | 0.50 |
| NO1 | 001 | 087 | 001 | 130 | 0.72 | 0.36 | 0.36 | 95.82 | 95.10 | 6.50 | S2 | I3 | 1.10 | 6.00 | S2 | I3 | 1.40 |
| NO1 | 001 | 145 | 001 | 070 | 0.72 | 0.36 | 0.36 | 53.34 | 52.62 | 4.00 | S2 | I5 | 1.00 | 2.90 | s0 | I3 | 1.00 |
| NO1 | 002 | 087 | 001 | 130 | 1.08 | 0.72 | 0.36 | 95.82 | 94.74 | 6.00 | S2 | I3 | 1.30 | 5.90 | S2 | I3 | 0.80 |
| NO1 | 002 | 088 | 002 | 129 | 1.44 | 0.72 | 0.72 | 95.46 | 94.02 | 6.10 | S2 | I3 | 1.10 | 6.00 | S2 | I3 | 1.10 |
| NO1 | 002 | 146 | 001 | 071 | 1.08 | 0.72 | 0.36 | 53.34 | 52.26 | 3.00 | s0 | i3 | 1.10 | 2.00 | s0 | i3 | 1.00 |
| NO1 | 002 | 146 | 002 | 070 | 1.44 | 0.72 | 0.72 | 53.34 | 51.90 | 2.40 | s0 | i3 | 0.70 | 1.70 | s0 | i3 | 0.80 |
| NO1 | 004 | 089 | 002 | 130 | 2.16 | 1.44 | 0.72 | 95.46 | 93.30 | 4.60 | s2 | i3 | 1.10 | 4.50 | s2 | I3 | 1.00 |
| NO1 | 004 | 089 | 004 | 128 | 2.88 | 1.44 | 1.44 | 95.46 | 92.58 | 5.00 | s2 | i3 | 1.20 | 4.60 | s2 | i3 | 1.50 |
| NO1 | 004 | 148 | 002 | 071 | 2.16 | 1.44 | 0.72 | 52.98 | 50.82 | -2.04 | s0 | i3 | 0.80 | -0.30 | s0 | i3 | 1.00 |
| NO1 | 004 | 148 | 004 | 069 | 2.88 | 1.44 | 1.44 | 52.98 | 50.10 | -1.41 | s2 | i5 | 0.70 | -5.86 | s2 | i5 | 0.80 |
| NO1 | 008 | 092 | 004 | 130 | 4.32 | 2.88 | 1.44 | 95.10 | 90.78 | 3.80 | s2 | i3 | 1.20 | 3.20 | s2 | i3 | 1.50 |
| NO1 | 008 | 092 | 008 | 125 | 5.76 | 2.88 | 2.88 | 94.74 | 88.98 | 4.70 | ac | i3 | 1.10 | 3.90 | ac | i3 | 1.40 |
| NO1 | 008 | 150 | 004 | 070 | 4.32 | 2.88 | 1.44 | 52.62 | 48.30 | -5.16 | s2 | i5 | 0.70 | -8.26 | ac | i5 | 1.20 |
| NO1 | 008 | 150 | 008 | 067 | 5.76 | 2.88 | 2.88 | 52.98 | 47.22 | -2.26 | s0 | i3 | 0.90 | -3.46 | s0 | i3 | 1.10 |
| NO1 | 016 | 096 | 008 | 129 | 8.64 | 5.76 | 2.88 | 94.74 | 86.10 | 3.60 | ac | i3 | 1.00 | 2.90 | ac | i3 | 1.10 |
| NO1 | 016 | 096 | 016 | 121 | 11.52 | 5.76 | 5.76 | 94.74 | 83.22 | 4.50 | ac | i3 | 1.00 | 3.50 | ac | i3 | 1.20 |
| NO1 | 016 | 156 | 008 | 069 | 8.64 | 5.76 | 2.88 | 51.54 | 42.90 | -5.30 | ac | i5 | 0.70 | -7.78 | ac | i5 | 0.90 |
| NO1 | 016 | 156 | 016 | 061 | 11.52 | 5.76 | 5.76 | 51.54 | 40.02 | -3.23 | ac | i5 | 0.80 | -4.72 | ac | i5 | 0.80 |
| NO1 | 027 | 103 | 008 | 133 | 12.60 | 9.72 | 2.88 | 93.66 | 81.06 | 2.50 | AC | I3 | 0.90 | 1.90 | ac | i3 | 1.00 |
| NO1 | 027 | 103 | 027 | 114 | 19.44 | 9.72 | 9.72 | 93.66 | 74.22 | 4.20 | AC | I3 | 1.00 | 4.00 | ac | i3 | 1.40 |
| NO1 | 027 | 163 | 008 | 072 | 12.60 | 9.72 | 2.88 | 50.10 | 37.50 | -7.74 | ac | i5 | 0.20 | -8.44 | ac | i5 | 0.80 |
| NO1 | 027 | 163 | 027 | 053 | 19.44 | 9.72 | 9.72 | 50.10 | 30.66 | -4.60 | ac | i5 | 0.70 | -4.62 | ac | i5 | 1.00 |
| NO1 | 054 | 119 | 001 | 151 | 19.80 | 19.44 | 0.36 | 91.86 | 72.06 | 0.80 | ac | i3 | -3.20 | -4.55 | ac | i3 | 1.60 |
| NO1 | 054 | 119 | 054 | 098 | 38.88 | 19.44 | 19.44 | 91.86 | 52.98 | 6.20 | ac | i3 | 1.00 | 3.20 | ac | i3 | 1.00 |
| NO1 | 054 | 162 | 001 | 079 | 19.80 | 19.44 | 0.36 | 50.46 | 30.66 | -12.23 | s4 | floor | -3.80 | -12.21 | s4 | floor | 1.00 |
| NO1 | 054 | 162 | 054 | 026 | 38.88 | 19.44 | 19.44 | 50.46 | 11.58 | -0.47 | ac | i5 | 0.60 | -7.51 | ac | i5 | 0.60 |
| NO1 | 108 | 109 | 001 | 144 | 39.24 | 38.88 | 0.36 | 92.94 | 53.70 | -3.84 | ac | i3 | -1.30 | -8.06 | ac | i3 | 0.50 |
| NO1 | 108 | 109 | 108 | 037 | 77.76 | 38.88 | 38.88 | 92.94 | 15.18 | 3.50 | ac | i3 | 0.70 | 2.30 | ac | i3 | 0.70 |
| NO1 | 129 | 088 | 129 | 002 | 92.88 | 46.44 | 46.44 | 95.46 | 2.58 | 4.70 | ac | i3 | 0.80 |  |  |  |  |
| NO1 | 216 | 001 | 001 | 001 | 78.12 | 77.76 | 0.36 | 80.34 | 2.22 | 3.40 | ac | i3 | -0.30 | 1.50 | ac | i3 | 0.00 |
| NO2 | 001 | 000 | 001 | 216 | 0.72 | 0.36 | 0.36 | 158.10 | 157.38 | 6.30 | S4 | I5 | 1.60 | 5.70 | S4 | I5 | 1.90 |
| NO2 | 001 | 086 | 001 | 130 | 0.72 | 0.36 | 0.36 | 96.18 | 95.46 | 6.80 | S2 | I3 | 1.10 | 6.60 | S2 | I3 | 0.80 |
| NO2 | 002 | 000 | 001 | 216 | 1.08 | 0.72 | 0.36 | 158.10 | 157.02 | 4.60 | S4 | I5 | 1.70 | 4.50 | S4 | I5 | 2.40 |
| NO2 | 002 | 000 | 002 | 215 | 1.44 | 0.72 | 0.72 | 158.10 | 156.66 | 5.80 | S4 | I5 | 1.50 | 5.30 | S4 | I5 | 1.80 |
| NO2 | 002 | 086 | 001 | 130 | 1.08 | 0.72 | 0.36 | 96.18 | 95.10 | 6.10 | S2 | I3 | 1.10 | 6.00 | S2 | I3 | 1.40 |
| NO2 | 002 | 087 | 002 | 129 | 1.44 | 0.72 | 0.72 | 95.82 | 94.38 | 6.20 | S2 | I3 | 1.10 | 6.00 | S2 | I3 | 0.80 |
| NO2 | 004 | 000 | 002 | 215 | 2.16 | 1.44 | 0.72 | 158.10 | 155.94 | 4.50 | s4 | i5 | 1.80 | 4.00 | s4 | i5 | 1.30 |
| NO2 | 004 | 000 | 004 | 213 | 2.88 | 1.44 | 1.44 | 158.10 | 155.22 | 5.30 | s4 | i5 | 1.60 | 4.60 | s4 | i5 | 1.80 |
| NO2 | 004 | 088 | 002 | 130 | 2.16 | 1.44 | 0.72 | 95.82 | 93.66 | 5.00 | s2 | i3 | 1.20 | 4.50 | s2 | i3 | 1.10 |
| NO2 | 004 | 088 | 004 | 129 | 2.88 | 1.44 | 1.44 | 96.18 | 93.30 | 5.30 | s2 | i3 | 1.30 | 4.40 | s2 | i3 | 1.30 |
| NO2 | 008 | 000 | 004 | 213 | 4.32 | 2.88 | 1.44 | 158.10 | 153.78 | 3.80 | s2 | i3 | 1.80 | 3.20 | s4 | i5 | 2.00 |
| NO2 | 008 | 000 | 008 | 209 | 5.76 | 2.88 | 2.88 | 158.10 | 152.34 | 4.40 | AC | I3 | 1.70 | 4.40 | s4 | i5 | 1.30 |
| NO2 | 008 | 091 | 004 | 130 | 4.32 | 2.88 | 1.44 | 95.46 | 91.14 | 4.00 | AC | i3 | 1.20 | 2.90 | s2 | i3 | 1.60 |
| NO2 | 008 | 091 | 008 | 126 | 5.76 | 2.88 | 2.88 | 95.46 | 89.70 | 4.80 | AC | i3 | 1.10 | 4.00 | ac | i3 | 1.00 |
| NO2 | 016 | 000 | 008 | 209 | 8.64 | 5.76 | 2.88 | 158.10 | 149.46 | 4.00 | ac | i3 | 1.80 | 3.10 | ac | i3 | 1.80 |
| NO2 | 016 | 000 | 016 | 201 | 11.52 | 5.76 | 5.76 | 158.10 | 146.58 | 4.50 | ac | i3 | 1.60 | 3.70 | ac | i3 | 1.70 |
| NO2 | 016 | 095 | 008 | 129 | 8.64 | 5.76 | 2.88 | 95.10 | 86.46 | 3.80 | ac | i3 | 1.10 | 2.80 | ac | i3 | 0.90 |
| NO2 | 016 | 095 | 016 | 122 | 11.52 | 5.76 | 5.76 | 95.46 | 83.94 | 4.50 | AC | i3 | 1.10 | 3.80 | ac | i3 | 1.20 |
| NO2 | 027 | 000 | 008 | 209 | 12.60 | 9.72 | 2.88 | 158.10 | 145.50 | 4.20 | ac | i3 | 2.00 | 2.90 | ac | i3 | 2.10 |
| NO2 | 027 | 000 | 027 | 190 | 19.44 | 9.72 | 9.72 | 158.10 | 138.66 | 4.70 | ac | i3 | 1.50 | 4.10 | ac | i3 | 1.70 |
| NO2 | 027 | 102 | 008 | 133 | 12.60 | 9.72 | 2.88 | 94.02 | 81.42 | 2.70 | ac | i3 | 1.00 | 2.10 | ac | i3 | 1.30 |
| NO2 | 027 | 102 | 027 | 114 | 19.44 | 9.72 | 9.72 | 94.02 | 74.58 | 4.10 | ac | i3 | 1.00 | 3.70 | ac | i3 | 1.00 |
| NO2 | 054 | 000 | 001 | 216 | 19.80 | 19.44 | 0.36 | 158.10 | 138.30 | 4.00 | ac | i3 | 1.30 | 2.00 | ac | i3 | 3.00 |
| NO2 | 054 | 000 | 054 | 163 | 38.88 | 19.44 | 19.44 | 158.10 | 119.22 | 6.80 | ac | i3 | 1.50 | 3.90 | ac | i3 | 1.50 |
| NO2 | 054 | 118 | 001 | 151 | 19.80 | 19.44 | 0.36 | 92.22 | 72.42 | 0.90 | ac | i3 | -3.20 | -3.49 | ac | i3 | 1.70 |
| NO2 | 054 | 118 | 054 | 098 | 38.88 | 19.44 | 19.44 | 92.22 | 53.34 | 6.40 | ac | i3 | 1.00 | 3.20 | ac | i3 | 0.90 |
| NO2 | 108 | 000 | 001 | 130 | 39.24 | 38.88 | 0.36 | 127.14 | 87.90 | 4.00 | ac | i3 | 0.90 | 1.50 | ac | i3 | 1.40 |
| NO2 | 108 | 000 | 108 | 109 | 77.76 | 38.88 | 38.88 | 158.10 | 80.34 | 5.20 | ac | i3 | 1.30 | 3.90 | ac | i3 | 1.40 |
| NO2 | 108 | 108 | 001 | 144 | 39.24 | 38.88 | 0.36 | 93.30 | 54.06 | -4.13 | ac | i3 | -1.00 | -7.46 | ac | i3 | 0.70 |
| NO2 | 108 | 108 | 108 | 037 | 77.76 | 38.88 | 38.88 | 93.30 | 15.54 | 3.50 | ac | i3 | 0.70 | 2.60 | ac | i3 | 0.70 |
| NO2 | 216 | 000 | 001 | 001 | 78.12 | 77.76 | 0.36 | 80.70 | 2.58 | 3.80 | ac | i3 | -0.50 | 1.80 | ac | i3 | -0.30 |
| NO2 | 216 | 000 | 001 | 216 | 78.12 | 77.76 | 0.36 | 158.10 | 79.98 | 3.70 | ac | i3 | 1.60 | 1.70 | ac | i3 | 1.90 |
| NO2 | 216 | 000 | 216 | 001 | 155.52 | 77.76 | 77.76 | 158.10 | 2.58 | 4.70 | ac | i3 | 0.90 | 3.80 | ac | i3 | 1.10 |
| NO2 | 129 | 000 | 129 | 088 | 92.88 | 46.44 | 46.44 | 158.10 | 65.22 | 7.50 | ac | i3 | 1.30 |  |  |  |  |
| NO2 | 129 | 088 | 129 | 003 | 92.88 | 46.44 | 46.44 | 95.82 | 2.94 | 4.70 | ac | i3 | 0.70 |  |  |  |  |