3GPP TSG-RAN WG4 Meeting #114	R4-2502070

Athens, Greece, 17th – 21st February 2025

Source:	Nokia

Title:	CSI reporting requirement framework for CSI prediction

Agenda item:	7.19.2

Document for:	Discussion

Introduction

This paper discusses the CSI reporting requirement framework for CSI prediction. The testability and interoperability issues for CSI prediction has been discussed, and the agreements from RAN4#113 are summarize in WF [1]. In this paper, we address the challenges related to testability and LCM requirements for AI/ML-based CSI prediction use cases.

Discussion

On Rel-19 AIML-based CSI reporting

On metric for RAN4 requirements

This section first introduces the performance metric used to define the RAN4 requirements.

In the RAN1 discussions, the metric for evaluation of AIML-based CSI prediction includes SGCS and user throughput (UPT). In RAN4#112, it was agreed that only throughput-based requirement metric will be considered.

Issue 4-5: CSI prediction accuracy metric Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to

Then, an agreement in RAN4#113 was reached on the codebook used to obtain the reference throughput.

Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook.

RAN4 has used a throughput to define a performance metric. In Clause 6.3 of TS 38.101-4, the reporting of PMI has the requirements specified in terms of the ratio γ=tue,follow1,follow 2trnd1, rnd2. For instance, in the legacy (i.e., non-AIML-based) requirements, it is assumed that the multiple PMI with 16 Tx Enhanced Type II codebook is used for predicted PMI (FR1, FDD, 4RX). Under this assumption, the minimum requirements of γ are 1.8 for FDD and 2.5 for TDD, respectively.

In the case of CSI prediction, a similar concept of relative increase in throughput can be used as the performance metric. Consequently, the value of the performance metrics in the legacy requirement should be changed since the AIML-based CSI prediction use case assumes a different testing setting from the legacy use case. Therefore, it is necessary to derive and specify a new value for the performance metric, such as gamma, referring to the relative increase in throughput.

Next, it is essential to discuss the testing scenarios where we calculate the value of the given performance matric to define the requirements. For CSI prediction, RAN1 has defined a baseline scenario where model training uses the UMa channel model according to TR 38.901. Additionally, when the model is located on the UE side, the UE will follow the same CSI reporting format defined in RAN4 specifications. In this case, it is necessary to determine the number of predictions provided by the UE. Therefore, the initial evaluation in RAN4 can start with an N4 value of 1. Any potential values of N4 greater than 1 can be considered in subsequent steps.

RAN4 should start the discussion on the requirements with N4=1.

LCM requirements

The testing goals were discussed and agreed upon in RAN4#112, including the use of LCM to verify the minimum performance of AI/ML functionality/feature.

Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

Additionally, the scope of the LCM discussion was addressed in RAN4#113.

Agreement: - The discussion should consider use-cases specific aspects Agreement: - Whether transition from AI to non-AI would be UE initiated or network initiated - This will be a decision for other groups - RAN4 to assess what RRM requirements are needed for each case.

Moreover, for localized models, RAN1#116 has made the following agreements on testing.

Agreement For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region: - Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901. - E.g., Dropped in a specific cell or within a specific boundary. - Option 2: By using a scenario/configuration specific to the local region. - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

Since CSI prediction relies on a UE-side model, localized models deployed in the field can influence feature performance based on their specific characteristics. Consequently, monitoring these localized models becomes imperative as part of LCM for the deployed model. By associating each model’s ID with tracked performance metrics from the training and deployment phases, it is essential that RAN4 considers LCM actions based on the localization of the models in a given region. This includes studying model-ID-based LCM requirements for the functionality assigned to the UE.

RAN4 to study the model-ID-based LCM requirements if localized models are considered for CSI prediction.

Also, RAN2 has made the agreements related to LCM procedures.

Agreements on procedures from #127: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig. - 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. - 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: - 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI - 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: - 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). - 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality

Additionally, the agreement related LCM procedures in RAN4#111 are as follows:

Agreement: RAN 4 to consider defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning) and candidate are: - Identification - Selection - Activation - Deactivation - Switching - Fallback to non-AI operation - Performance monitoring - Others are not precluded

RAN4 needs to discuss how to manage dynamic and non-static conditions for CSI prediction. Initially, RAN4 can assume the LCM procedures defined in RAN2. For instance, a delay requirement may be needed in the procedure due to the potential processing time of the UE to determine failure or performance degradation. Therefore, RAN4 should also investigate how the RAN2 approach impacts RAN4 requirements.

RAN4 to study delay requirements in Steps 4 and 5 of the LCM procedure based on RAN2’s 5-step approach.

Conclusion

In this contribution, we have discussed the RRM impact of AI/ML-based CSI prediction. The proposals are presented as follows.

- RAN4 should start the discussion on the requirements with N4=1.

RAN4 to study the model-ID-based LCM requirements if localized models are considered for CSI prediction.

RAN4 to study delay requirements in Steps 4 and 5 of the LCM procedure based on RAN2’s 5-step approach.

- References

- R4-2420334, “WF on AI/ML air interface part 1,” RAN4#113, Orlando, USA