# 3GPP TSG-RAN WG4 Meeting # 114						R4-2502098

# Athens, Greece,17th–21st February 2025

Source:		Ericsson

Title:			A-IoT BS RF impact overview

Agenda item:		7.24.2.1

Document for:		Approval

# Introduction

In this paper, we present our overview for A-IoT BS RF requirements.

# Discussion

From the study item phase, RAN1 has agreed the A-IoT R2D waveform is based on OFDM transmitter and is quoted below in TR 38.769.

6.1.1.1	R2D waveform, modulation and numerology

An OFDM-based OOK waveform with subcarrier spacing of 15 kHz is studied for R2D, with OOK-1 for single-chip per OFDM symbol transmission, and OOK-4 for M-chip per OFDM symbol transmission, starting from the definitions in TR 38.869 [4].

The above means that the OFDM transmitter RF architecture can be kept the same for A-IoT BS and from this perspective, most of the legacy BS TX requirement could be kept. RAN1 also decide that the OOK-M signal is defined with he starting point of the LP-WUS signal in TR 38.869. In our view, this also means the OOK-M RF impact in LP-WUS work item can be referenced in the context of the A-IoT BS when transmitting the R2D signal with OOK-M per OFDM symbol transmission.

Legacy OFMD transmitter requirement can be kept mostly and LP-WUS RF impact in LP-WUS work item can be referenced.

In WID objective, it is the BS type 1-C to be defined.

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

With the transmission power definition in 38.104 and coexisting simulation assumption in 38.769, the medium range of type 1-C will be relevant to the specification.

Medium range type 1-C BS is to be specified for A-IoT BS.

Table 6.6.2.2-2: A-IoT reader RF parameters

| A-IoT reader parameters | Values for evaluation |
| --- | --- |
| A-IoT reader total Tx power | 33 dBm |
| A-IoT reader receiver Noise Figure (dB) | 10 |
| A-IoT reader antenna gain (dBi) | 6 |
| ACLR | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dB，ACLR2:50dB) |
| ACS | Same as legacy NR BS |
| Antenna configuration | Antenna Array Geometry：equals to omni-directional antenna pattern in GCG in horizontal Antenna pattern (horizontal): AHϕ=-min12ϕϕ3dB2,Am, φ3dB = 90°, Am = 15 dB Antenna pattern (horizontal): AVθ=-min12θ-90°θ3dB2,SLAv, θ3dB= 90°, SLAv = 15 dB Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss): 6 |

Another issue is that CW transmission by the A-IoT BS and whether it is in the scope of the WID. In 38.769, the CW is co-located with reader and it is up to the RAN4 to decide whether the CW can be transmitted by A-IoT BS also and if so, the RF impact of CW transmission also should be evaluated for A-IoT BS.

| CW deployment | For D1T1-A2 and D2T2-A2, CW and A-IoT reader are collocated. For D1T1-B and D2T2-B, CW topology layout is the same as A-IoT reader. For each device, the nearest CW node will be activated during the simulation, and CW node is not co-located with any activated reader |
| --- | --- |

The RF impact on the CW transmission by A-IoT BS could be evaluated if this is also a valid scenario to consider for Rel-19 work item.

In TR 38.769, it is agreed that RAN4 continue to discuss the transmission ON/OFF power requirement. As this requirement is only for TDD and A-IoT operates in HD-FDD mode, therefore, we think it will be benefit to have ON/OFF requirement other than the protection of the own receiver requirement for normal FDD operation. ON/OFF requirement relates to both emission requirement in OFF time and also the transient time from OFF to ON or ON to OFF, the transient time is crucial to be specified so not to disturb the UL backscatter receiving.

Specify the ON/OFF requirement for A-IoT BS for HD-FDD operation.

It is not discussed the RF impact on OOK-M transmission. Therefore, some of the analysis is illustrated below for what has been done based on the LP-WUS signal design. As the NR signal quality requirement are tested in frequency domain referring to the reference point definition below, so the analysis also done in frequency domain to evaluate the signal quality distortion for OOK-M signal.

Figure B.1-1 [3]: Reference point for EVM measurement

Figure 1:Constellation mapping for multi-bit OOK WUS (4-bit OOK case mapped to 64QAM).

Figure 2: Impact of mapping WUS constellation to existing QAM on OOK detection performance.

Figure 3: Impact of mapping WUS constellation to existing QAM on OFDM sequence detection performance.

- The tolerance of the distortion of the frequency domain data of OOK-4 is tested by remapping the frequency constellation of overlaid ZC sequence to one of existing modulation scheme.  Mapping the generated frequency domain values (before IFFT) to existing sequences/QAM modulations could be considered to reduce the impact on gNB. As one example, for OOK4 waveform, the higher modulation (e.g 64QAM) than QPSK may be used to map the frequency domain OOK4 symbols (left constellation in  Figure 1) to 64QAM constellation points (right constellation in  Figure 1) for CP-OFDM symbols as illustrated in Figure 1.  Figure 2 and Figure 3 illustrates the initial LLS result to evaluate the impact of OOK and OFDM sequence detection due to the remapping, It can be observed that the performance loss of such mapping on WUS detection performance (OOK and OFDM) is around 0.2 dB when mapping to regular data structure generated from 64QAM and around 0.6 dB when mapped to 16QAM.

The performance loss for OOK signal detection at UE receiver due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

From the constellation re-mapping, it can be observed that the time domain signal quality of OOK-4 is quite robust and can be maintained without big degradation of SNR when the frequency domain OOK-4 data is distorted by the amount of EVM equivalent to the 64QAM. The frequency domain data of OOK-4 is degraded if it is distorted by the amount of the EVM equivalent to the 16QAM.

The time domain OOK-4 signal can tolerate a distortion in frequency domain equivalent to and less than the EVM of 64QM before iFFT.

Specify the OOK-M signal quality requirement with equivalent EVM requirement better than 64QAM.

As the A-IoT BS can only transmit R2D signal for now, it is a question whether the legacy test model can be reused though the OFDM transmitter is agreed. To define a test model or requirement for R2D signal, RAN4 may wait RAN1 further progress.

Wait RAN1 further progress on the R2D signal design.

For ACLR, there is no coexisting result concern for the assumed ACLR in Table 6.6.2.2-2 in TR 38.769 below. RAN4 confirm this value can be specified for A-IoT BS for 1 RB transmission bandwidth definition. We think maybe RAN4 should wait RAN1 further progress and see whether other bandwidth would be defined.

Use the ACLR 1 (40 dB) and ACLR2 (50 dB) can be specified for one RB transmission bandwidth.

Wait RAN1 progress on transmission bandwidth definition other than 1 RB.

Table 6.6.2.2-2 [2]: A-IoT reader RF parameters

| A-IoT reader parameters | Values for evaluation |
| --- | --- |
| A-IoT reader total Tx power | 33 dBm |
| A-IoT reader receiver Noise Figure (dB) | 10 |
| A-IoT reader antenna gain (dBi) | 6 |
| ACLR | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dB，ACLR2:50dB) |
| ACS | Same as legacy NR BS |
| Antenna configuration | Antenna Array Geometry：equals to omni-directional antenna pattern in GCG in horizontal Antenna pattern (horizontal): AHϕ=-min12ϕϕ3dB2,Am, φ3dB = 90°, Am = 15 dB Antenna pattern (horizontal): AVθ=-min12θ-90°θ3dB2,SLAv, θ3dB= 90°, SLAv = 15 dB Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss): 6 |

For the Tx spurious requirements, if the Tx ON/OFF would be specified, there is no need to specify the protection of the own receiver. For co-location requirements, this should be further discussed. Though it is a objective in SID but there is no such objective in WID. RAN4 needs to confirm if A-IoT BS can be deployed co-located with other BS as the co-location requirement is protection on the other co-located BS deployed in other bands. In our view, it is better to specify the co-location requirement if co-located with other BS deployment can not be excluded. Adding the co-located requirement in later release may be problematic for the deployed A-IoT BS.

No need to specify protection of the own receiver if TX ON/OFF would be specified.

Specify the co-location requirement if the deployment of A-IoT BS with other co-located BS is not excluded.

For TAE requirement, we think the TR conclusion can be kept.

##### 6.7.2.1.4 Transmit signal quality

Frequency error and modulation quality (EVM) are potentially needed. TAE is only applicable for BS transmitting from multiple antennas via MIMO, CA, or combination of them. A-IoT BS doesn’t support these features, so TAE requirement is not needed.

TAE is not applicable for A-IoT BS as no CA or MIMO will be defined for R2D signal.

For Tx intermodulation requirement, it can be discussed whether the A-IoT BS can be co-located with another A-IoT BS transmitting in another sector. From the LLS simulation, it is observed that the R2D receiving will not be impacted when interfering signal is below -22 dB while the test of the Tx inter-modulation is -30 dB assuming the antenna isolation between antenna of wanted signal and antenna of the interfering signal. Therefore, we believe this still is a valid deployment case and Tx intermodulation to be defined for A-IoT BS.

Tx intermodulation should be defined for A-IoT BS.

Figure 4: BLER performance with 49 RB NR signal

For receiver requirement, the first requirement is the REFSENS requirement. As the SNR of the D2R requirement needs to be decided first, the signal design needs to be agreed first in RAN1. Therefore, RAN4 can wait Ran1 further progress on D2R signal design before making simulation campaign on the SNR threshold for D2R decoding. Meanwhile, RAN4 can confirm the noise figure of 10 dB for A-IoT BS receiver in Table 6.6.2.2-2 [2].

Wait RAN1 progress on D2R signal design before simulation campaign is initiated in RAN4.

Use the 10dB noise figure for the REFSENS specification.

For ACS and IBB, it is agreed that coexisting simulation will be needed to decide the interference level.

##### 6.7.2.2.3 In-band selectivity and blocking

Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study and wanted signal configuration and its corresponding configuration could be postponed to WI phase.

- In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, the wanted signal configuration and its corresponding configuration could be postponed to WI phase.

From the coexisting simulation results for D1T1-B- NR UE indoor (R2D: DL, CW2D/ D2R: UL), it is observed that NR DL throughput degradation. Therefore, it is a question how to mitigate this case. In TR 38.769, below text is proposed in 6.6.5.10. As the legacy UE cannot read the R2D signal and make the measurement on the A-IoT signal not be possible.

For the deployment scenarios with legacy NR UE indoor (scenario 1-2, 2-2, 5-2, 6-2), which either significant NR throughput degradation or significant SINR degradation for ambient IOT system is observed, some existing mechanisms based on gNB implementation can be considered to avoid/mitigate the interference. For example, for RRC idle mode, NR UEs can camp on a frequency different from ambient IOT system based on frequency priority configured by network; for RRC connected mode, network can handover NR UEs to a different cell (e.g. indoor gNB on a different frequency) based on existing measurement report triggering and event. Besides, other interference management solutions to reduce the inter-system interference from ambient IOT to proximity NR UE and vice versa can also be considered.

It is not clear that the mitigation scheme described in TR 38.769 in above text will be effective as there is no RAN1 agreement that the A-IoT BS can also be configured as a legacy NR cell, thus making the A-IoT BS coexisting with NR as one deployment cases. The key issue is that legacy UE cannot recognize A-IoT R2D or D2R signal and therefore, it is question how the legacy UE camp on the frequency other than A-IoT frequency.

Table 6.6.5.4-1: Scenario 2-2 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
| f | NR UL | reader | Under the assumption of 10% and 100% NR UE indoor, significant SINR degradation is observed |
| c | Reader | NR DL | Under the assumption of 10% and 100% NR UE indoor, NR DL throughput degradation is observed. |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

Clarify the mitigation scheme in TR 38.769 on whether a NR cell to be configured at the same A-IoT BS.

Coexisitng is not within the objective for now and there is a need to start the coexisting to help on the inband blocking requirement and ACS requirement. Placing more guard RB will help to reduce the impact on the ACS and blocking of A-IoT BS receiver, there is no conclusion on how many guard RB will be configured at macro Base station and as such, additional coexisting simulation will be needed adding the simulation parameter of number of guard RB around the A-IoT RB.

Coexisting simulation will be needed to study the ACS and inband blocking requirement for D1T1-B.

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for BS type 1-C in the presence of two interfering signals which have a specific frequency relationship to the wanted signal. As the FDM case of UL signal is within the RAN1 study scope but it is not clear how the FDMA will be specified as the small frequency shift is supported in device 1 as listed in Table 5.3-1 in TR 38.769. This also relates to the in-channel receiving and as such, both ICS and Rx intermodulation should be postponed.

##### 6.1.2.7.1	Multiple access schemes

Time-domain multiple access, and frequency domain multiple access at least by using a small frequency shift in baseband are studied.

Wait RAN1 progress on FDMA of D2R before RAN4 study the receiver intermodulation and ICS requirement.

# Conclusions

In this contribution, we present our overview on the A-IoT BS RF with below proposal and observations:

Observation 1 Legacy OFMD transmitter requirement can be kept mostly and LP-WUS RF impact in LP-WUS work item can be referenced.

Observation 2 Medium range type 1-C BS is to be specified for A-IoT BS.

Proposal-1: The RF impact on the CW transmission by A-IoT BS could be evaluated if this is also a valid scenario to consider for Rel-19 work item.

Proposal-2: Specify the ON/OFF requirement for A-IoT BS for HD-FDD operation.

Observation 3 The performance loss for OOK signal detection at UE receiver due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

Observation 4 The time domain OOK-4 signal can tolerate a distortion in frequency domain equivalent to and less than the EVM of 64QM before iFFT.

Proposal-3: Specify the OOK-M signal quality requirement with equivalent EVM requirement better than 64QAM.

Proposal-4: Wait RAN1 further progress on the R2D signal design.

Proposal-5: Use the ACLR 1 (40 dB) and ACLR2 (50 dB) can be specified for one RB transmission bandwidth.

Proposal-6: Wait RAN1 progress on transmission bandwidth definition other than 1 RB.

Proposal-7: No need to specify protection of the own receiver if TX ON/OFF would be specified.

Proposal-8: Specify the co-location requirement if the deployment of A-IoT BS with other co-located BS is not excluded.

Proposal-9: TAE is not applicable for A-IoT BS as no CA or MIMO will be defined for R2D signal.

Proposal-10: Tx intermodulation should be defined for A-IoT BS.

Proposal-11: Wait RAN1 progress on D2R signal design before simulation campaign is initiated in RAN4.

Proposal-12: Use the 10dB noise figure for the REFSENS specification.

Proposal-13: Clarify the mitigation scheme in TR 38.769 on whether a NR cell to be configured at the same A-IoT BS.

Proposal-14: Coexisting simulation will be needed to study the ACS and inband blocking requirement for D1T1-B.

Proposal-15: Wait RAN1 progress on FDMA of D2R before RAN4 study the receiver intermodulation and ICS requirement.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- TR 38.769