3GPP TSG-RAN WG4 Meeting # 114						R4-2502099

Athens, Greece,17th–21st February 2025

Source:		Ericsson

Title:			A-IoT general overview

Agenda item:		7.24.1

Document for:		Approval

# Introduction

In this paper, we present our thoughts on general aspects for A-IoT WI [1].

# Discussion

## system parameter

The relevant TP in TR38.769 [2] is quoted below:

#### 6.7.1.1	Operating band

According to the SID (RP-240826), Ambient IoT uses FR1 licensed spectrum in FDD. NR band n8 can be used as an example band.

Table 6.7.1.1-1 summarized the A-IoT system spectrum usage from co-existence scenario. It can be observed that The spectrum usage for A-IoT operation can be flexible, and it depends on several factors, such as whether the reader is A-IoT BS or intermediate UE, and whether CW transmission is deployed inside or outside topology. For the case which different spectrum is used for D2R and R2D, the FDD band definition can be kept. As CW transmission may interfere the R2D reception, the R2D and D2R should be TDM and therefore, for FDD band definition, it should be half duplex mode.

#### 6.7.1.2	Channel bandwidth

The R2D waveform is agreed to be OFDM-based waveform, therefore the channel bandwidth definition can follow the channel bandwidth definition in NR. The R2D channel bandwidth can be defined as one or multiple PRBs with guardband if necessary. The value for this can be determined during the WI phase.

#### 6.7.1.3	Channel arrangement

The channel spacing may be specified for D2R FDM operation for device 1/2a.

For Ambient IoT BS and intermediate UE, channel spacing and channel raster may be needed for deployment purpose, for example, to co-deploy with existing NR network.

In the WID [1], the n8 is listed as exemplar band and for operating band, the conclusion from TR can be followed, which the HD-FDD mode should be specified to avoid the CW interference to R2D reception. For channel bandwith and channel arrangement, RAN4 can wait RAN1 progressing on the signal design.

HD-FDD should be specified for band n8.

Wait RAN1 progress on the channel bandwidth and channel arrangement.

## Coexisting aspects

In WID [1], the system deployment scenario is listed as below:

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- The overall objective shall be to standardize the following Ambient IoT device:

Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Deployment scenario 1 with Topology 1, according to D1T1-B.

- FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

- Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

It can be observed that the coexisting scope in Rel-19 WID is reduced compared to the Rel-19 SI. The spectrum usage limitation is only the same with legacy FDD band and topology is limited to D1T1-B. For the spectrum deployment in point D above, it should be clarified whether the outdoor macro NR is only considered or indoor NR deployment also. This may relate to the whether 2GHz band also needs to be considered in the Rel-19 WID.  In SI phase, it is agreed that n8 only for outdoor deployment so outdoor macro NR together with n8 implicating the outdoor NR deployment. However, as feature specification is generic and for other NR bands, the indoor NR deployment is possible, so indoor NR coexisting with the A-IoT deployment should be considered also in coexisting scope. In light of this, we propose to clarify the item D in WID as below:

- Spectrum deployment in-band to outdoor NR and A-IoT BS indoor, also the standalone mode for A-IoT BS indoor deployment.

As the in-band deployment of a macro NR outdoor may have more interference to the reader due to the in-band emission, it is not clear for now that whether additional guard RB should be configured to reduce the interference and if so, how many guard RB would be necessary. This is also a question whether there is a need for coexisting for standalone deployment for A-IoT indoor. Reason is that the additional guard RB usage in in-band deployment may reduce the NR BS interference to reader but it is not clear whether such interference should be lowered in par with the interference level similar with the stand-alone deployment.  If so, the coexisting simulation for stand-alone may be exempt. However, there is no guard RB coexisting in SI phase and neither the SA mode. In this regards, we suggest that the coexisting simulation should be scoped in the WID phase for this purpose.

Update the WID to clarify the general scope of the spectrum deployment for coexisting scope.

RAN4 to discuss the coexisting for other band than n8 for potential coexisting scope in Rel-19 WID, for example, whether in-door NR deployment also relevant for the generic feature specification of A-IoT.

Coexisting for stand-alone deployment of the A-IoT can be exempt if the guard RB usage in in-band deployment can reduce the interference level in par with the SA mode deployment.

Coexisting simulation should be added in Rel-19 to further confirm SA deployment and # of guard RB needed for in-band deployment.

On top of the scope clarification in the WID [1] objective, there are some open issues relating to the coexisting in the SI phase. For example, for the SINR calculation for the R2D and below WF agreements:

Issue 4-1-3: Rx for device 1 and 2a

Agreement in RAN4#111: Assume no frequency selectivity for co-existence evaluation for calibration purposes for device 1 and 2a. Agreement in RAN4#112: - Alt-1: Assume no frequency selectivity - Alt-2: SINR; R2D with LPF. Interference from NR; Frequency selectivity: [7.5 dB]. Noise bandwidth; [180kHz]. SINRR2D_with_LPF=Wanted_SR2D_180kHzNoiseFloor 180kHz+(InterferenceNR10MHz/LPF_Selectivitylinear). - Alt-3: For R2D receiver with LPF. RF BW; 10MHz. BB BW; [180kHz]. SINRi (linear, not dB); Input SINR at the antenna connector with noise/interferencde BW=RF BW. SINRo (linear, not dB); Output SINR after BB LPF assuming downconverted noise/interference BW=BB BW. SINRo=SINRi28*SINRi+4*BWRFBWBB. - Alt-4: other LLS assumptions are not excluded

RAN4 has lengthy email discussion around the above topic but non consensus reached. The key issue is that how to factor in the 10MHz NR signal into the baseband SINR calculation as the A-IoT device has no channel filter on wanted signal. There is no maintenance work for study item, so such open issues can only be solved during the work item phase.

RAN4 continue on the discussion of the SINR calculation for R2D in coexisting study in work item phase.

Another issue is the outage performance, in TR 38.769, we have below text:

6.6.5.10        General remarks on coexistence findings

For D1T1 deployment scenario 1-1 (R2D, CW2D/D2R: DL) and 2-1(R2D: DL, CW2D/D2R: UL), which has no legacy NR UE indoor. Negligible performance degradation for NR system is observed. Less than 1 dB SINR degradation due to inter-system interference from NR to ambient IOT system is observed. Co-existence between ambient IOT and NR is feasible based on performance metric of NR throughput degradation and ambient IOT SINR degradation, no additional coexistence measures are required. Ambient IOT intra-system performance evaluated by link level simulation is also one of the key aspects affecting ambient IOT system performance, which will have impact to the outage of ambient IOT system.

This relate to the WF (R4-2414304) agreement below:

For inter-system interference (between AIOT and NR):

- If SINR degradation is smaller than or equal to [1]dB, it can be considered that inter-system interference is negligible.

- If SINR degradation is lager than [1]dB, consider following criteria:

  - Outage percentage consider SINR level with [10%] BLER

  - FFS on the outage percentage

- Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR

For intra-system interference (between AIOT and AIOT):

  - Outage percentage consider SINR level with [10%] BLER

  - FFS on the outage percentage

There is no progressing on the outage, and this is mainly due to there is no RAN1 consensus on the SNR LLS threshold during the study phase. However, RAN4 needs to specify the REFSENS and SNR threshold is one parameter to be decided first. For outage aspect, RAN4 can draw conclusion once SNR would be decided.

Outage performance for coexisting can be evaluated once SNR for REFSENS can be decided in RAN4.

From the discussion above, we propose to update the RAN4 objective with below coexisting aspect:

- RAN4 scope:

  - Coexisting study according to the spectrum deployment in general scope, e.g the guard RB needed for in-band deployment, SINR calculation for R2D coexisting evaluation.

Update the RAN4 objective with adding coexisting study aspect.

## CWT node specification

In the WID[1], there is no new specification planned for the CW node specification if the CW can be transmitted in a separate node other than BS.  Our understanding is that CW transmission core requirement should be within the same infrastructure specification, but current title of it does not reflect this, we suggest to updating this together for next WID update.

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| New TS | TS 38.xxx | Requirements for Ambient IoT device |  | RAN#109 | RAN4 Core part Note: including RF requirements and RRM requirements (if any) |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) radio transmission and reception and CW radio transmission. |  | RAN#109 | RAN4 Core part |

The CW node specification is not mentioned in WID.

Update the WID with adding the CW transmission in new specification title.

# Conclusions

In this contribution, we present our overview on the A-IoT general issues with below observation and proposals:

Proposal-1: HD-FDD should be specified for band n8.

Proposal-2:Wait RAN1 progress on the channel bandwidth and channel arrangement.

Proposal-3:Update the WID to clarify the general scope of the spectrum deployment for coexisting scope.

Proposal-4:RAN4 to discuss the coexisting for other band than n8 for potential coexisting scope in Rel-19 WID, for example, whether in-door NR deployment also relevant for the generic feature specification of A-IoT.

Observation 1 Coexisting for stand-alone deployment of the A-IoT can be exempt if the guard RB usage in in-band deployment can reduce the interference level in par with the SA mode deployment.

Proposal-5: Coexisting simulation should be added in Rel-19 to further confirm SA deployment and # of guard RB needed for in-band deployment.

Proposal-6: RAN4 continue on the discussion of the SINR calculation for R2D in coexisting study in work item phase.

Observation 2 Outage performance for coexisting can be evaluated once SNR for REFSENS can be decided in RAN4.

Proposal-7: Update the RAN4 objective with adding coexisting study aspect.

Observation 3 The CW node specification is not mentioned in WID.

Proposal-8: Update the WID with adding the CW transmission in new specification title.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- TR 38.769