# 3GPP TSG-RAN WG4 Meeting # 114						R4-2502101

# Athens, Greece,17th–21st February 2025

Source:		Ericsson

Title:			CW node RF impact overview

Agenda item:		7.24.2.3

Document for:		Approval

# Introduction

In this paper, we present our overview for RF requirements for a CW node.

# Discussion

Below is the recap of the RAN1 and RAN agreement relating to CW in general.

RAN1#116:

Agreement For R19 A-IoT study item, at least single-tone unmodulated sinusoid waveform is a candidate waveform for carrier wave for D2R backscattering. Agreement For R19 A-IoT study item, multi-tone waveforms for carrier wave for D2R backscattering can be studied.

RAN1#116bis:

Agreement For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones - FFS: how large gap is needed between tones Agreement For CW waveform for D2R backscattering, contiguous multi-tone OFDM signal is not studied in R19 SI. Agreement Study at least the following characteristics of unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering: - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation

RAN#103:

Proposal 3v2 (endorsed) - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc.

From the RAN1 and RAN agreements, it can be observed that the CW waveform is defined with unmodulated sinusoid waveform, and maximum number of CW signal is limited to two. As the CW transmission is controlled in terms of the timing, power, bandwidth, and spectrum, CW signal is relevant and RAN4 can continue to specify the RF characteristic of CW signal based on these basic assumptions.

CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

CW transmission’s timing, power, bandwidth, spectrum is controlled.

In WID objective, the relevant part for CW node RF discussion is quoted below.

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- The overall objective shall be to standardize the following Ambient IoT device:

Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Deployment scenario 1 with Topology 1, according to D1T1-B.

- FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

CW node is infrastructure node and deployed at fixed location according to the coexisting simulation assumptions in SI phase. The CW RF parameter in TR 38.769 is listed as below.

Table 6.6.2.2-4: CW RF parameters

| CW parameters | D1T1 | D2T2 |
| --- | --- | --- |
| Tx power (dBm) | If UL spectrum is used, UE Tx power is assumed, i.e. 23dB If DL spectrum is used, A-IoT reader Tx power is assumed, i.e. 33 dBm | Intermediate UE Tx power is assumed. |
| Antenna gain | Same as A-IoT reader | Same as intermediate UE |

Table 6.6.2.1-1. Deployment parameters for D1T1 and D2T2

| Parameter | D1T1 | D2T2 |
| --- | --- | --- |
| CW deployment | For D1T1-A2 and D2T2-A2, CW and A-IoT reader are collocated. For D1T1-B and D2T2-B, CW topology layout is the same as A-IoT reader. For each device, the nearest CW node will be activated during the simulation, and CW node is not co-located with any activated reader |  |
| Waveform (CW) | Unmodulated single tone |  |

The WF[3] can be starting point for the RF discussion for CW node:

### Issue 2-6: CW for D1T1

| RF Requirement for CW for D1T1 | Requirements potentially needed or not |
| --- | --- |
| Output power | YES |
| ON/OFF power | TBD in WI phase |
| Frequency error | YES |
| Phase noise | No for inside topology TBD in WI phase for outside topology |
| Unwanted emission | YES |
| Spurious emissions | YES |
| Intermodulation | TBD in WI phase |

For output power, the WID states the CW in UL spectrum which is corresponding to the 23 dBm output power. According to the link budget analysis in [4] Figure 3 shows the reader range will be 5.4 meters for CW output power of 23 dBm compared to 16.9 meters for CW output power of 33 dBm. In 38.769, the maximum coverage distance for device 1 is 15 meters.

-	Applicable maximum distance target value(s): The maximum distance targets are set separately for device 1, device 2a, device 2b, respectively. The distance target is compared to the evaluation results which assume the agreed mandatory evaluation assumptions.

For device 1 in D1T1: 15 m

The CW transmission power of 23 dBm cannot meet the 15 meter maximum distance target in TR 38.769.

Study the a reasonable CW output power to meet the 15 meter design target, e.g 33 dBm.

There is no coexisting simulation for higher CW power and we suggest adding this case for the coexisting if CW output power of 33 dBm or other value larger than 23 dBm will be needed.

Adding a coexisting simulation case for a CW output power larger than 23 dBm to meet the design target.

Figure 1: device 1 range considering the energy harvesting with CW transmission power of 33 dBm and 23 dBm

The output power should be measured within a certain bandwidth where 99% emitted power is confined. To study the needed bandwidth, a simulation with CW signal with phase noise profile in TR 38.803 is illustrated in Figure 2 and it can be observed that CW OCC is around 100Hz. If channel bandwidth would be defined around 100Hz.

A bandwidth should be defined together with CW output power so that the power can be measured with OCC bandwidth.

Figure 2: Occupied BW for a CW signal

For ON/OFF requirement, as the CW transmission will be during the D2R transmission, it will be beneficial to have predefined transient time so the receiving time of the D2R would be expected. This will also make sure the interference between CW ramp up time and R2D receiving to be minimal.

Transient time for R2D signal ramp down, transient time for the CW power ramp up and the synchronization time error between the A-IoT BS and CW node may overlap to each other and the total time between R2D signal ends to CW start may decide the TR2D_min in RAN1 agreement.

ON/OFF TX requirement should be specified.

For the phase noise, there are concerns that the close-in phase noise of the CW transmission may distort the D2R receiving for the outside topology. It is true that the unwanted emission generated by the phase noise and the transmitter non-linearity may appear higher than the backscatter signal at the antenna when CW is transmitted at the same A-IoT BS. This is illustrated in Figure 3. The phase noise profile in TR 38.803 is used and it can be observed that around -50 dBm at the frequency offset of the 300kHz. If the reader REFSENS is around -70 dBm, such level of emission will drown the backscatter signal. If only the outside topology is relevant, the -50 dBm@300kHz will experience the pathloss and therefore, it may not compete with the backscattered signal.

The close-in unwanted emission may not be critical for the outside topology.

Figure 3: the CW transmission with phase noise

Using the TR 38.803 phase noise profile, the CW transmission and the unwanted emission by phase noise is simulated and illustrated in Figure 4. The ACLR1 is around 40 dBc and ACLR2 is around 50 dBc.  Adding the PA non-linearity margin of 3 dB, we think the unwanted emission if modelled as the ACLR1 and ACLR2 should be 37 dBc and 47 dBc.

CW unwanted emission can be modeled with 37 dBc for ACLR1 and 47 dBc for ACLR2.

Figure 4: the CW transmission with phase noise

For inter-modulation requirement, there is a need to discuss whether the co-location with other NR BS would be a valid case to consider. As CW node can only transmit the CW signal, so the requirement would not reuse the NR BS requirement if there is a need to define.

Discuss if the co-location with other BS is a valid scenario before discussing the inter-modulation requirement.

# Conclusions

In this contribution, we present our overview on the A-IoT BS RF with below proposal and observations:

Observation 1 CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

Observation 2 CW transmission’s timing, power, bandwidth, spectrum is controlled.

Observation 3 The CW transmission power of 23 dBm cannot meet the 15 meter maximum distance target in TR 38.769.

Proposal-1: Study the a reasonable CW output power to meet the 15 meter design target, e.g 33 dBm.

Proposal-2: Adding a coexisting simulation case for a CW output power larger than 23 dBm to meet the design target.

Proposal-3: A bandwidth should be defined together with CW output power so that the power can be measured with OCC bandwidth.

Proposal-4: ON/OFF TX requirement should be specified.

Observation 4 The close-in unwanted emission may not be critical for the outside topology.

Proposal-5: CW unwanted emission can be modeled with 37 dBc for ACLR1 and 47 dBc for ACLR2.

Proposal-6: Discuss if the co-location with other BS is a valid scenario before discussing the inter-modulation requirement.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- TR 38.769

- R4-2420470, WF on RF requirements for A-IoT, Huawei

- R4-2415452, On CW cancellation capability, Ericsson