# 3GPP TSG-RAN WG4 Meeting # 114						R4-2502102

# Athens, Greece,17th–21st February 2025

Source:		Ericsson

Title:			RF Impact for FDD bands for PC2 RedCap

Agenda item:		7.7.1

Document for:		Approval

# Introduction

In this paper, we present our view on PC2 RedCap UE RF for FDD bands added in WID [1].

# Discussion

In revised WID[1], the newly added scope of the RedCap PC2 is quoted as below:

- Specify the PC2 RF requirements for RedCap UE on both TDD and FDD bands, based on 1Tx architecture with 26 dBm PA.

  - The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed.

  - For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way.

    - Note: Not differentiate the applicable form factors for TDD bands in the specification.

  - For FDD bands, specify RF requirements for PC2 RedCap UE

    - Band specific requirements are specified for the example bands including n3 and n1 when band agnostic way is not applicable (Example bands are selected by considering different sensitivity degradation causes)

    - Note: Wearable device supporting HPUE is not in the scope for FDD bands in this work item

For Tx requirement, as objective indicates that UE maxim output power, Tx power tolerance. MPR and A-MPR needs to discuss. As 1 Tx architecture with 26 dBm PA is assumed, the NR non-RedCap UE MPR and A-MPR can be reused for the PC2 RedCap UE. This is also agreed in WF [2] and quoted below.

Issue 1-1-1: A-MPR for FD-FDD RedCap PC2 UE

Online agreement:

- A-MPR for Redcap PC2 UE needs to be specified based on single Tx architecture by re-using the non-RedCap PC2 where applicable

Issue 1-2-2: RSD for FD-FDD RedCap PC2 1Rx UE

Online agreement:

- RSD for RedCap PC2 1Rx UE needs to be specified based on single Tx architecture

As the WID[1] is based on 1 Tx architecture, and there is also a note in A-MPR table in clause 6.2.3 and quoted below. The MPR and A-MPR for non-(e)Redcap can be reused for (e)RedCap.

NOTE 7: 	The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 or txDiversity2Tx-r18 [TS 38.306], the additional relaxation of [2] dB is applicable

NR PC2 UE MPR and A-MPR can be reused for PC2 RedCap.

For RSD, the scaling factor for the REFSENF for 1 Rx is based on the 2Rx REFSENS of PC3. For some of FDD bands for PC2, there are additional RSD allowed and this is specified in Table 7.3.2-1c in TS 38.101-1 and quoted below. The exemplar band n1 and n3 is specified for 2 Rx relaxation.

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 | 0.8 | 0.9 | 1.1 | 1.2 | 1.3 | 2.7 | 2.8 | 3.5 |  |  |
| n3 | 0.5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.8 | 1.1 | 1.5 | 2.3 | 2.8 |
| n7 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |  | 2.0 |
| n8 | 0.5 | 0.7 | 0.8 | 2.3 | 2.8 | 3.2 | 3.1 |  |  |  |
| n13 | 0.8 | 0.9 |  |  |  |  |  |  |  |  |
| n14 | 0.6 | 0.8 |  |  |  |  |  |  |  |  |
| n25 | 0.8 | 0.8 | 0.9 | 1.1 | 1.3 | 2.7 | 2.8 | 3.5 | 3.7 |  |
| n66 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 | 0.5 | 0.9 | 0.9 | 2.2 | 2.42 2.53 | 2.52 2.43 | 2.92 3.13 |  |  |  |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| n85 | 0.6 | 1.0 | 2.6 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |

However, as this relaxation is for two Rx, there is a need to see how to specify the 1 Rx RSD for PC2. During the Rel-17 1 Rx scaling factor discussion FD-FDD band, the Tx interference to Rx interference is considered when scaling 2 Rx to 1Rx in Table 7.3I.2-1 in TS 38.101-1. This scaling factor considers the factor that the main Rx branch may have more interference compared to the diversity branch and also the factor that the Tx interference may be higher as the channel bandwidth increased.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |

Though the FD-FDD band scaling factor for 1 Rx is generic, the tightening factor for HD-FDD is band specific as agreed in WF[3]. This is condiering some band with shorter duplex distance and

- Agreement: Compared to 2Rx FDD REFSENS, apply 0.8 dB tightening for REFSENS requirement for 2Rx HD-FDD REFSENS as baseline in general, and check band by band whether the additional tightening or relaxation is needed for a certain band.

The scaling factor for HD-FDD comparing to the 2RX FDD REFSENS.

Reuse the scaling factor of 2 Rx to 1Rx for PC3 for PC2 (e)RedCap UE.

- For example, the band specific scaling factor is considered in table below.

          - For 5MHz channel BW

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -99 dBm | 0 dB | n1, n18, n24, n30, n65, n66, n70, n74 |
| > -99 dBm and ≤ -98 dBm | 0.5 dB | n2, n5, n7, n28, |
| > -98 dBm | 0.8 dB | n3, n8, n12, n13, n14, n20, n25, n26, n71 |

From the above discussion, the RSD for the 1 RX cannot be specified generically based on the legacy scaling factor approach. Our suggestion is that FD-FDD and HD-FDD scaling factor can follow the legacy FD-FDD and HD-FDD approach in Rel-17 and only specify the n3 and n1 in Rel-19 unless there are more FDD band for PC2 is requested by operator.  Following the legacy HD-FDD scaling factor discussion, the HD-FDD tightening  is 0 dB for n1 and 0.8 dB for n3.

Specify only n3 and n1 for PC2 RedCap in Rel-19 unless there is more PC2 FDD band requested by operator.

HD-FDD tightening  is 0 dB for n1 and 0.8 dB for n3 compared to the 2Rx FDD REFSENS.

# Conclusions

In this contribution, we present our overview on the PC2 RedCap RF overview with below proposal:

Proposal-1: NR PC2 UE MPR and A-MPR can be reused for PC2 RedCap.

Observation 1 The scaling factor for HD-FDD comparing to the 2RX FDD REFSENS.

Proposal-2: Reuse the scaling factor of 2 Rx to 1Rx for PC3 for PC2 (e)RedCap UE.

Proposal-3: Specify only n3 and n1 for PC2 RedCap in Rel-19 unless there is more PC2 FDD band requested by operator.

# References

- RP‑243267, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom

- R4-2314748, WF on NR power class 2 RedCap UE in FR1

- R4-2120067 , WF on the RedCap RF, Ericsson