# 3GPP TSG-RAN WG4 Meeting # 114						R4-2502105

# Athens, Greece,17th–21st February 2025

Source:		Ericsson

Title:			On IoT NTN UE RF requirment

Agenda item:		7.30.2

Document for:		Approval

# Introduction

In this paper, we present our IoT NTN UE RF impact on uplink capability objective.

# Discussion

In WF [1], the issues below are to be studied.

#### Issue 1-4: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2

- Agreement:

  - Pending on RAN1 progress of slot-level OCC2 resource mapping.

The related RAN1 agreement are listed below:

Agreement

For 3.75kHz SCS OCC for NPUSCH format 1, the maximum OCC length is 2 for connected mode.

Agreement

For single tone 15kHz SCS Slot-level OCC for NPUSCH format 1, OCC length larger than 2 is not supported.

Agreement

For NPUSCH Format 1 single-tone 15kHz SCS, the slot-level scheme for non-DMRS symbols is that spreading is performed in the unit of one slot.

- Note: whether RU length is extended or not after applying OCC is a separate discussion.

RAN1 has concluded that the OCC mapping length is 2 slots for 15KHz SCS. The RU length is not counted in for OCC2 scheme mapping to slot level as RAN1 agree below from above agreements.

#### Issue 1-4: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2

- Agreement:

  - Pending on RAN1 progress of slot-level OCC2 resource mapping.

Figure 1:OCC across 2 slots with/without CFO constant in 8 repetitions

The simulation illustrated in Figure 1 shows that for OCC2 for different CFO in two time slots for NR NTN, there is slight performance degradation (less than 0.1 dB) for 2% BLER. This is neglectable so there is no RF impact for OCC2. The simulation is done for NR NTN, for OOC2 performance for IoT NTN, we believe the same conclusion also apply to IoT NTN though only one tone applied to NPUSCH compared to NR NTN.

No RF impact on OCC2 for IoT NTN for 15kHz SCS.

#### Issue 1-5: Whether there would be UE RF requirement impact when OCC shemes would cross gap in UL transmission.

- Agreement:

  - Whether to study the case that gap between OCC slots may affect RF requirement pending on RAN1.

For the issues above, there is no progress in RAN1 on how scheme in relation to the gap defined in RAN1 and RAN4 should wait more RAN1 progress on this.

Wait RAN1 progress on the gap definition and relation to OCC scheme.

#### Issue 1-6: Whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

- Agreement:

  - Further discuss whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

In our paper on OCC analysis for NR NTN [2], the variance of the frequency offset could be used as one condition to group the UE for OCC4 scheduling. The analysis [2] does not assume the periodicity of the UE makes the frequency synchronization. As discussed above, our view is that there is no RF impact for slot-level OCC2 for 15kHz SCS, meaning OCC performance would not be impacted when one UE has 0.1 ppm and another UE has -0.1ppm. Therefore, Issue 1-6 can depend on the issue 1-4.

Issue 1-6 depends on consensus on issue 1-4.

# Conclusions

In this contribution, we present our general overview for IoT NTN UE RF impact for UL capacity objective with below proposal:

Proposal-1: No RF impact on OCC2 for IoT NTN.

Proposal-2:Wait RAN1 progress on the gap definition and relation to OCC scheme.

Observation 1 Issue 1-6 depends on consensus on issue 1-4.

# References

- R4-2419899,	Topic summary for [113][315] IoT_NTN_Ph3, MediaTek Inc

- R4-25xyz, Other NTN UE RF impact, Ericsson