3GPP TSG-RAN WG4 Meeting #114	R4-2502106

# Athens, GR, 17th – 21st Feburary, 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | x | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TEI18) CR to 36.102 NB-IoT NTN in-band operation with NR NTN [NTN NB-IoT] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NB-IoT NTN only supports stand-alone operation but is lack of in-band operation. There is strong NTN industry demand to enable NB-IoT NTN in-band operation with NR NTN firstly. Therefore, the addition of the optional feature is proposed. This CR providing Ericsson view on the ongoing CR modification discussion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Regarding support for this new optional feature, additional formulas and clarifications are introduced to enable operation of in-band NTN NB-IoT with NR NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not supported by UE: In-band deployments of NB-IoT NTN will be impossible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.4B.1, 5.4B.2, 5.4B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================Start of the change 1==========================

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFRaster	Band dependent channel raster granularity

BWChannel	Channel bandwidth

F					Frequency

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer			Frequency of the interferer

FIoffset				Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

FC	Frequency of the carrier centre frequency

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

GB                       Guard-band

IB                         In-band

FOOB	The boundary between the E-UTRA out of band emission and spurious emission domains.

LCtone	Transmission bandwidth which represents the length of a contiguous sub-carrier allocation expressed in units of tones

NDL 	Downlink EARFCN

NOffs-DL 	Offset used for calculating downlink EARFCN

NOffs-UL 	Offset used for calculating uplink EARFCN

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_alloc	Total number of simultaneously transmitted resource blocks in Channel bandwidth or Aggregated Channel Bandwidth.

Ntone	Transmission bandwidth configuration for category NB1 and NB2, expressed in units of tones.

Ntone 3.75kHz	Transmission bandwidth configuration for category NB1 and NB2 with 3.75 kHz sub-carrier spacing, expressed in units of tones.

Ntone 15kHz 	Transmission bandwidth configuration for category NB1 and NB2 with 15 kHz sub-carrier spacing, expressed in units of tones.

NUL	Uplink EARFCN.

PCMAX	The configured maximum UE output power.

PInterferer	Modulated mean power of the interferer

PPowerClass	PPowerClass is the nominal UE power (i.e., no tolerance).

PPowerClass_Default	PPowerClass_Default is the default nominal UE power (i.e., no tolerance) for the band.

PUMAX	The measured configured maximum UE output power.

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

ΔfOOB	Δ Frequency of Out Of Band emission

====================End of the change 1==========================

====================Start of the change 2==========================

## 5.4B	Channel arrangement for category NB1 and NB2

### 5.4B.1	Channel spacing

Nominal channel spacing for UE category NB1 and NB2 in stand-alone mode is 200 kHz. For in-band operation, the nominal channel spacing between two adjacent category NB1 or NB2 carriers is 180 kHz.

====================End of the change 2==========================

====================Start of the change 3==========================

### 5.4B.2	Channel raster, carrier frequency and EARFCN

#### 5.4B.2.1	General

The channel raster of UE category NB1/NB2 shall be as defined in clause 5.4A.2, and the channel raster per-frequency band shall be as defined in table 5.4A.2-1.

The carrier frequency of UE category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD.

The carrier frequency of UE category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD.

#### 5.4B.2.2	Stand-alone operation

The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL)

The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

For the carrier including NPSS/NSSS for stand-alone operation, the MDL = 0 is used.

Note 1: For the NB-IoT in-band operation in NTN, non-anchor carrier operation mode can be assumed the same with anchor operation mode when IE inbandCarrierInfo is not present in either DL-CarrierConfigDedicated-NB or DL-CarrierConfigCommon-NB, the operation mode for anchor carrier is  indicated in operationModeInfo IE with a value standalone in MasterInformationBlock-NB.

#### 5.4B.2.3	In-band and guard band operation

In Rel-18, NTN NB-IoT UEs [can/may] optionally support in-band operation mode.

The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

For the carrier including NPSS/NSSS, MDL is selected from {-2,-1,0,1}. For in-band and guard-band operations, MDL = -0.5 is not applicable.

<Need further discussion and whether capture this content in CR or WF>]

NOTE 1: For the NB-IoT in-band operation in NTN, non-anchor carrier operation mode can be assumed the same with anchor operation mode when IE inbandCarrierInfo is not present in either DL-CarrierConfigDedicated-NB or DL-CarrierConfigCommon-NB, the operation mode for anchor carrier is  indicated in operationModeInfo IE with a value guardband-r13 in MasterInformationBlock-NB

[

NOTE 3: UE is not expected to be aware of where the configured NB-IoT UL carrier is within the NTN NR carrier. It is presumed that operators will ensure, through network configuration during deployment, that there is sufficient guardband between NB-IoT carrier and operating band and/or spectrum block edges to meet emission requirements of clause 6.5B outside its allocated spectrum block.

[NOTE 4: The NB-IoT GB operation in NR NTN is up to UE and SAN implementation with no conformance testing requirements. <Need further discussion and whether capture this content in CR or WF>]

[NOTE 5: No conformance testing requirements for NTN in-band operation with power imbalance. Alternatively, No UE RX RF/Demod requirements to be associated with NTN in-band operation power imbalance. <Need further discussion and whether capture this content in CR or WF>]

]     NOTE 6: Mixed mode capability in 4.3.4.115 in TS 36. is not supported in NTN.

====================End of the change 3==========================

====================Start of the change 4==========================

### 5.4B.3	TX–RX frequency separation

For UE category NB1/NB2 operation in stand-alone mode, the default TX-RX frequency separation shall be as specified in Table 5.4A.3-1 for the NB-IoT TX and RX channel bandwidth defined in Table 5.3B-1.

For in-band operation, the category NB1 and NB2 TX-RX frequency separation is flexible within the assigned channel bandwidth of NR NTN carrier with the TX-RX frequency separation of the NR NTN carriers as specified in TS 38.101-5.

====================End of the change 4==========================