# 3GPP TSG-RAN WG4 Meeting #114	R4-2502137

# Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 		Consideration of BS RF aspects for 108 sub-bands introduction

Agenda Item:	6.21.4

Document for:	Discussion

# Introduction

According to the WID in [1], the following objectives were defined for LTE_terr_bcast_bands_sub_108 WID:

In this RAN4 WI two new bands for LTE-based 5G Broadcast will be specified as a subset of band 108, with the following objective: - Specify two new SDO bands from 470 to 608 MHz and 606 to 698 MHz, as subsets of the existing band 108 in a Release independent manner from Rel-17. - Specify related UE RF core requirements for 6/7/8 MHz PMCH [RAN4]1 - Specify related BS RF core requirements for 6/7/8 MHz PMCH [RAN4]

In this contribution, we provide analysis of the BS RF aspects for 108 sub-bands.

# Discussion

According to the WID [1], BS RF requirements are to be defined for two 108 sub-bands in TS 36.104. As this work relies on 108, it is assumed that it will be basically reusing the work already done for 108. Below, we have provided brief analysis of all the BS RF requirements in TS 36.104 evaluated one-by-one, and summarized:

Table 1

|  | BS RF requirement | E-UTRA Band 108 | 108 sub-bands introduction |
| --- | --- | --- | --- |
| TX | Base station output power | Applies | Same as for band 108 |
|  | Output power dynamics | Not applicable |  |
|  | Transmit OFF power | Applies |  |
|  | Transmitter transient period | Applies |  |
|  | Frequency error | Applies |  |
|  | EVM | Applies, except 1024QAM |  |
|  | TAE | Not applicable |  |
|  | Unwanted emissions | Applies; additional dedicated regional requirements in clause 6.6.0 |  |
|  | Occupied bandwidth | Applies |  |
|  | ACLR | Applies; dedicated requirements in clause 6.6.2 |  |
|  | Transmitter spurious emissions | Applies; dedicated requirements in clause 6.6.4.5 |  |
|  | OBUE | Applies; dedicated requirements in clause 6.6.3.4 |  |
|  | Co-ex requirements | Not applicable |  |
|  | Transmitter intermodulation | Applies |  |
| RX | Narrowband blocking | Not applicable (for BS supporting only LTE-based 5G terrestrial broadcast) |  |
|  | Blocking |  |  |
|  | Out-of-band blocking |  |  |
|  | Co-location with other base stations |  |  |
|  | Receiver spurious emissions |  |  |
|  | Intermodulation |  |  |
|  | Narrowband intermodulation |  |  |

Proposal 1: There is no impact on the content of TS 36.104 BS RF (Tx and Rx) requirements with the introduction of band 108 sub-bands.

The only formal issue for BS RF would be to reassure that all 108 requirements also apply to its sub-bands. Solution to this depends on the decision on the (sub-)bands numbering.

Proposal 2: It is sufficient to use the following statement in TS 36.104, to reassure that Band 108 BS RF requirements also apply to its sub-bands: “Whenever band 108 is referred, the same requirements also apply to its sub-band(s), unless otherwise stated”.

## Regulatory aspects

Referring to the WID [1], it has stated that the aim is to help with market introduction in larger regions such as Europe, U.S. and India. Now referring to TS 36.104 clause 6.6.0 on Unwanted emissions for LTE based 5G terrestrial broadcast, it lists references to regional requirements on unwanted emissions for terrestrial broadcast BS, including ITU region-specifics.

To fulfil WID objectives, it is seen essential to reassure that the list of all applicable regional regulations is still valid and up to date (more specifically, it seems that at least one market listed in the WID is not explicitly addressed in table 6.6.0 of TS 36.104).

Proposal 3: Reassure that regional regulations applicable to all intended markets and regions are covered and up-to-date in table 6.6.0 (i.e. References to regional requirements on unwanted emissions for terrestrial broadcast BS) of TS 36.104.

# Conclusions

Based on the discussion above, the following is proposed:

Proposal 1: There is no impact on the content of TS 36.104 BS RF (Tx and Rx) requirements with the introduction of band 108 sub-bands.

Proposal 2: It is sufficient to use the following statement in TS 36.104, to reassure that Band 108 BS RF requirements also apply to its sub-bands: “Whenever band 108 is referred, the same requirements also apply to its sub-band(s), unless otherwise stated”.

Proposal 3: Reassure that regional regulations applicable to all intended markets and regions are covered and up-to-date in table 6.6.0 (i.e. References to regional requirements on unwanted emissions for terrestrial broadcast BS) of TS 36.104.

# References

[1] 	 RP-243268, New bands for LTE based 5G terrestrial broadcast for early deployments, LTE_terr_bcast_bands_sub_108, WID, RAN#106

[2]		RP-232590, New bands and bandwidth allocation for LTE based 5G terrestrial broadcast -part 2, LTE_terr_bcast_bands_part2, WID, RAN#101