3GPP TSG-RAN4 Meeting #114	R4-2502161

# Athens, Greece, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draft | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on MPR for SL intra-band non-contiguous CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_intraB_CA_ITS-Core |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For PC2/PC3 SL intra-band non-contiguous CA, MPR requirements are specified for PSSCH/PSCCH, PSFCH, and SSSB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add MPR requirements of PSSCH/PSCCH, PSFCH, and SSSB for PC2 and PC3 SL intra-band non-contiguous CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For SL intra-band non-contiguous CA, PC2 and PC3 MPR requirements for PSSCH/PSCCH, PSFCH, and SSSB are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.2.1A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

##### 6.2E.2.1A.2	MPR for sidelink intra-band non-contiguous CA

##### 6.2E.2.1A.2.0	General

For SL intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determins the MPR type as follows:

For UE indicating dualPA-Architecture supported

If OR (LCRB1 = 0, LCRB2 = 0)

MPR defined in Clause 6.2E.2.2 for PC3 and PC2 UE.

Else If AND (FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPR defined in Clause 6.2E.2.1A.2.1 to meet -13dBm/MHz for PC3 and PC2 UE.

Else

MPR defined in Clause 6.2E.2.1A.2.1 to meet -30dBm/MHz for PC3 and PC2 UE.

For UE without indicating dualPA-Architecture supported

If OR (LCRB1 = 0, LCRB2 = 0 )

MPR defined in Clause 6.2E.2.2 for PC3 and PC2 UE.

Else If AND (FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPR defined in Clause 6.2E.2.1A.2.2 to meet -13dBm/MHz for PC3 and PC2 UE.

Else

MPR defined in Clause 6.2E.2.1A.2.2 to meet -30dBm/MHz for PC3 and PC2 UE.

where

-	LCRB1 is for CC1, which is the component carrier with lower frequency

-	LCRB2 is for CC2, which is the component carrier with higher frequency

-	B =  (LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

-	FIM3,high_block,high = (2 * Fhigh_alloc,high_edge ) – Flow_alloc,low_edge

-	FIM3,low_block,low = (2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of the lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of the lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of the upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of the upper transmission bandwidth allocation.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

MPRs in section 6.2E.2.1A.2.2 are applicable only when the Gap between the component carriers is ≤ the overall channel bandwidth summed across all the component carriers.

The definition of the gap is between the component carriers in a spectrum that is not part of any configured component carrier that is located in between the lowest edge of the component carrier with higher center frequency and the highest edge of the component carrier with center frequency that is located lower in frequency.

##### 6.2E.2.1A.2.1	MPR with indicating dualPA-Architecture supported

MPR in this clause is for SL intra-band non-contiguous CA for UEs indicating IE dualPA-Architecture supported.

The allowed maximum output power reduction for PSSCH/PSCCH is defined in Table 6.2E.2.1A.2.1-1 for power class 3 and Table 6.2E.2.1A.2.1-2 for power class 2.

Table 6.2E.2.1A.2.1-1: PSSCH/PSCCH MPR for intra-band SL CA for power class 3

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B < 5.04 | ≤ [6.5] | ≤ 13.5 |
| 5.04 ≤ B < 10.08 |  | ≤ 12.5 |
| 10.08 ≤ B < 20.16 |  | ≤ 11.5 |
| 20.16 ≤ B |  | ≤ 10.5 |

Table 6.2E.2.1A.2.1-2: PSSCH/PSCCH MPR for intra-band SL CA for power class 2

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B < 5.04 | ≤ [7.0] | ≤ 15.0 |
| 5.04 ≤ B < 10.08 |  | ≤ 13.0 |
| 10.08 ≤ B < 20.16 |  | ≤ 12.0 |
| 20.16 ≤ B |  | ≤ 10.0 |

The allowed maximum output power reduction for PSFCH is defined in Table 6.2E.2.1A.2.1-3 for power class 3 and Table 6.2E.2.1A.2.1-4 for power class 2.

Table 6.2E.2.1A.2.1-3: PSFCH MPR for intra-band SL CA for power class 3

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 9.0 | ≤ 16.0 |

Table 6.2E.2.1A.2.1-4: PSFCH MPR for intra-band SL CA for power class 2

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 9.5 | ≤ 16.5 |

The allowed maximum output power reduction for S-SSB is defined in Table 6.2E.2.1A.2.1-5 for power class 3 and Table 6.2E.2.1A.2.1-6 for power class 2.

Table 6.2E.2.1A.2.1-5: S-SSB MPR for intra-band SL CA for power class 3

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 8.5 | ≤ 15.0 |

Table 6.2E.2.1A.2.1-6: S-SSB MPR for intra-band SL CA for power class 2

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 9.0 | ≤ 17.0 |

##### 6.2E.2.1A.2.2	MPR without indicating dualPA-Architecture supported

MPR in this clause is for SL intra-band non-contiguous CA for UEs without indicating IE dualPA-Architecture supported.

The allowed maximum output power reduction for PSSCH/PSCCH is defined in Table 6.2E.2.1A.2.2-1 for power class 3 and Table 6.2E.2.1A.2.2-2 for power class 2.

Table 6.2E.2.1A.2.2-1: PSSCH/PSCCH MPR for intra-band SL CA for power class 3

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B < 5.04 | ≤ [10.5] | ≤ 17.0 |
| 5.04 ≤ B < 10.08 | ≤[ 9.5] | ≤ 16.0 |
| 10.08 ≤ B |  | ≤ 15.5 |

Table 6.2E.2.1A.2.2-2: PSSCH/PSCCH MPR for intra-band SL CA for power class 2

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B < 5.04 | ≤ [11.5] | ≤ 19.0 |
| 5.04 ≤ B < 10.08 | ≤[ 11.0] | ≤ 17.5 |
| 10.08 ≤ B |  | ≤ 16.5 |

The allowed maximum output power reduction for PSFCH is defined in Table 6.2E.2.1A.2.2-3 for power class 3 and Table 6.2E.2.1A.2.2-4 for power class 2.

Table 6.2E.2.1A.2.2-3: PSFCH MPR for intra-band SL CA for power class 3

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 12.0 | ≤ 18.0 |

Table 6.2E.2.1A.2.2-4: PSFCH MPR for intra-band SL CA for power class 2

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 15.0 | ≤ 20.0 |

The allowed maximum output power reduction for S-SSB is defined in Table 6.2E.2.1A.2.2-5 for power class 3 and Table 6.2E.2.1A.2.2-6 for power class 2.

Table 6.2E.2.1A.2.2-5: S-SSB MPR for intra-band SL CA for power class 3

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 13.0 | ≤ 21.0 |

Table 6.2E.2.1A.2.2-6: S-SSB MPR for intra-band SL CA for power class 2

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 14.0 | ≤ 23.0 |

< END OF CHANGE #1 >