3GPP TSG-RAN WG4 Meeting #114		R4-2502163

Athens, Greece, 17th – 21nd February, 2025

Agenda Item:	7.17.2.2

Source: 		Qualcomm Incorporated

Title: 			Enhancement on FR2 CSSF

Document for:	Discussion

# 1.	Introduction

In this paper, we present our view on the enhancement of FR2 CSSF.

# 2.  Past Agreements

Following agreements happened during the last meeting [1].

Issue 2-4-1: Tentative solution 1: measure one serving CC per band [Wayforwad]: FFS the following solution: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured, - Reuse the legacy principle for neighbour cell measurement - Measure PCC if PCC in band or - Measure PSCC if PSCC in band or - Measure SCC with neighbor cell MO or - UE implementation - FFS whether indication is per-UE or per-band - Other options are not precluded. - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs [Wayforward]: FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1: - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement. - FFS whether network indication is needed - Details is FFS - FFS on whether and how PCC and PSCC resource sharing is supported Issue 2-4-3: Tentative solution 3: 3 searchers for CSSF Agreement: RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is: - FFS: FR1 scenario (CA and NR-DC) only. - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only. - FFS: All scenarios. Agreement: - RAN4 to introduce new individual capability(s) for CSSF reduction in R19. - delay the capability detail discussion to the end of the core part. ### Issue 2-7: Other WID scope discussion Agreement: - after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable

# 3. 	Discussion

## Impact of Three Searchers

RAN4 agreed to discuss the solutions regarding three searchers. The use of three searchers can reduce L3 measurement delay significantly.  

Table1 shows the CSSF in FR1 + FR2 CA (FR2 PCell) scenario of SA mode assuming two searchers. Table2 shows the CSSF in same FR1 + FR2 CA scenarios of SA mode assuming three searchers. Both these tables assume the absence of CSI-RS based L3 configuration.

Table 1: CSSF in FR1 + FR2 CA scenarios of SA mode (two searchers)

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 +FR2 CA (FR2 PCell)Note 1 | N/A | NSCC_SSB + Y | 1 | N/A | NSCC_SSB + Y | NSCC_SSB + Y |
| NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. Note: CSI-RS based L3 measurement is not configured in PCC and SCC. |  |  |  |  |  |  |

Table 2: CSSF in FR1 + FR2 CA (FR2 PCell) of SA mode (with one FR1 + two FR2 searchers)

| Scenario |  | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 +FR2 CA (FR2 PCell)Note 1 | N/A |  | NSCC_SSB_FR1 + Y_FR1 | 1 | N/A | NSCC_SSB_FR2 + Y_FR2-1 | NSCC_SSB_FR2 + Y_FR2-1 |
| - NSCC_SSB_FR1 = Number of configured SCell(s) in FR1 with only SSB based L3 measurement configured, which is measured without MG. - NSCC_SSB_FR2 = Number of configured SCell(s) in FR2 with only SSB based L3 measurement configured, which is measured without MG. - Y_FR1 is the number of configured inter-frequency FR1 MOs without MG that are being measured outside of MG; otherwise, it is 0. - Y_FR2 is the number of configured inter-frequency FR2 MOs without MG that are being measured outside of MG; otherwise, it is 0. Note: CSI-RS based L3 measurement is not configured in PCC and SCC. |  |  |  |  |  |  |  |

Let’s assume, NSCC_SSB_FR1 = NSCC_SSB_FR2 = 0.5 * NSCC_SSB and Y_FR1 = Y_FR2 = 0.5 * Y.

In that case, the CSSF for FR1 SCC, FR2 SCC and FR2 inter-freq MOs without measurement gap in one FR1 + two FR2 searcher scenario will be roughly half of that of existing legacy scenario.

That means, the assumption of one FR1 + two FR2 searchers will reduce the L3 delay of measuring SCCs and inter-freq MOs without measurement gap by approximately a factor of 2. Similarly, two FR1 + one FR2 and three FR2 searcher assumptions will significantly reduce the L3 delay of inter-freq measurement without measurement gap.

Observation 1: The assumption of three searchers significantly reduces the L3 delay of measuring FR2 SCCs and inter-freq  MOs without measurement gap.

- For example, assuming the number of SCCs and inter-freq MOs to be equally distributed in an FR1 + FR2 CA (FR2 Pcell) scenario, the assumption of one FR1 + two FR2 searchers will reduce the L3 delay of measuring SCCs and inter-freq MOs without measurement gap by approximately a factor of 2.

- The CSSF table in FR1 + FR2 CA (FR2 PCell) takes the form of Table 3 with and without three searcher assumption.

Table 3: CSSF in FR1 + FR2 CA (FR2 PCell) Scenario

| # of searchers |  | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Two searchers across FR1 & FR2 | N/A |  | NSCC_SSB + Y | 1 | N/A | NSCC_SSB + Y | NSCC_SSB + Y |
| One FR1 + Two FR2 searchers | N/A |  | Ceil(0.5 * (NSCC_SSB + Y)) | 1 | N/A | Ceil(0.5 * (NSCC_SSB + Y)) -1 | Ceil(0.5 * (NSCC_SSB + Y)) -1 |
| NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NSCC_SSB is equally distributed between FR1 and FR2. Y is equally distributed between FR1 and FR2, too. Note: CSI-RS based L3 measurement is not configured in PCC and SCC. |  |  |  |  |  |  |  |

As [3] shows, the processing capability of UE already matches that of three searchers in certain legacy scenarios. For example, when one UE measures within gap in one frequency range and another UE measures without gap in another frequency range, legacy rules require UE to apply three searchers.

Observation 2: When one UE measures within gap in one frequency range and another UE measures without gap in another frequency range, legacy rules require UE to apply three searchers.

The complexity associated with supporting three searchers in FR2 is greater than that of two FR2 + one FR1 searchers. Some UE might be able to support two FR2 + one FR1 searchers but not three FR2 searchers. Hence, it makes sense to allow to indicate whether it supports three FR2 searchers or two FR2 plus one FR1 searchers.

Observation 3:  Some UE might be able to support two FR2 plus one FR1 searchers but not three FR2 searchers.

Proposal 1: RAN4 introduces a new optional capability to allow UE to signal support of three searchers for CSSF enhancement.

- Note: RAN4 can consider UE indicating the support of different levels of three searchers in the new capability:

  - Indication 00: UE supports three searchers in FR1 only (CA and DC) scenario.

  - Indication 01: UE supports three searchers in both FR1 (CA and DC) and FR1 + FR2 (CA and DC) scenarios.

  - Indication 10: UE supports three searchers in all (FR1, FR1+FR2 and FR2) (CA and DC) scenarios.

## Measurement of One Serving Cell MO Among Multiple Ones in Same Band

During last meeting, companies discussed different options to enable UE to measure one serving cell MO among multiple configured ones in the same band.

The proponents discuss this option because multiple serving CCs in the same band could be co-located and they want to reduce UE’s measurement requirements through this option.

The FFS of last meeting does not allow network to indicate the serving CC that UE needs to measure. If network doesn’t know the serving CC that UE is measuring, network cannot know how UE is classifying SCells between “known” and “unknown” category. Hence, network needs to control the SCells’ servingCellMO that UE needs to measure.

Observation 4: Network needs to control the SCells’ servingCellMO that UE needs to measure.

- If network doesn’t know the serving CC that UE is measuring, network cannot know how UE is classifying SCells between “known” and “unknown” category.

During initial configuration of a SCell, network can choose to configure or not configure servingCellMO. ServingCellMO IE is optional for SCells (see Appendix a).

Observation 5: During initial configuration of a SCell, network can choose to configure or not configure servingCellMO. ServingCellMO IE is optional for SCells

RAN2 spec shows that servingCellMO is a “Need M” IE for SCells. Hence, once network configures servingCellMO for a SCell, UE will retain the config of this servingCellMO even if network sends servingCellConfig without including servingCellMO IE in it.

However, even legacy spec allows network to remove servingCellMO from a Scell in two steps after initially configuring the Scell with servingCellMO.

- Step 1: Network releases the Scell using SCellToReleaseList IE.

- Step 2: Network adds the Scell using SCellToAddList IE without configuring servingCellMO for that Scell.

Observation 6: Legacy spec allows network to remove servingCellMO from a Scell in two steps after initially configuring the Scell with servingCellMO.

- Step 1: Network releases the Scell using SCellToReleaseList IE.

- Step 2: Network adds the Scell using SCellToAddList IE without configuring servingCellMO for that Scell

After step 2, UE will not have configuration for the MO of that particular serving cell. Obviously, UE won’t have to measure any MO associated with that serving cell.

Each SCell addition and release message causes 1 ms interruption to other inter-band cells and roughly 6ms interruption to other intra-band cells. However, network does not need to remove servingCellMO from a Scell frequently. Hence, the Scell release and addition based option won’t cause too much interruption, in total, and the whole procedure can be implemented through legacy signalling and procedure.

Observation 7: Network does not need to remove servingCellMO from a Scell frequently. Hence, the interruption due to Scell release and addition-based removal of servingCellMO should not be an issue.

Proposal 2: RAN4 does not introduce any new procedure to allow UE to measure one serving CC per band if multiple serving CCs are configured per band.

- Note: Network can use legacy signalling to inform UE to measure MO corresponding to one or multiple serving CCs per band.

# 4.	Conclusion

Observation 1: The assumption of three searchers significantly reduces the L3 delay of measuring FR2 SCCs and inter-freq  MOs without measurement gap.

- For example, assuming the number of SCCs and inter-freq MOs to be equally distributed in an FR1 + FR2 CA (FR2 Pcell) scenario, the assumption of one FR1 + two FR2 searchers will reduce the L3 delay of measuring SCCs and inter-freq MOs without measurement gap by approximately a factor of 2.

- The CSSF table in FR1 + FR2 CA (FR2 PCell) takes the form of Table 3 with and without three searcher assumption.

Table 3: CSSF in FR1 + FR2 CA (FR2 PCell) Scenario

| # of searchers |  | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Two searchers across FR1 & FR2 | N/A |  | NSCC_SSB + Y | 1 | N/A | NSCC_SSB + Y | NSCC_SSB + Y |
| One FR1 + Two FR2 searchers | N/A |  | Ceil(0.5 * (NSCC_SSB + Y)) | 1 | N/A | Ceil(0.5 * (NSCC_SSB + Y)) -1 | Ceil(0.5 * (NSCC_SSB + Y)) -1 |
| NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NSCC_SSB is equally distributed between FR1 and FR2. Y is equally distributed between FR1 and FR2, too. Note: CSI-RS based L3 measurement is not configured in PCC and SCC. |  |  |  |  |  |  |  |

Observation 2: When one UE measures within gap in one frequency range and another UE measures without gap in another frequency range, legacy rules require UE to apply three searchers.

Observation 3:  Some UE might be able to support two FR2 plus one FR1 searchers but not three FR2 searchers.

Proposal 1: RAN4 introduces a new optional capability to allow UE to signal support of three searchers for CSSF enhancement.

- Note: RAN4 can consider UE indicating the support of different levels of three searchers in the new capability:

  - Indication 00: UE supports three searchers in FR1 only (CA and DC) scenario.

  - Indication 01: UE supports three searchers in both FR1 (CA and DC) and FR1 + FR2 (CA and DC) scenarios.

  - Indication 10: UE supports three searchers in all (FR1, FR1+FR2 and FR2) (CA and DC) scenarios.

Observation 4: Network needs to control the SCells’ servingCellMO that UE needs to measure.

- If network doesn’t know the serving CC that UE is measuring, network cannot know how UE is classifying SCells between “known” and “unknown” category.

Observation 5: During initial configuration of a SCell, network can choose to configure or not configure servingCellMO. ServingCellMO IE is optional for SCells

Observation 6: Legacy spec allows network to remove servingCellMO from a Scell in two steps after initially configuring the Scell with servingCellMO.

- Step 1: Network releases the Scell using SCellToReleaseList IE.

- Step 2: Network adds the Scell using SCellToAddList IE without configuring servingCellMO for that Scell

Observation 7: Network does not need to remove servingCellMO from a Scell frequently. Hence, the interruption due to Scell release and addition based removal of servingCellMO should not be an issue.

Proposal 2: RAN4 does not introduce any new procedure to allow UE to measure one serving CC per band if multiple serving CCs are configured per band.

- Note: Network can use legacy signalling to inform UE to measure MO corresponding to one or multiple serving CCs per band.

# 5.  Reference

[1] R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN3 #113.

[2] 3GPP TS 38.331 V 18.2.0

# 6. Appendix A: ServingCellMO and SCellConfig in RAN2 Spec

Following spec snippets are obtained from 38.331 [2]:

sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig OPTIONAL, -- Need N sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex OPTIONAL, -- Need N ..., SCellConfig ::= SEQUENCE { sCellIndex SCellIndex, sCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Cond SCellAdd sCellConfigDedicated ServingCellConfig OPTIONAL, -- Cond SCellAddMod ..., …. } ServingCellConfig ::= SEQUENCE { …. servingCellMO MeasObjectId OPTIONAL, -- Cond MeasObject ..., [[ Conditional Presence; Explanation. AsyncCA; This field is mandatory present for SCells whose slot offset between the SpCell is not 0. Otherwise it is absent, Need S.. MeasObject; This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. For RedCap UEs, this field is optionally present, Need M..