3GPP TSG-RAN4 Meeting #114	R4-2502168

# Athens Greece, 17th – 21st February, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for Positioning Accuracy

Agenda item:	7.19.4

Document for:	Discussion

# Introduction

In this paper, we focus on RRM core requirements for positioning accuracy

# Prior Agreements

Following agreement happened a couple of meetings ago [1].

Issue 3-2: Requirements for case 2a Agreement: RAN4 should not define any positioning accuracy requirements because positioning is LMF based Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS Issue 3-5: Reported metrics for case 2b Agreement: postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported. Issue 3-6: Reported metrics for case 3a/3b Agreement: postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

The following agreement happened during the last meeting [5]

Issue 3-1: Requirements for case 1 Agreement: Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible Issue 3-3: Report applicability for existing reported metrics Agreement: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b.

The AI/ML WID shows the following priority for different cases of positioning [2]:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

# Discussion

## Case 2a and 2b

A paper submitted to RAN plenary [3] quoted the recent guidance from RAN2 chair regarding 2nd priority use cases:

#### 8.1.2.3 LCM for Positioning use case Contributions should focus on UE-sided model, but can discuss NW-sided model and should focus on 1st priority positioning use cases

Based on this RAN2 guidance, RAN plenary included the following note in AI/ML WID [2]: “Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.”

Since there is limited time left in Rel-19, RAN4 should focus on 1st priority work items and skip defining new requirements for 2nd priority work items.

Since neither RAN1 nor RAN2 will discuss proposals related to 2nd priority work items (including LOS/NLOS metric of case 2a) in Q4 2024, RAN4 should postpone defining requirement for LOS/NLOS until RAN1/RAN2 make further progress regarding case 2a and this metric.

Observation 1: RAN2 chair has guided companies to focus on 1st priority positioning use cases. RAN plenary has suggested RAN1 not to discuss any proposal targeting 2nd priority issues in Q4 of 2024.

Proposal 1: RAN4 postpones defining requirements for 2a and 2b until RAN1 and RAN2 make further progress regarding the 2nd priority work items.

## Case 1

UE based positioning with UE side model is a 1st priority item of AI/ML positioning. RAN4 accuracy tests were not defined for legacy (non AI-ML) UE based positioning algorithm with UE side model.

However, legacy spec has previously defined the signalling for reporting UE’s location via DL-TDoA IE in non-AI-ML scenario [4]. It is expected that the same DL-TDoA will be used to report UE’s position with UE-sided AI/ML model, too.

-- ASN1START NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { systemFrameNumber-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, ... } OPTIONAL, ..., [[ locationCoordinates-r17 LocationCoordinates OPTIONAL, -- Cond batch1 locationSource-r17 LocationSource-r13 OPTIONAL -- Cond batch2 ]] } -- ASN1STOP

Observation 2: UE based positioning with UE side model is a 1st priority item of AI/ML positioning.

Observation 3: Legacy spec has previously defined the signalling for reporting UE’s location via DL-TDoA IE in non-AI-ML scenario.

RAN2 has recently agreed to LTE positioning protocol (LPP) based UE reporting of inferred location in case 1 [7]

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

Step 6 of above RAN2 agreement allows network to collect UE’s inferred location in one report via LPP.

Observation 4: RAN2 has agreed to LTE positioning protocol (LPP) based UE reporting of inferred location for LCM of case 1.

Proposal 2: RAN4 defines requirements for case 1, i.e., UE-based positioning with UE-side model.

# Conclusions

Observation 1: RAN2 chair has guided companies to focus on 1st priority positioning use cases. RAN plenary has suggested RAN1 not to discuss any proposal targeting 2nd priority issues in Q4 of 2024.

Observation 2: UE based positioning with UE side model is a 1st priority item of AI/ML positioning.

Observation 3: Legacy spec has previously defined the signalling for reporting UE’s location via DL-TDoA IE in non-AI-ML scenario.

Observation 4: RAN2 has agreed to LTE positioning protocol (LPP) based UE reporting of inferred location for LCM of case 1.

Proposal 1: RAN4 postpones defining requirements for 2a and 2b until RAN1 and RAN2 make further progress regarding the 2nd priority work items.

Proposal 2: RAN4 defines requirements for case 1, i.e., UE-based positioning with UE-side model.

# References

[1] R4-2414323, “WF on AI/ML”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112.

[2] RP-2423999, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN 105, Sep 2024.

[3] RP-242030, “Checkpoint for AI/ML PHY”, Ericsson.

[4] 37.355, “LTE Positioning Protocol”, V 18.3.0

[5] R4-2417212, “WF on requirements for AI/ML air interface”, 3GPP TSG-RAN WG4 Meeting #112bis

[6] Chairman’s Notes, 3GPP TSG RAN WG1 #116-bis, April 2024

[7] Chairman’s Notes, 3GPP TSG RAN WG2 #127-bis, Oct 2024