3GPP TSG-RAN WG4 Meeting #114	R4-2502178

Athens, Greece, February 17 – 21, 2025

Agenda Item:	7.26.4.1

Source:	MediaTek Inc.

Title:	Simulation assumptions for R19 LP-WUS

Document for:	Discussion

# Introduction

In the last meeting, RAN4 discussed the simulation assumptions for LP-WUS/WUR in FR1, and some agreements were achieved as captured in the WF [1]. In this contribution, we further discuss the remaining aspects in simulation assumption for RRM performance evaluation for LP-WUS/WUR UE, including the general parameters, cell specific parameters and UE specific parameters. In addition, we provide some initial simulation results based on the simulation assumption provided.

# Simulation assumptions and results

One of the issues discussed in RAN4 was the definition of the target SNR for LP-WUR RRM measurements as captured in the WF [1].

Issue 2-1-1: SINR setting - Proposals - P1: Include SINR = -6dB to the {-3dB; -0.5dB; 2dB} set for simulation assumptions. FFS whether -6dB is a low priority case. (Apple MTK) - P2: As baseline, RAN4 to use -3dB SINR as side condition of accuracy requirements for both OOK based and OFDM based LR.(Huawei)

For the RRM simulation purpose only, it was agreed to use -3dB, 0.5dB, 2dB dB Ês/Iot value for serving cell measurement for both OOK based and OFDM based LP-WUR. These values were selected based on the previous agreement made in RAN1 (RAN1#117). On the other hand, the side condition requirement was still FFS and RAN4 needs to discuss the value considering the measurement inaccuracy of LP-WUR UE. It is also noted that RAN1 have made further agreement in the meeting (RAN1#118) on the RRM measurement accuracy as captured below:

Agreement (RAN1) - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report.

Therefore, RAN4 can discuss the side condition requirements taking into account the current agreement in RAN1. Considering the two values of SNR from RAN1 (-3dB and -6dB), RAN4 to discuss the worst case scenario of SNR=-6dB based on RAN1 agreement to define the side condition for RRM measurement accuracy.

Proposal 1: RAN4 to consider the worst-case scenario of SNR=-6dB based on RAN1 agreement to define the side condition for RRM measurement accuracy for LP-WUS capable UE.

Issue 2-1-4: On LP-SS sequence for simulation purpose

For Issue 2-1-4, on LP-SS binary sequence used by OOK-based LP-WUR, we propose to use L=24 for OOK-4 (M=2) and L=56 for OOK-4 (M=4). Note that, the binary sequence is selected to achieve the target timing accuracy agreed by RAN1 and RAN4 two meetings ago as captured below.

Issue 2-1-3: Time/frequency error Agreement: Table 2: Cell-specific parameters Parameter; Unit; Cell 1; Cell 2. Frequency offset relative to UE frequency reference; Hz; OFDM based receiver [5] ppm OOK based receiver [5 10] ppm; N/A. Timing error; Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: - T= 5us for OOK-1 - T= 2us for OOK-4 with M=2 - T= 1us for OOK-4 with M=4.

Proposal 2: For OOK-based LP-WUR, use LP-SS binary sequence L=24 for OOK-4 (M=2) and L=56 for OOK-4 (M=4). The binary sequence is selected to meet the target timing accuracy agreed by RAN1 and RAN4.

Furthermore, the overall simulation assumption list for RRM performance evaluation for LP-WUS/WUR UE is provided below including the general parameters, cell specific parameters and UE specific parameters.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | Data, SSB and LP-SS have the same SCS |
| Subcarrier spacing | 30KHz |
| Measurement period (in number of measurement samples) | 5 |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-4 with M = 2,4 | when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 |
| LP-SS binary sequence |  | L=24 for OOK-4 with M = 2 L=56 for OOK-4 with M = 4 | L=24 for OOK-4 with M = 2 L=56 for OOK-4 with M = 4 |
| LP-SS pattern |  | M sequence with circular extension to match the LPSS length | M sequence with circular extension to match the LPSS length Note: Cell 2 utilizes a different LP-SS pattern than Cell 1 |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [5 10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: o T= 5us for OOK-1 o T= 2us for OOK-4 with M=2 o T= 1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | When Ês/Iot = -3 dB - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
| Ês/Iot | dB | -3dB | N/A |
| Propagation conditions | - | FR1: TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | 3rd Order Butterworth with 3.84MHz bandwidth |
| --- | --- |
| [Receiver ADC bit width] | 4-bit ADC |
| [Receiver Sampling Rate for LP-SS only] | 7.68MHz |
| Receiver architecture | I-branch only for OOK-WUR |

Based on the above simulation assumption, we also provide some preliminary simulation results as shown in  Table 4. The simulation results are demonstrated in terms of delta RSRP and are achieved at 90 percentiles of the CDF for the measured LP-SS.

Table 4: RSRP error (dB) in TDL-C channel

| RSRP error in dB (90%) |  |  |  |  |
| --- | --- | --- | --- | --- |
| SINR= -3dB SNR (Cell 1) = -2.4 SNR (Cell 2) = -8.4 | Number of samples | 1 sample | 3 samples | 5 samples |
|  | OOK-4 (M=2) L=24 binary sequency | 1.67 | 0.91 | 0.74 |
|  | OOK-4 (M=4) L=56 binary sequency | 1.61 | 0.91 | 0.72 |

The simulation results above demonstrate the RSRP error using different number of samples in TDL-C channel. It can be noticed that even with 1 sample measurement LP-WUS capable UE can achieve satisfactory results for the given simulation assumption in Table 1,2, and 3. However, to select number of samples for RRM performance evaluation for LP-WUS capable UE, we propose to use at least 3 or 5 samples to guarantee achieving the required performance (as 1 sample might be impacted due to deep fading and might not be detected by the LP-WUS capable UE).

Proposal 3: For OOK-based LP-WUR, use 3 or 5 samples for RSRP measurements.

In addition, for the simulation results obtained for OOK-based LP-WUR, it is our understanding that the ideal RSRP and the measured RSRP are derived based on the assumption that the OOK-based LP-WUR is equipped with I-branch only (even though the transmitted LP-SS is an I/Q signal). Therefore, RAN4 needs to align on how the RSRP measurement results are obtained for OOK-based LP-WUR, otherwise results in RAN4 might not be comparable.

Proposal 4: The ideal RSRP and the measured RSRP are derived based on the assumption that the OOK-based LP-WUR is equipped with I-branch only, and RAN4 needs to align the understanding for the obtained results.

# Summary

In this contribution, the following proposals are concluded:

Proposal 1: RAN4 to consider the worst-case scenario of SNR=-6dB based on RAN1 agreement to define the side condition for RRM measurement accuracy for LP-WUS capable UE.

Proposal 2: For OOK-based LP-WUR, use LP-SS binary sequence L=24 for OOK-4 (M=2) and L=56 for OOK-4 (M=4). The binary sequence is selected to meet the target timing accuracy agreed by RAN1 and RAN4.

Proposal 3: For OOK-based LP-WUR, use 3 or 5 samples for RSRP measurements.

Proposal 4: The ideal RSRP and the measured RSRP are derived based on the assumption that the OOK-based LP-WUR is equipped with I-branch only, and RAN4 needs to align the understanding for the obtained results.

# References

- R4-2420100, WF on RRM requirements for NR_LPWUS, vivo.