3GPP TSG-RAN WG4 Meeting # 114	R4-2502182

Athens, GR, Feb 17 – Feb 21, 2025

Agenda Item: 7.22.3.1

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for UE-initiated/event-driven beam management

Document for: Discussion

# Introduction

According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WID [1] is divided into two items to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “UE-initiated/event-driven beam management”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- RRM requirement on UE-initiated/event-driven beam management

- Capability on UE-initiated/event-driven beam management

## 2.1 RRM requirement on UE-initiated/event-driven beam management

In the last meeting, RAN4 reached the following agreements as captured in the WF [2]:

Issue 1-1: How to define Event triggered measurement reporting delay? < Agreement> - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air. - Ending point is the point when the UE transmits a first PUCCH triggered by the event. Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay? < Agreement> Agreement: For specifying the RAN4 delay requirement for event 2 - Measure both the current beam and new beam after the starting point, FFS the details. - Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion. - Start the RAN4 RRM discussion for FR2. - Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS. Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2? Case 1: no time window based Case 2: with time window based, M is configurable < Agreement> Agreement: It is RAN4 common understanding that no additional filtering applies for case 2 compared to case 1, if RAN4 will define requirement for case 2. Issue 1-4: Reporting delay requirements <Way forward > - Option 1: (Qualcomm) - Tgap + TL1-RSRP-measure; TL1-RSRP-measure in Issue 1-2; The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on; In test cases, set Tgap = 0; - Option 2: (Nokia) - The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event. - Option 3: (Xiaomi) - If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams. - Option 4: (Samsung) - Need further RAN1 progress for RSs relationship between current beam and new beams. - Option 5: (CMCC) - The event triggered measurement reporting delay shall be less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index. The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on - Option 6: (Apple) - The total reporting delay is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH, Add TPucch in reporting delay - Option 7: (MediaTek) - The total reporting delay is TL1-RSRP_measure + Tfirst UL channel, TL1-RSRP_measure is base on SSB or CSI-RS - Event evaluation period for a SSB or CSI-RS is same as their respective measurement period. - Option 8: (Ericsson) - Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel - RAN4 to discuss components and criteria for defining TTime to first UL channel. Issue 1-7: CSI-RS based L1-RSRP for inter-cell BM <Way forward > - Option 1: (Qualcomm, Nokia, Samsung, MediaTek) - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case. - Option 2: (CMCC) - For reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused. - Option 3: (Apple, Huawei) - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. - Option 4: (ZTE) - RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. - Option 5: (Ericsson) - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2 - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

According to the recent LS sent by RAN1 [3] as shown below, RAN1 made agreement that the periodicity of current beam is the same as new beam for event 2 and the evaluation period is the same as well. Based on this agreement, we can discuss a unified measurement and reporting delay requirement for event 2 including both current beam and new beam.

Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting.

We provide our view on how to specify measurement, evaluation, and reporting delay requirement in the following cases:

- Case 1: no time window based.

- Case 2: with time window based, M is configurable.

  - M is the event instance counter for UE to check if meet the event condition. E.g., M=2, UE start to initiate beam reporting after the number of instances for event 2 (i.e., the quality of new beam is better than the current beam by a threshold configured by NW) is larger than or equal to 2 (M).

In our understanding, case 1 is kind of single event instance without time window. The number of samples measured for single event is still not decided yet. Since the quality of beam may change a lot in one sample, it’s better to consider more samples to avoid UE sending frequent RS beam reporting. Same as legacy, the number of samples could be 1 or 3 for L1-RSRP measurement. On the other hand, case 2 could be multiple events instances based on M counter. The number of samples is equal to 1 based on the agreement on Issue 1-3. Therefore, the L1-RSRP measurement period of event 2 for both case 1 and case 2 could be defined as below:

- TL1-RSRP_measure = ceil(M*P*N) * TRS

- M: number of samples for L1-RSRP measurement.

  - M = 1 or 3 for case 1 (no time window)

  - M = 1 for case 2 (with time window)

- P: sharing factor, same as legacy.

- N: beam sweeping factor, same as legacy.

- TRS is the periodicity of reference signal (RS). The RS could be either SSB or CSI-RS for current beam and new beam.

After L1-RSRP measurement, UE start to evaluate if event 2 condition is met. The difference of evaluation period between case 1 and case 2 is “time window” and “event instance counter”. In case 2, UE initiate beam reporting only if the number of instances for event 2 (i.e., the quality of new beam becomes a threshold value better than current beam) is larger than or equal to event instance counter. In addition, the case 2 evaluation should be done within time window. Otherwise, UE should re-evaluate whether the event condition is met since time window is changed.

Take above into account, the evaluation period of event 2 beam management could be defined as below:

- For case 1: Tevaluate_L1_event = TL1-RSRP_measure + Tuncertainty

  - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS.

  - Tuncertainty: the uncertainty time to fulfill the event criteria. The value could be zero or times of TL1-RSRP_measure.

- For case 2: Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window

  - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS

  - C: event instance counter is configured by NW.

  - Ttime_window: time window is configured by NW.

  - Tuncertainty: the uncertainty time to fulfill the event criteria. The value could be zero or times of TL1-RSRP_measure.

After UE evaluate and meet the event 2 condition, it start to send first PUCCH to notify the NW of the event beam reporting. For both case 1 and case 2, the measurement reporting delay could be defined as:

- Treporting_L1_event = Tevaluate_L1_event + Tfirst UL channel

  - Tevaluate_L1_event: the evaluation time of event trigger beam for case 1 or case 2.

  - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Since there’s no L1-RSRP measurement for CSI-RS on inter-cell in legacy. To save time, we prefer to postpone the discussion until the enhancement of existing requirements are finished.

Proposal 1: Reuse legacy measurement period for event 2 on UE-initiated/event-driven beam management.

- SSB based L1-RSRP measurement for both intra-cell and inter-cell.

- CSI-RS based L1-RSRP measurement for intra-cell.

  - Descope CSI-RS based L1-RSRP measurement for Inter-cell case.

Proposal 2: RAN4 to consider a unified measurement delay requirement of event 2 for both case 1 (no time window based) and case 2 (with time window based):

- TL1-RSRP_measure = ceil(M*P*N) * TRS

- M: number of samples for L1-RSRP measurement.

  - M = 1 or 3 for case 1.

  - M = 1 for case 2.

- P: sharing factor, same as legacy.

- N: beam sweeping factor, same as legacy.

- TRS is the periodicity of reference signal (RS). The RS could be either SSB or CSI-RS for current beam and new beam.

Proposal 3: RAN4 to consider the reporting delay requirement of event 2 for both case 1 and case 2:

- Tevent_reporting_delay = Tevaluate_L1_event + Tfirst UL channel

  - Tevaluate_L1_event: the evaluation period of event 2 for current and new beam.

    - For case 1: Tevaluate_L1_event = TL1-RSRP_measure + Tuncertainty

    - For case 2: Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window

      - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS.

      - Tuncertainty: the uncertainty time to fulfill the event criteria. The value could be zero or times of TL1-RSRP_measure.

      - C: event instance counter is configured by NW.

      - Ttime_window: time window configured by NW

  - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Issue 1-8: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM <Way forward > - Option 1: (Qualcomm, Nokia, Samsung, Apple, Huawei) - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM - Option 2: (vivo) - Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions. - Option 3: (ZTE) - Need to define the condition of known cell for UEIBM. The definition in ICBM can be starting point. Issue 1-9: Whether to specify new unified TCI switching delay requirements? <Way forward > - Option 1: (Qualcomm, Nokia, Samsung (For known conditions for TCI states), Apple, ZTE, Ericsson) - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management. - Option 2: (Samsung) - No unknown case is applied if the TCI states switching is based on UEIBM. - Option 3: (ZTE) - Need to decide whether to define L1-RSRP requirements for unknown cell

In our understanding, after UE send L1 event beam reporting to NW, it can be expected that there will be TCI state switching command to target event beam from NW. Since there’s no new behavior on TCI state switching for event beam management. The existing known/unknown condition for TCI state and unified TCI switching delay requirement could be reused.

Proposal 4: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM.

Proposal 5: The existing requirements for TCI state switch delay should be reused for UE-initiated/event-driven beam management.

Issue 1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting? <Way forward > - Option 1: (Nokia) - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting. - Option 2: (Xiaomi, Samsung, Apple, Ericsson, MediaTek) - Reuse current beam sweeping factor for UE/event triggered beam reporting Issue 1-13: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward > - Option 1: (Qualcomm, Nokia, Xiaomi, Apple, MediaTek) - The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements. - Option 2: (Samsung) - Focus on core requirements firstly, FFS on accuracy requirements. - Option 3: (CMCC) - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam report, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account. - Option 4: (ZTE) - Discuss the accuracy for the two trigger-event solutions respectively. - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement;

The beam sweeping enhancement on multi-RX in Rel-18 is under multi-TRP scenario. In Rel-19, UE-initial event beam reporting is targeting at FR2 and single TRP. Therefore, beam sweeping enhancement is kind of not in the scope in our understanding.

Proposal 6: RAN4 do not consider beam sweeping enhancement on UE event triggered reporting since it is out of scope in this WI.

For the accuracy requirement, RAN4 to reuse legacy requirement since there’s no new measurement metrics except for L1-RSRP for event triggered reporting.

Proposal 7: RAN4 to reuse the legacy L1-RSRP accuracy requirement for UE-initiated/event-driven beam management.

## 2.2 Capability on UE-initiated/event-driven beam management

Issue 1-11: Capabilities for Support of Event Triggering and Reporting Criteria? <Way forward > - Option 1: (Nokia) - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item. - Option 2: (Apple) - Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added. - Option 3: (Ericsson) - Define UE capability for total number of events supported for UE-initiated beam management across single and multiple TRP. - Option 4: (MediaTek) - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency. Issue 1-12: Compatibility of capabilities with LTM <Way forward > - Option 1: (Nokia) - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item. - Option 2: (Samsung, Apple) - Do not combine capabilities across different L1 measurements. - Option 3: (Ericsson) - RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility. - Option 4: (MediaTek) - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency. TS 38.133 Table 9.1.4.2-1: Requirements for reporting criteria per measurement category Measurement category; Ecat; Note. Intra-frequency Note 1,2,3,4,5; 9; Events for any one or a combination of intra-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NGRAN intra-frequency cells. Inter-frequency Note 2,3,4,5; 10; Events for any one or a combination of inter-frequencyvSS-RSRP, SS-RSRQ, SS-SINR, CSIRSRP, CSI-RSRQ, and CSI-SINR for NG-RANvinter-frequency cells. …; …; …. NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode.

In legacy, capabilities for support of event triggered L3 measurement reporting had been specified in TS 38.133 section 9.1.4. For event-based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in Table 9.1.4.2-1.

In our understanding, we could follow the similar principle to define the number of events supported for UE-initiated beam reporting in MIMO. Since we’re targeting FR2 and sTRP with intra- and inter-cell beam management, the number of events for UE-initiated beam reporting should be specified for intra-frequency.

Proposal 8: Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

We prefer to specify a unified capability that indicates the total number of L1 events supported for mobility and MIMO. This allows UE with more flexibility to utilize the whole capability of L1 events compared to share the total number of L1 events to separate UE capability.

Proposal 9: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

# Summary

In this paper, the discussion of R19 MIMO is provided. We have the following proposals:

Proposal 1: Reuse legacy measurement period for event 2 on UE-initiated/event-driven beam management.

- SSB based L1-RSRP measurement for both intra-cell and inter-cell.

- CSI-RS based L1-RSRP measurement for intra-cell.

  - Descope CSI-RS based L1-RSRP measurement for Inter-cell case.

Proposal 2: RAN4 to consider a unified measurement delay requirement of event 2 for both case 1 (no time window based) and case 2 (with time window based):

- TL1-RSRP_measure = ceil(M*P*N) * TRS

- M: number of samples for L1-RSRP measurement.

  - M = 1 or 3 for case 1.

  - M = 1 for case 2.

- P: sharing factor, same as legacy.

- N: beam sweeping factor, same as legacy.

- TRS is the periodicity of reference signal (RS). The RS could be either SSB or CSI-RS for current beam and new beam.

Proposal 3: RAN4 to consider the reporting delay requirement of event 2 for both case 1 and case 2:

- Tevent_reporting_delay = Tevaluate_L1_event + Tfirst UL channel

  - Tevaluate_L1_event: the evaluation period of event 2 for current and new beam.

    - For case 1: Tevaluate_L1_event = TL1-RSRP_measure + Tuncertainty

    - For case 2: Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window

      - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS.

      - Tuncertainty: the uncertainty time to fulfill the event criteria. The value could be zero or times of TL1-RSRP_measure.

      - C: event instance counter is configured by NW.

      - Ttime_window: time window configured by NW

  - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Proposal 4: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM.

Proposal 5: The existing requirements for TCI state switch delay should be reused for UE-initiated/event-driven beam management.

Proposal 6: RAN4 do not consider beam sweeping enhancement on UE event triggered reporting since it is out of scope in this WI.

Proposal 7: RAN4 to reuse the legacy L1-RSRP accuracy requirement for UE-initiated/event-driven beam management.

Proposal 8: Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

Proposal 9: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

# Reference

[1] RP-233090 New WID - Rel-19 MIMO

[2] R4-2420096, WF on RRM requirements for NR_MIMO_Ph5_Part1 (Samsung)

[3] R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management