3GPP TSG-RAN WG4 Meeting #114									                        R4-2502197

Athens, Greece, February 17 – 21, 2025

Agenda item:			7.10.3.1

Source:	Ericsson

Title:	On system parameters for less than 5 MHz CBW for FR1-NTN

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify less than 5 MHz for NTN with the following objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

In this contribution, we focus on the remaining issues for the system parameters for channel bandwidths of less than 5 MHz for NR-NTN in FR1-NTN bands from the approved WF [2], and we provide our view on them.

# Discussion

In the approved WF [2], the three following issues are FFS:

Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

- Option 1: Optional

- Option 2: Mandatory

In TN, 3MHz channel bandwidth is optional for some (n26, n28, n85 and n100) and mandatory for other (n31, n72 and n106) bands, where all bands which support it are <1GHz FDD bands. n31, n72 and n106 are all narrow bands with only 5 MHz available bandwidth on DL/UL, which is not the case for FR1-NTN bands. Since the usual practice is to make the support for a new channel bandwidth optional in the Release where it is introduced unless there is a specific request from an operator, we propose to proceed in the same way for 3 MHz CBW for FR1-NTN bands:

Proposal 1: 3MHz channel bandwidth for NR-NTN in FR1-NTN bands should be optional in this Release unless there is a specific request from an operator for some band.

Issue 1-2: UE capability Signalling

- FFS on re-use of existing rel-18 signalling parameters

For FR1 TN bands, support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 UE capabilities are currently defined in TS38.306 [3]. For FR1-NTN bands, the same capabilities could be reused. However, their description should be modified – other than TS38.101-1, TS38.101-5 should also be referenced to, where in the case of support3MHz-ChannelBW-Symmetric-r18 capability the correct table reference should be used for “Applicable SS raster entries per operating band for 3 MHz channel bandwidth” table.

Proposal 2: The exiting UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 could be reused for NTN operation as well, under certain modifications.

Issue 1-3: If 3MHz is a mandatory feature, the UE behaviour to support 3MHz Channel bandwidth

- WF:

  - FFS on UE behaviours if 3MHz CHBW is mandatory for certain band.

    - Reception of 12 PRB PBCH based on RB-level puncturing;

    - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS;

    - Reception of 15 PRB CORESET0;

For NTN, SS/PBCH for 3MHz CBW should follow the same puncturing as defined in RAN1 for TN, i.e. the SSB of 12 PRBs (punctured from 20 PRBs).

According to TS38.306, UE indicating support3MHz-ChannelBW-Symmetric-r18 or support3MHz-ChannelBW-Asymmetric-r18 UE capabilities, also indicates the support for short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS.

For TN 3 MHz channel bandwidth, CORESET#0 size is defined to be 15 PRBs, which was derived from puncturing the original CORESET#0 consisting of 24 PRBs. 12 PRB CORESET#0 was defined for one specific sync raster frequency for n100, and in that case a new CORESET#0 size consisting of 12 PRBs was added (i.e., no puncturing in this case, but new size). Its usage was not precluded for the main case of 3MHz CBW where transmissions can span up to the maximum transmission bandwidth, but if in such a case transmissions can span up to 15 PRBs then is preferred to use a 15 PRBs CORESET#0 (due to a link budget).

No matter if 3MHz CBW is a mandatory or optional feature, the same design for SS/PBCH, RACH preamble formats and CORESET#0 should be followed.

Proposal 3: 3MHz CBW for NTN shall follow the same design as for TN for a general case of 3 MHz CBW, no matter if it is mandatory or optional feature:

- 12 PRB SS/PBCH obtained by puncturing a legacy 20 PRBs SS/PBCH.

- Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS.

- 15 PRB CORESET#0 obtained by puncturing a legacy 24 PRBs CORESET#0.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: 3MHz channel bandwidth for NR-NTN in FR1-NTN bands should be optional in this Release unless there is a specific request from an operator for some band.

Proposal 2: The exiting UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 could be reused for NTN operation as well, under certain modifications.

Proposal 3: 3MHz CBW for NTN shall follow the same design as for TN for a general case of 3 MHz CBW, no matter if it is mandatory or optional feature:

- 12 PRB SS/PBCH obtained by puncturing a legacy 20 PRBs SS/PBCH.

- Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS.

- 15 PRB CORESET#0 obtained by puncturing a legacy 24 PRBs CORESET#0.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2419789, “Way Forward for [113][311] NR_IoT_NTN_less_than_5MHz_UERF”, Xiaomi, RAN4#113.

- TS38.306 V18.4.0, “NR; User Equipment (UE) radio access capabilities”