3GPP TSG-RAN WG4 Meeting # 114	R4-2502199

# Athens, Greece, 17th – 21st February, 2025

Source:			T-Mobile USA, China Telecom, Ericsson

Title:			Release Independence of the 7 MHz channel bandwidth

Agenda item:		7.9.2

Document for:		Discussion

# Introduction

At RAN#106 a new Work Item to introduce a 7 MHz channel bandwidth was approved [1].

The objectives included the following:

- Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4]

  - No impact to 38.307 is expected.

- Once RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2]
Note:	This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective.

- Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3]

Note:	This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective.

# Discussion

## Release independence for channel BWs in 38.307

TS 38.307 [2] says NR channel bandwidths, all unless otherwise stated are release independent from Release 15:

Table 5.1-3: NR channel bandwidths

| - Feature | - Duplex-mode | - Release - independent from | - Requirements to be fulfilled - (see TS 38.307 of the release in which the channel bandwidth was introduced) |
| --- | --- | --- | --- |
| NR channel bandwidths, all unless otherwise stated | FDD, TDD, SDL, SUL | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Channel bandwidth of 3 MHz | FDD | Rel-18 | Table B.4.1-1 |

Observation 1: NR channel bandwidths are release independent from Rel-15 unless otherwise stated in 38.307.

Observation 2: The WID says, “No impact to 38.307 is expected.”

## 35 and 45 MHz Channel bandwidths

The 35 and 45 MHz channel bandwidths were introduced in Release 17, but are release independent from Rel-15.

Observation 3: The 35 and 45 MHz channel bandwidths were introduced in a Release 17 Work Item, but are release independent from Rel-15.

## 3 MHz Channel bandwidth

The 3 MHz channel bandwidth was introduced in Rel-18, and is only release independent from Rel-18, primarily because 3 MHz required RAN1 changes including support for a punctured SSB.

Observation 4: The 3 MHz channel bandwidth was introduced in Rel-18, and is only release independent from Rel-18, primarily because 3 MHz required RAN1 changes including support for a punctured SSB.

Observation 5: Since the 7 MHz channel bandwidth won’t require anything special like the punctured SSB, it should be release independent from Rel-15

## RAN2 signalling

Although RAN2 signalling is out of scope for RAN4, in offline discussions at least one RAN4 delegate has raised concern that 7 MHz cannot be release independent from Release 15 because the Release 15 capability signalling does not include 7 MHz. This section is being provided for information only in response to that claim.

The Release 15 ASN.1 includes spare bits for future UEs for new channel bandwidths as they are introduced. channelBWs-DL-v1590 and channelBWs-UL-v1590 are 16 bit strings, of which only 4 bits are currently used.

From 38.331 15.28.0 [3]

channelBWs-DL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL,

channelBWs-UL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL

From 38.306 version 15.26.0 [4]:

For FR1, the leading/leftmost bit in channelBWs-DL-v1590 indicates 70MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-DL-v1590 shall be set to 0.

Observation 6: The Release 15 ASN.1 has 16 bits in the channelBWs-DL-v1590 and channelBWs-UL-v1590 IEs, only 4 of which are currently allocated in 38.306. The remaining 12 are reserved for new channel bandwidths.

Observation 7: When 35 and 45 MHz were added in a Release 17 Work Item, bits were allocated in the Release 15 version of 38.307 without changing the ASN.1

There is also a SupportedBandwidth IE and a Supported Bandwidth-v1700 IE which are optionally used to indicate the maximum and minimum channel bandwidth for a given carrier in a band combination, if the supported channel bandwidths in the combination are different than the channel bandwidth that the UE supports for the band:

-- ASN1START

-- TAG-SUPPORTEDBANDWIDTH-START

SupportedBandwidth ::=      CHOICE {

fr1                         ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2                         ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}

SupportedBandwidth-v1700 ::= CHOICE {

fr1-r17    ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100},

fr2-r17    ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz800, mhz1600, mhz2000}

}

-- TAG-SUPPORTEDBANDWIDTH-STOP

-- ASN1STOP

There should be no urgent need to add 7 MHz to SupportedBandwidth, because it is unlikely that 7 MHz will be the largest or smallest channel bandwidth that a UE supports in a given band combination. But, as was the case with 35 and 45 MHz, they can still be Release independent from Release 15, even if 7 MHz is added to SupportedBandwidth after Release 15.

While it is not in RAN4’s purview to determine if it is possible to use another reserved bit in the Release-15 ANS.1 for 7 MHz, RAN4 should not rule out Release independence from Release 15 because of some perceived signalling issue. That should be up to RAN2.

Observation 8: It is up to RAN2 to decide if the signallng can support 7 MHz channel bandwidths from Release 15. RAN4 should not rule out release independence from Release 15 due to some perceived signalling issue.

## Optionality

It has long been the case in RAN4 that when a new channel bandwidth is introduced for a given band, it is optional in the release that it was introduced, and mandatory in the following release. This should help allay any concerns about Release independence forcing vendors to implement 7 MHz in earlier releases.

Observation 9: Release independence from Release 15 does not mandate support to Release 15, it only makes it optional from Release 15.

## Proposals

The above information should clarify that it is possible from a RAN4 perspective for 7 MHz to be release independent from Release 15.

Proposal 1: From a RAN4 perspective the 7 MHz channel bandwidth can be release independent from Rel-15.

Proposal 2: RAN4 should send an LS to RAN2 to see if RAN2 would have any concerns about 7 MHz being release independent from Rel-15.

# Summary

Observation 1: NR channel bandwidths are release independent from Rel-15 unless otherwise stated in 38.307.

Observation 2: The WID says, “No impact to 38.307 is expected.”

Observation 3: The 35 and 45 MHz channel bandwidths were introduced in a Release 17 Work Item, but are release independent from Rel-15.

Observation 4: The 3 MHz channel bandwidth was introduced in Rel-18, and is only release independent from Rel-18, primarily because 3 MHz required RAN1 changes including support for a punctured SSB.

Observation 5: Since the 7 MHz channel bandwidth won’t require anything special like the punctured SSB, it should be release independent from Rel-15

Observation 6: The Release 15 ASN.1 has 16 bits in the channelBWs-DL-v1590 and channelBWs-UL-v1590 IEs, only 4 of which are currently allocated in 38.306. The remaining 12 are reserved for new channel bandwidths.

Observation 7: When 35 and 45 MHz were added in a Release 17 Work Item, bits were allocated in the Release 15 version of 38.307 without changing the ASN.1

Observation 8: It is up to RAN2 to decide if the signallng can support 7 MHz channel bandwidths from Release 15. RAN4 should not rule out release independence from Release 15 due to some perceived signalling issue.

Observation 9: Release independence from Release 15 does not mandate support to Release 15, it only makes it optional from Release 15.

Proposal 1: From a RAN4 perspective the 7 MHz channel bandwidth can be release independent from Rel-15.

Proposal 2: RAN4 should send an LS to RAN2 to see if RAN2 would have any concerns about 7 MHz being release independent from Rel-15.

References

- RP-243323, “New WID on 7 MHz Channel Bandwidth for n26 and n5,” China Telecom, T-Mobile USA

- TS 38.307, “NR; Requirements on User Equipments (UEs) supporting a release-independent frequency band,” 3GPP

- TS 38.331, “NR; Radio Resource Control (RRC); Protocol specification,” version 15.28.0, 3GPP

- TS 38.306, “NR; User Equipment (UE) radio access capabilities,” version 15.26.0, 3GPP