# 3GPP TSG-RAN WG4 Meeting #114		        R4-2502233

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	6.9.2

Source: 	Huawei, HiSilicon

Title:	On UE RF requirements for band n28

Document for:	Approval

# 1	Introduction

The features of PC2 and 40MHz have been introduced for band n28 [1] with the completion of the WI. During the course of reviewing the changes, some minor issues are identified, including: tolerance for maximum output power, release independence of PC2.

# 2	Discussion

The maximum output power for PC3 and PC2 for band n28 are specified as follows [2]:

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n2 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n3 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n5 |  |  |  |  |  |  | 23 | ±2 |
| n7 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n8 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n12 |  |  |  |  |  |  | 23 | ±23 |
| n13 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n14 | 316,7 | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n18 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±23 |
| n24 |  |  |  |  |  |  | 23 | +2/-33 |
| n25 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n26 |  |  |  |  |  |  | 23 | ±23 |
| n28 |  |  |  |  | 26 | +2/-3 | 23 | +2/-2.5 |
| …… |  |  |  |  |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: Achieved via 2Tx. NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. NOTE 8: PC1 in Band n100 and n101 is allowed only for FRMCS cab-radio’s, i.e. user equipments installed on trains with external antenna on top of the train roof at approximately 4m over ground. |  |  |  |  |  |  |  |  |

The tolerance is +2/-3dB for PC2 and +2/-2.5dB for PC3. For some FDD bands, larger tolerance as specified in Note 3 is applicable, such as band n2, n3, n7, n8, etc. And the value in Note 3 is converted to ∆TC,c in the formulas for the configured transmitted power.

As explained in [3], the figure of merit to determine deltaTC is relative duplex gap:

“Relative duplex gap for example is calculated by dividing the gap width with the centre frequency value of the gap and converting the figure into percentage value.”

“All the bands that have the relative duplex gap < 1.75% should have deltaTC relaxation.”

Assuming a full-band duplexer for band n28, the relative duplex gap becomes 1.33%, which is < 1.75%. In early releases, 30MHz dual-duplexers are assumed for band n28, resulting in a relative duplex gap of 3.35%.

In order to accommodate the full-band duplexer implementation, it’s proposed to apply deltaTC to band n28 from Rel-19.

Proposal 1: Apply deltaTC (NOTE 3) to band n28 from Rel-19.

The CR for introducing release independence of 40MHz for n28 was also agreed [4].

Table 5.1-2: NR UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Power Class 1 | FDD | Rel-15 | Table B.4.1-1 |
| Power Class 1.5 | TDD | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Power Class 2 | FDD, TDD | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Power Class 3 | FDD, TDD, SUL | Rel-15 | Table B.4.1-1, Table B.4.3-1 |

Table 5.1-3: NR channel bandwidths

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the channel bandwidth was introduced) |
| --- | --- | --- | --- |
| NR channel bandwidths, all unless otherwise stated | FDD, TDD, SDL, SUL | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Channel bandwidth of 3 MHz | FDD | Rel-18 | Table B.4.1-1 |
| Channel bandwidth of 40MHz for band n281 | FDD | Rel-16 | Table B.4.1-1 |
| Note 1: The UE shall support the enhanced channel raster capability |  |  |  |

It can be seen that power class 2 is release independent from Rel-15, and 40MHz from Rel-16. Both features share the same table of requirements to be fulfilled, i.e. Table B.4.1-1, which is duplicated below from TS 38.307 [5].

Table B.4.1-1: Common UE RF requirements for a release independent band

| Clause / Clause | Description |
| --- | --- |
| 5.2 | Operating bands |
| 5.3 | UE Channel bandwidth |
| 5.4 | Channel arrangement |
| 6.2 | Transmitter power |
| 6.3 | Output power dynamics |
| 6.4 | Transmit signal quality |
| 6.5 | Output RF spectrum emissions |
| 6.6 of [3] | Beam correspondence |
| 7.3 | Reference sensitivity |
| 7.4 | Maximum input level |
| 7.5 | Adjacent Channel Selectivity |
| 7.6 | Blocking characteristics |
| 7.7 of [2] | Spurious response |
| 7.8 of [2] | Intermodulation characteristics |
| 7.9 | Spurious emissions |
| L.1 | Indication of modified MPR behavior |
| NOTE: A UE which supports any FR2 band introduced in release N, where N > 15, shall meet the requirements according to the FR2 UE multi-band relaxation factors defined in Table 6.2.1.3-4 of the release N version of [3] for all FR2 bands which it supports. |  |

Although 40MHz channel bandwidth is an optional feature for band n28 in Rel-19, there can be new features added to Clause 5.2, 5.3 or 5.4, which are mandatory. For example, the enhanced channel raster is added as mandatory for band n28 since Rel-18.

As a result, if a Rel-17 UE intends to support PC2 for band n28 via release independence, it should also implement the enhanced channel raster as required by Clause 5.4 listed in Table B.4.1-1. This could be unnecessary burden for the UE if it doesn’t intend to support 40MHz channel bandwidth at the same time.

Observation 1: The release independent features of operating bands, UE power class and channel bandwidths share the same table of common UE RF requirements, which might contain unnecessary requirements for individual features.

When adding a new power class to an existing band in FR1, usually only a few clauses need to be changed, such as clause 6.2 and 7.3. For example, if a Rel-17 UE intends to support PC2 for band n28, it shall meet all the mandatory requirements for band n28 specified in Rel-17, plus the additional requirements for PC2 as specified in Rel-19.

Observation 2: When adding a new power class to an existing band in FR1, usually only a few clauses need to be changed, such as clause 6.2 and 7.3.

In summary, a separate table to list the common UE RF requirements could be added for power class. Some of the clauses such as 5.2, 5.3, 5.4, 7.4-7.9  could be excluded from the new table. The potential changes are shown below.

Table B.4.1a-1: Common UE RF requirements for a release independent power class

| Clause / Clause | Description |
| --- | --- |
| 5.2 | Operating bands |
| 5.3 | UE Channel bandwidth |
| 5.4 | Channel arrangement |
| 6.2 | Transmitter power |
| 6.3 | Output power dynamics |
| 6.4 | Transmit signal quality |
| 6.5 | Output RF spectrum emissions |
| 6.6 of [3] | Beam correspondence |
| 7.3 | Reference sensitivity |
| 7.4 | Maximum input level |
| 7.5 | Adjacent Channel Selectivity |
| 7.6 | Blocking characteristics |
| 7.7 of [2] | Spurious response |
| 7.8 of [2] | Intermodulation characteristics |
| 7.9 | Spurious emissions |
| L.1 | Indication of modified MPR behavior |
| NOTE: A UE which supports any FR2 band introduced in release N, where N > 15, shall meet the requirements according to the FR2 UE multi-band relaxation factors defined in Table 6.2.1.3-4 of the release N version of [3] for all FR2 bands which it supports. |  |

Proposal 2: RAN4 to discuss whether to define a separate table (in TS 38.307) which includes only the necessary UE RF requirements for a release independent power class.

# 3	Conclusion

Some maintenance issues for band n28 have been discussed, and the following observations and proposals are made:

Proposal 1: Apply deltaTC (NOTE 3) to band n28 from Rel-19.

Observation 1: The release independent features of operating bands, UE power class and channel bandwidths share the same table of common UE RF requirements, which might contain unnecessary requirements for individual features.

Observation 2: When adding a new power class to an existing band in FR1, usually only a few clauses need to be changed, such as clause 6.2 and 7.3.

Proposal 2: RAN4 to discuss whether to define a separate table (in TS 38.307) which includes only the necessary UE RF requirements for a release independent power class.

# References

- RP-243079 CR on introduction of PC2 and 40MHz for n28, CMCC et al, RAN#106

- 3GPP TS 38.101-1 v19.0.0

- R4-091742 deltaTC and deltaRC definition, Nokia, RAN4#51

- R4-2420522 CR on introducing release independence of 40MHz for n28, CMCC et al, RAN4#113

- 3GPP TS 38.307 v18.2.0