3GPP TSG-RAN WG4 Meeting #114	R4- 2502235

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.24.2.2

Source: 	Huawei, HiSilicon

Title:	On the RF Requirements for Ambient IoT Devices

Document for:	Approval

# 1	Introduction

As per the Ambient IoT WID [1], the following scope and objectives are applicable for RAN4. In the sequel, we share our views on how to define the RF requirements for device 1.

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- The overall objective shall be to standardize the following Ambient IoT device:

Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Deployment scenario 1 with Topology 1, according to D1T1-B.

- FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

- Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

- Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command).

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769:

  - Case 1-4 for D1T1-B

- Proximity determination via Solution 1 in TR 38.769 only.

- Device (un)availability via Direction 1 in TR 38.769 only.

The following objectives are set, within the General Scope:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

# 2	Discussion

### 2.1 Overview

A number of Radio Frequency Identification (RFID) technologies have been developed for the UHF frequency range (840-960MHz).  Table 1 lists some of the well-known standards defined by international, regional or industrial bodies.

Table 1: UHF RFID standards

| Organisation | Standards |
| --- | --- |
| ISO/IEC | ISO/IEC 18000-6:2013 Information technology — Radio frequency identification for item management Part 6: Parameters for air interface communications at 860 MHz to 960 MHz General |
|  | ISO/IEC 18047-6:2017 Information technology — Radio frequency identification device conformance test methods Part 6: Test methods for air interface communications at 860 MHz to 960 MHz |
|  | ISO/IEC 18046-3:2020 Information technology — Radio frequency identification device performance test methods Part 3: Test methods for tag performance |
| CEPT/ETSI | ETSI EN 302 208 Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W, V3.4.1 |
| SAC/TC28 | GB/T 29768-2013 Information technology - Radio frequency identification - Air interface protocol at 800/900 MHz |
| GS1/EPCglobal | GS1 EPC Radio-Frequency Identity Generation-2 UHF RFID Standard: Specification for RFID Air Interface Protocol for Communications at 860 MHz – 930 MHz, Version 3.0 |
|  | GS1 EPC Compliant Generation-2 RFID Devices Conformance Requirements |
|  | GS1 Tagged-Item Performance Protocol (TIPP) Tagged-Item Grading: Testing Methodology Guideline |
|  | GS1 Tagged-Item Performance Protocol (TIPP) Tagged-Item Grading: Grade Definitions Guideline |

Observation 1: In general, the existing RFID standards use separate specifications to specify the requirements for protocol conformance and those for device performances. The conformance standard contain normative and often mandatory requirements that a device shall comply with; the performance standards describe the test methods to be used and performance metrics to be declared by device vendors.

For example, the GS1 UHF RFID protocol [3] has specified the following RF requirements for tags.

Figure 1: RF related requirements for UHF tags [3]

In contrast to conventional 3GPP specifications, no minimum requirements for tag sensitivity or backscatter power are defined. Instead, such performance figures are for tag vendors to declare as shown below [3].

Figure 2: Tag sensitivity and backscatter performance requirements [3]

In ETSI RFID standard [4], only two conformance requirements for tags are defined, i.e. radiated power and unwanted emissions. Given that AIoT device 1 has similar architecture and power consumption as a RFID tag, and has similar application scenarios, it’s reasonable to consider similar RF requirements to be defined in 3GPP.

Proposal 1: RAN4 to discuss and decide the minimum scope of necessary RF requirements for device 1.

Based on the discussions in the study phase, radiated performance can be considered as the baseline for device RF requirements. In contrast with conductive performance, the antenna gain pattern as well as the frequency variation of both the antenna and the matching network in the device will have considerable impact.

Proposal 2: When specifying radiated performances, the orientation of the device antenna as well as the frequency variation of the antenna and the matching network should be accounted for, in addition to the performance of the RF IC chip.

### 2.2 Receiver RF Requirements

According to the WF on A-IoT RF requirements [2], the following Rx requirements can be considered for device 1.

Table 2: Potentially needed Rx requirements [2]

| A-IoT device- RX Requirement | Potentially needed or not |
| --- | --- |
|  | Device 1 |
| Reference sensitivity | YES |
| Maximum input power | TBD |
| Interference rejection | TBD |

Receiver Sensitivity:

In 3GPP, the REFSENS is typically defined as the minimum DL signal level received by a UE to successfully decode the DL RMC (with BLER<=5%). The REFSENS of a UE is mainly determined by the receiver Noise Figure and the required SNR of the baseband demodulator. However, this may not be applicable for A-IoT devices.

In the study phase, around -36dBm sensitivity is reported by different companies. Assuming that NF=24dB, and the device can receive any signal in band n8 (i.e. BW=35MHz), the SNR at the device antenna connector is:

SNR = -36 – (-174+24+10log10(35e6) = 38.6 dB

It’s unlikely that a device needs such high SNR to demodulate the R2D signal.

As discussed in our previous contribution [6], the device sensitivity level is more likely to be determined by the physical limit of the receiver components, such as the threshold voltage of the 1-bit comparator. After the envelope detector and the BB LPF, the received voltage signal is fed to the comparator to decide whether the received bit is 0 or 1 after comparing with the inherent threshold voltage. The signal power at the antenna should be large enough for the comparator to generate the correct decisions.

Observation 2: Unlike conventional REFSENS, the device sensitivity level is more likely to be determined by the physical limits of the receiver circuitry, such as the threshold voltage of the 1-bit comparator.

Proposal 3: Conventional BLER vs SNR simulations are not needed for determining device sensitivity.

Instead, the device sensitivity is to be determined by measurement. And the measurement command sequence such as: Select, Query, RN16, ACK and EPC/CRC can be decided after RAN1 and RAN2 design are relatively stable.

Proposal 4: When measuring radiated sensitivity performance, either TRS as in FR1 or EIS as in FR2 can be considered.

Proposal 5: Whether and how to define the minimum requirement for device 1 sensitivity, consider the following options:

- Alt-1: Do not define compulsory minimum sensitivity requirement. Device vendors follow the specified test method and declare the measured performance;

- Alt-2: Define one very relaxed requirement to accommodate diverse implementations tailored for different use cases;

- Alt-3: Define the sensitivity requirement as a range (similar to REFSENS for FR2 gNB);

- Alt-4: Define multiple levels of sensitivity requirements.

Maximum input power:

The maximum input power corresponds to the scenario when the device is very close to the reader. If the input level exceeds the receiver dynamic range, the R2D signal cannot be correctly detected. Additionally, some protection circuitry such as the shunt regulator should start to function when the input level gets high. This could change the load impedance and in turn affect the backscatter, i.e., D2R performance.

Proposal 6: RAN4 to discuss whether to define a minimum limit for the maximum input level or allow device vendor to declare the performance based on 3GPP test method.

Interference rejection:

Narrowband interference from an adjacent reader could be received. And wideband interference from outdoor NR gNB is obvious for in-band deployment. It’s desired that the device can cope with both types of interference to a certain level.

Proposal 7: RAN4 to discuss whether to define a minimum requirement for the interference rejection capability or allow device vendor to declare the performance based on 3GPP test method.

### 2.2 Transmitter RF requirements

According to the WF on A-IoT RF requirements [2], the following Tx requirements can be considered for device 1.

Table 3: Potentially needed Tx requirements [2]

| Tx requirements | Device 1 | Potentially needed or not |
| --- | --- | --- |
| Transmit output power | Backscattering power or power backscattering loss | YES |
| Transmit signal quality | Modulation quality requirements | TBD |
| Output RF spectrum emissions | Occupied bandwidth | YES |
|  | SEM | TBD |
|  | Spurious emissions | TBD |
| Unwanted emissions |  | TBD |

Transmit output power:

For RFID, the minimum backscatter power is measured when the incident CW signal level is as low as close to the sensitivity level. For example [5]:

This is because the CW signal is transmitted from the same RFID reader. However, backscatter power is usually not the limiting factor of the read range in practice.

For A-IoT D1T1-B, the CW node is outside the topology. And the CW node is assumed not to be co-located with the active reader. Due to different propagation path, the incident CW signal and the R2D signal might not reach the minimum at the same time. If needed, more CW nodes than readers can be deployed.

Proposal 8: RAN4 to discuss the measurement configuration including incident CW power and orientation for measuring the backscatter power or backscatter loss.

Modulation signal quality:

As per the WID, D2R will support both OOK and BPSK, which are also supported by RFID tags. However, RFID does not have modulation signal quality requirements for tags.

For passive devices, the modulation is implemented by switching the load impedance seen by the antenna. Due to low complexity, it’s very difficult for the device to control the modulation quality. On the other hand, the modulation quality should be good enough to pass any tests that involves a response from the device.

Proposal 9: The modulation signal quality requirement can be considered as implicitly embedded in other RF or demod requirements. No explicit requirement for devices is needed.

Occupied bandwidth:

The OBW requirement depends on the discussion on the system channel bandwidth.

Proposal 10: RAN4 to discuss the OBW requirement after defining the system channel bandwidth.

SEM:

The NR SEM could be used as the starting point, since the devices operate on NR bands. Further simplification is possible, given the relatively low backscatter power. The exact mask also depends on D2R signal design, system channel BW, etc.

Proposal 11: Use NR FR1 SEM requirement as the starting point and further discuss whether to define TRP or EIRP requirement as well as potential simplifications.

Spurious emissions:

The NR general spurious emission limits can be used as the starting point, and further discuss the OOB boundary.

Table 6.5.3.1-2: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz | 4 |

Proposal 12: Use NR FR1 general spurious emission requirement as the starting point and further discuss the OOB boundary.

# 3	Conclusion

In this paper, we have shared initial analysis on the potential RF requirements for A-IoT devices. The following observations and proposals are made:

Observation 1: In general, the existing RFID standards use separate specifications to specify the requirements for protocol conformance and those for device performances. The conformance standard contain normative and often mandatory requirements that a device shall comply with; the performance standards describe the test methods to be used and performance metrics to be declared by device vendors.

Proposal 1: RAN4 to discuss and decide the minimum scope of necessary RF requirements for device 1.

Proposal 2: When specifying radiated performances, the orientation of the device antenna as well as the frequency variation of the antenna and the matching network should be accounted for, in addition to the performance of the RF IC chip.

Observation 2: Unlike conventional REFSENS, the device sensitivity level is more likely to be determined by the physical limits of the receiver circuitry, such as the threshold voltage of the 1-bit comparator.

Proposal 3: Conventional BLER vs SNR simulations are not needed for determining the device sensitivity.

Proposal 4: When measuring the radiated sensitivity performance, either TRS as in FR1 or EIS as in FR2 can be considered.

Proposal 5: Whether and how to define the minimum requirement for device 1 sensitivity, consider the following options:

- Alt-1: Do not define compulsory minimum sensitivity requirement. Device vendors follow the specified test method and declare the measured performance;

- Alt-2: Define one very relaxed requirement to accommodate diverse implementations tailored for different use cases;

- Alt-3: Define the sensitivity requirement as a range (similar to REFSENS for FR2 gNB);

- Alt-4: Define multiple levels of sensitivity requirements.

Proposal 6: RAN4 to discuss whether to define a minimum limit for the maximum input level or allow device vendor to declare the performance based on 3GPP test method.

Proposal 7: RAN4 to discuss whether to define a minimum requirement for the interference rejection capability or allow device vendor to declare the performance based on 3GPP test method.

Proposal 8: RAN4 to discuss the measurement configuration including incident CW power and orientation for measuring the backscatter power or backscatter loss.

Proposal 9: The modulation signal quality requirement can be considered as implicitly embedded in other RF or demod requirements. No explicit requirement for devices is needed.

Proposal 10: RAN4 to discuss the OBW requirement after defining the system channel bandwidth.

Proposal 11: Use NR FR1 SEM requirement as the starting point and further discuss whether to define TRP or EIRP requirement as well as potential simplifications.

Proposal 12: Use NR FR1 general spurious emission requirement as the starting point and further discuss the OOB boundary.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG RAN#106

- R4-2420470 WF on RF requirements for A-IoT, Moderator (Huawei), RAN4#113

- GS1 EPC Radio-Frequency Identity Generation-2 UHF RFID Standard: Specification for RFID Air Interface Protocol for Communications at 860 MHz – 930 MHz, Version 3.0

- ETSI EN 302 208, Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W; Harmonised Standard for access to radio spectrum

- GS1 Tagged-Item Performance Protocol (TIPP) Tagged-Item Grading: Testing Methodology Guideline

- R4-2413030 On the RF requirements for Ambient IoT devices, Huawei, HiSilicon, RAN4#112