# 3GPP TSG-RAN WG4 Meeting #114		            R4-2502236

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	4.2.1

Source: 	Huawei, HiSilicon

Title:	On the requirements of modified MPR behavior

Document for:	Approval

# 1	Introduction

The issue around the modifiedMPR-Behavior IE was raised in the last meeting [1][2]. In this paper, we present our analysis and views on the matter.

# 2	Discussion

In the Foreword of every 3GPP technical specification, the meanings of modal verbs are defined, e.g. in [3]:

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

Observation 1: In 3GPP technical specifications, “shall” indicates a mandatory requirement, and “may” indicates permission to do something.

In TS 38.101-1 [1], the modified MPR behaviour is defined in the Annex L, and mixed usages of “shall” and “may” are observed. An excerpt of the table from TS 38.101-1 v18.7.0 is shown below.

Table L.1-1: Definitions of the bits in the field modifiedMPR-Behavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n30 | 0 (leftmost bit) | Requirements for network signalling value NS_21 as defined in Clause 6.5.2.3.y of 38.101-1 v17.6.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v17.6.0. | This bit shall be set to 1 by a UE supporting the Rel-17 version of the specification. If the bit is not set, then requirements for NS_21 as defined in Clause 6.5.2.3.3 of 38.101-1 v16.11.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v16.11.0 apply. |
| … |  |  |  |
| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
|  | 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n71 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |
| … |  |  |  |
| n96 | 0 (leftmost bit) | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit may be set to 1 by a Rel-16 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 |
|  | 1 | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit may be set to 1 by a Rel-16 and Rel-17 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 |

Observation 2: Both “shall” and “may” are used in the definitions of the modified MPR behaviour, leading to confusion.

Use band n41 as an example. If a UE supports DC_(n)41AA, it is mandatory to (“shall”) set bit 0 to 1 according to the Notes column. Meanwhile, the UE may set bit 2 to 1 if it can support the R16 MPR and A-MPR (v16.4.0). In this case, both bit 0 and 2 are set to 1, how can the NW/TE tell which version of MPR/A-MPR that the UE complies with?

Table 1: Illustration of the contradiction in the definitions of the modifiedMPR-Behavior for band n41

| Bit number | Condition | Action | Consequence (i.e. supported functionality) |
| --- | --- | --- | --- |
| 0 (leftmost bit) | a UE supporting DC_(n)41AA UE EN-DC | This bit shall be set to 1 | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 |
| 1 | a UE supporting DC_41A_n41A EN-DC | This bit shall be set to 1 | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 |
| 2 | a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC | This bit may be set to 1 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. |

Observation 3: Contradiction exists in the definition of modifiedMPR-Behavior for band n41 due to the misuse of “shall”.

When examining band n71, bit 0 shall be set to 1 if a UE supports DC_(n)71AA. However, the applicable MPR points to Rel-15 (v15.5.0). In fact, the MPR requirements for contiguous intraband EN-DC have been revised a number of times since v15.5.0. A Rel-16 or later UE should comply with the MPR requirements in the corresponding release instead of v15.5.0.

| n71 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |
| --- | --- | --- | --- |

Observation 4: The definition of modifiedMPR-Behavior for band n71 is obsolete, and should be removed from the spec from Rel-16 and onwards.

Band n30 also has some problems. The Notes say: “This bit shall be set to 1 by a UE supporting the Rel-17 version of the specification.”. A UE supporting the Rel-17 version of the specification has to support all the mandatory features defined in Rel-17. Hence, it cannot be a Rel-16 or earlier UE.

Applying the notes would force a Rel-17 UE to comply with the new A-MPR requirements for NS_21 updated in v17.6.0. But this is unnecessary, since a Rel-17 UE should meet all the requirements defined in the Rel-17 specs at least in accordance with the subversion numbers when the specs were frozen.

| n30 | 0 (leftmost bit) | Requirements for network signalling value NS_21 as defined in Clause 6.5.2.3.y of 38.101-1 v17.6.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v17.6.0. | This bit shall be set to 1 by a UE supporting the Rel-17 version of the specification. If the bit is not set, then requirements for NS_21 as defined in Clause 6.5.2.3.3 of 38.101-1 v16.11.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v16.11.0 apply. |
| --- | --- | --- | --- |

For a Rel-18 UE, it shall meet the requirements defined in the Rel-18 specs, hence it need not set bit 0 to 1. However, the Notes say: “If the bit is not set, then requirements for NS_21 as defined in Clause 6.5.2.3.3 of 38.101-1 v16.11.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v16.11.0 apply.”. This cannot be true for a Rel-18 and later UE. Such wording is not forward-compatible.

Observation 5: The definition of modifiedMPR-Behavior for band n30 is improper, and should be removed from the spec from Rel-17 and onwards.

On the other hand, the definition of modfiedMPR-Behavior for band n96 is a good example. It shows that the proper usage of the UE capability parameter is to enable a UE of a given release to indicate the support of the requirements defined in a later release (i.e., early implementation). And the use of “may” in the Notes does not cause any confusion.

Observation 6: The definition of modfiedMPR-Behavior for band n96 is a good example to show how to enable a UE of a given release to indicate the support of the MPR/A-MPR requirements defined in a specific later release.

| n96 | 0 (leftmost bit) | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit may be set to 1 by a Rel-16 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 |
| --- | --- | --- | --- |
|  | 1 | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit may be set to 1 by a Rel-16 and Rel-17 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 |

In summary, the main use case for the modifiedMPR-Behavior is when an MPR/A-MPR requirement is changed in an open release (Rel-N). An early release (Rel-M) UE may optionally support the new requirement and indicate it by the modifiedMPR-Behavior parameter. It’s unnecessary for a Rel-N UE to indicate this parameter, since the new MPR/A-MPR requirement is the only one left in the Rel-N spec.

Furthermore, the MPR/A-MPR requirement can be changed multiple times and/or in different releases. Hence, it’s difficult to be forward-compatible with future releases if the signalling of a given bit is made mandatory (i.e. avoid using “shall”).

Observation 7: In general, “may” should be used to define the requirement of the modifiedMPR-behavior, considering forward-compatibility with future releases. The usage of “shall” can only be considered for exceptional cases.

Based on the above observations, some maintenance for the modifiedMPR-Behavior table is needed. Hence, we propose:

Proposal 1: RAN4 to discuss and decide how to align the requirements defined in the modifiedMPR-Behavior table of TS 38.101-1 for different NR bands and across different releases.

When the requirements for NS_05 and NS_05U are updated [1], new modifiedMPR-Behavior for band n1 can be introduced to the Rel-19 TS as follows:

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 and A-MPR as defined in clause 6.2.3.4 of 38.101-1 Rel-19 | This bit may be set to 1 by a Rel-18 or earlier release UE. |

UEs indicating an earlier release in the accessStratumRelease parameter can support the new A-MPR requirements via early implementation. The same changes can be added to the frozen releases of TS 38.101-1.

Proposal 2: Add the definition of modifiedMPR-Behavior for band n1 in Rel-15 to [Rel-19] specs to allow UEs of earlier releases to indicate the support of new requirements for NS_05 and NS_05U updated in Rel-19.

# 3	Conclusion

Based on our analysis on the current modifiedMPR-Behavior table in TS 38.101-1 [3], we have shared the following observations and proposals:

Observation 1: In 3GPP technical specifications, “shall” indicates a mandatory requirement, and “may” indicates permission to do something.

Observation 2: Both “shall” and “may” are used in the definitions of the modified MPR behaviour, leading to confusion.

Observation 3: Contradiction exists in the definition of modifiedMPR-Behavior for band n41 due to the misuse of “shall”.

Observation 4: The definition of modifiedMPR-Behavior for band n71 is obsolete, and should be removed from the spec from Rel-16 and onwards.

Observation 5: The definition of modifiedMPR-Behavior for band n30 is improper, and should be removed from the spec from Rel-17 and onwards.

Observation 6: The definition of modfiedMPR-Behavior for band n96 is a good example to show how to enable a UE of a given release to indicate the support of the MPR/A-MPR requirements defined in a specific later release.

Observation 7: In general, “may” should be used to define the requirement of the modifiedMPR-behavior, considering forward-compatibility with future releases. The usage of “shall” can only be considered for exceptional cases.

Proposal 1: RAN4 to discuss and decide how to align the requirements defined in the modifiedMPR-Behavior table of TS 38.101-1 for different NR bands and across different releases.

Proposal 2: Add the definition of modifiedMPR-Behavior for band n1 in Rel-15 to [Rel-19] specs to allow UEs of earlier releases to indicate the support of new requirements for NS_05 and NS_05U updated in Rel-19.

Table 2: Changes for Rel-15 to Rel-18 specs

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 and A-MPR as defined in clause 6.2.3.4 of 38.101-1 Rel-19 | This bit may be set to 1 by a UE supporting this release of the specification. |

Table 3: Changes for Rel-19 spec

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 and A-MPR as defined in clause 6.2.3.4 of 38.101-1 Rel-19 | This bit may be set to 1 by a Rel-18 or earlier UE. |

# References

- R4-2418123 Topic summary for [113][101] Upto_R17_UERF_maintenance, Moderator (OPPO), RAN4#113

- R4-2418299 LS on modifiedMPR-Behaviour, Samsung, RAN4#113

- 3GPP TS 38.101-1 v18.7.0