3GPP TSG-RAN WG4 Meeting #114	R4-2502241

Athens, Greece, February 17 – 21, 2025

Source:	Nokia

Title:	General aspects for XR enhancements

Agenda item:	7.28.1

Document for:	Discussion

Introduction

In this paper we present Nokia’s views on general aspects for XR enhancements [1] with regard to deployment scenarios for XR enhancements and CA scheduling restrictions treated in RAN4 #113 [2]. As part of this discussion, we provide corresponding observations and proposals.

Deployment scenarios for XR enhancements

RAN4 #113 achieved following agreements and way forward related to deployment scenarios for XR with measurement skipping [3].

Topic #1: Scenarios for measurement skipping ## Issue 1-1: Deployment scenarios < Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID <Way forward>: - Further discussion needed for - FR2-2 - NR-DC

FR 2-2

RAN4 #112bis agreed to consider measurement skipping for XR for FR1 and FR2-1. As already discussed in [4], for FR2-2, the beam sweeping scaling factor is extended to 12 instead of 8, and there is an additional scaling factor of KFR=2 and KFR=3 for 480 kHz and 960 kHz, respectively. As a result, the measurement delay in FR2-2 is 1.5x, 3x and 4.5x times larger than the delay for FR2-1 for 120 kHz, 480 kHz and 960 kHz SCS respectively. Additionally, FR2-2 has shorter slot durations and larger bandwidth, and as a result, the measurement skipping feature is not as needed as in other scenarios, since there are more transmission opportunities within the packet delay budget of XR services than longer slot durations. Considering the long measurement delays for FR2-2 we believe it is best to avoid further extensions that would be resulting from measurement skipping.

- FR2-2 measurement delay is 1.5x, 3x and 4.5x longer than FR2-1 measurements for 120 kHz, 480 kHz and 960 kHz SCS.

Furthermore, FR2-2 is dedicated to NR-U deployments and NR-U cannot provide a guaranteed QOS which is indispensable for conversational services such as XR.

- No guaranteed QoS for conversational services can be provided in FR2-2 using unlicensed bands.

Thus, XR deployments in FR2-2 using measurement skipping should not be considered in the NR_XR_ph3 work item.

- RAN4 to not apply measurement skipping for FR2-2 measurements.

NR-DC

RAN4 #112bis agreed to consider SA and CA deployment scenarios for XR enhancements in RRC_CONNECTED state. The support of dual connectivity (EN-DC, NE-DC) deployment scenarios was ruled out at RAN4 #113. What remains to be discussed in RAN4 is whether to consider RRM impacts due to measurement skipping for NR-DC. This is discussed below.

When UE is in NR-DC and RRC_CONNECTED state, the network can provide a separate measurement configuration for both PCell and PsCell. Thus, the UE may receive two independent measurement configurations from the network:

- a measurement configuration, associated with MCG, being included in the RRCReconfiguration message received via SRB1; and

- another measurement configuration, associated with SCG, being included in the RRCReconfiguration message received via SRB3, or, alternatively, included within a RRCReconfiguration message embedded in a RRCReconfiguration message received via SRB1.

Hence two independent measurement gap patterns can be assumed for PCell and PSCell. The measurement configuration also includes the measurement objects. Independent of the NR-DC configuration, the number of measurement objects is roughly doubled compared to the NR SA case.

Following NR-DC configurations are distinguished:

NR-DC Config 1: MCG and SCG in different FRs with per-FR gaps

In this configuration, PSCell and SCells associated with SCG are measured in different FR and hence the measurement can be independent of the XR traffic assuming it is served in the PCell in the other FR, and per-FR gaps are used. The DCI carrying the MG skipping command in the PCell thus is only applicable to SCells and PCell associated with MCG.

For NR-DC Config 1, there is no RRM impact due to measurement skipping in the PCell.

NR-DC Config 2: MCG and SCG in different FRs with per-UE gaps

In this configuration, PSCell and SCells associated with SCG are measured in different FR and per-UE gaps are used. In this case, MG skipping applies for MCG and SCG as for NR-DC Config 3. Hence the measurement in SCG cannot be independent of the XR traffic assuming it is served in the PCell in the other FR. However, the DCI carrying the MG skipping command in the PCell is not known in the PSCell, similar as for NR-DC Config 3 below.

For NR-DC Config 2, there is a significant RRM impact due to measurement skipping in the PCell.

NR-DC Config 3: MCG and SCG in the same FR

In this configuration, PSCell and SCells associated with SCG are measured in the same FR, either with per-UE or per-FR gap.  Hence the measurement of cells associated with SCG will be impacted in the same way as for cells in MCG. The DCI carrying the MG skipping command in the PCell however will not be known in the PSCell, and hence the secondary node will assume no MG skipping, whilst the UE has to perform MG skipping. This scenario will create a significant RRM impact, further it has not been studied by RAN1 and RAN2. Hence it is proposed not to be considered in RAN4 for MG skipping.

For NR-DC Config 3, there is a significant RRM impact due to measurement skipping in the PCell.

From the above, it is concluded there is no additional RRM impact from NR-DC compared to NR SA due to measurement skipping in favour of XR traffic for NR-DC Config 1 with per-FR gaps. Thereagainst, NR-DC Config 2 with NR-DC in different FRs using per-UE gap and NR-DC Config 3 with NR-DC in the same FR shall not be supported for XR enhancements.

Furthermore, given the Rel-19 timeline and the small size of 0.5 TU for the XR enhancements work in RAN4, RAN4 in general should minimize the RRM impact from XR enhancements to NR-DC.

RAN4 to consider for XR enhancements only NR-DC scenario with MCG and SCG in different FRs using per-FR gaps.

CA scheduling restrictions

In RAN1#118bis, RAN1 made following agreements with regard to support of carrier aggregation configuration when XR is operated:

Agreement At least for self-scheduling case, for explicit indication by DCI to skip a particular gap/restriction, one bit indication is included as part of DCI formats 0_1/1_1 and 0_2/1_2. - FFS: Cross carrier scheduling. - FFS: DCI formats 0_3/1_3. Explicit indication can be configured for each DCI format individually by higher layer. Agreement In case of cross carrier scheduling, for explicit indication by DCI to skip a particular gap/restriction, one bit indication included as a part of DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell.

And in RAN4 #113 the following issue was left for discussion regarding CA:

## Issue 6-2: Skipping indication on frequency domain <Way forward>: - Proposals - Proposal 1: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. - Proposal 2: - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs. - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated. - Proposal 3: It needs to be clarified that when per-FR gaps are configured, the indication of the particular gap to be skipped by DCI or MAC-CE/RRC is for which gap pattern(s).

Effectively, the afore RAN1 decision means that the skipping indication carried by the DCI applies to gap/restriction on the scheduled cell i.e. where PDSCH/PUSCH is scheduled, so that UE can conduct reception/transmission on the cell, as illustrated in Figure 1 when carrier aggregation (CA) with two cells has been configured with Component Carriers (CC) A and B.

(a)

(b)

Figure 1. Illustration of DCI based skipping indication behaviour for (a) self-scheduling and (b) cross-carrier scheduling.

Following this RAN1 agreement, RAN4 should investigate scheduling restrictions due to both self-scheduling and cross-carrier DCI scheduling.

RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

In context of TS 38.133, the measurement gap can be configured to be one of the three types; per-UE, per-FR1 or per-FR2 (based on UE capability to support). This determines whether the gap applies to cells for a given frequency range (e.g. per-FR1) or to all cells on all frequencies (per-UE). That is, in which cells UE is not required to conduct reception/transmission during the gap based on the measurement gap configuration. From UE RF perspective this can be considered to indicate the dependency/independence of the reception/transmission operation on different carriers/CCs. That is, whether UE receiver shares some components or operations between the different carriers/CCs, e.g. frequency synthesis etc.

Correspondingly for scheduling restrictions within SMTC, there needs to be also consideration by RAN4 for the applicability over different cells based on the frequency domain relation. In short, the scheduling restrictions allowed for the UE apply on all serving cells within same band during the symbols that fully or partially overlap with the restricted symbols. The assumption here is that UE baseband processing, e.g. FFT etc., can be shared over the carriers in the same band so that scheduling restrictions shall be allowed on all ‘intra-band’ cells, which are belonging to the applicable range of carriers.

In RAN1#118bis the implications in context of skipping were discussed, aiming to clarify expected UE behavior when UE is configured with CA and a particular gap/restriction is skipped. As noted above, this could be interpreted that gap/restriction on all cells that fall within the applicable range of carriers, cannot be used by the UE to do measurements (due to assumed sharing of RX components/processes). Effectively, with this assumption, impact on UE measurement requirements should be considered for all cells in the applicable range of carriers, e.g. intra-band cells for scheduling restrictions within SMTC.

- If it is assumed that for cells within the applicable range of carriers, UE can share (parts of) receiver over different cells, measurement requirement impact due to skipping of gap/restriction should be accounted for in all those cells.

Furthermore, if afore observation is valid, it should be clarified whether UE is able to conduct reception/transmission in all cells within the applicable range of carriers or only in the scheduled cell to which the indication is given. As noted above, the range of carriers/cells where the gap/restrictions are applied/allowed relates fundamentally to the UE receiver design. For example, UE can be configured with CA covering two cells on two component carriers, CC A and CC B, as illustrated in Figure 2, together with measurement gap configuration so that both cells/carriers fall within the range of applicable carriers where gap/restriction is applied/allowed.

Figure 2. Illustration of DCI skipping indication with multiple cells within applied range of gap/restrictions.

In this scenario, if skip is indicated on a serving cell in CC A, it will result that UE cannot do measurements on CC B, due to dependency in the receiver operation on these carriers/cells. Thus, it should be discussed and clarified whether UE can conduct reception/transmission also on all the carriers/cells that fall within the applicable range of carriers during the skipped gap/restriction.

- While measurements are skipped on CC A, UE cannot perform concurrent gap-assisted measurements on CC B due to dependency on common receiver components for a range of carriers.

RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

Conclusion

In this paper we have presented Nokia’s views with regard to open issues on deployment scenarios and CA scheduling restrictions for XR enhancements.

As part of this discussion, the following observations and proposals were made:

Deployments scenarios for XR enhancements

- FR2-2 measurement delay is 1.5x, 3x and 4.5x longer than FR2-1 measurements for 120 kHz, 480 kHz and 960 kHz SCS.

- No guaranteed QoS for conversational services can be provided in FR2-2 using unlicensed bands.

For NR-DC Config 1, there is no RRM impact due to measurement skipping in the PCell.

For NR-DC Config 2, there is a significant RRM impact due to measurement skipping in the PCell

For NR-DC Config 3, there is a significant RRM impact due to measurement skipping in the PCell.

- RAN4 to not apply measurement skipping for FR2-2 measurements.

RAN4 to consider for XR enhancements only NR-DC scenario with MCG and SCG in different FRs using per-FR gaps.

CA scheduling restrictions

RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

- References

- RP-243318, Revised WID: XR (eXtended Reality) for NR Phase 3, RAN #106, Madrid, Spain, December 9-12, 2024

- R4-2418287, Topic summary for [113][229] NR_XR_Ph3, Nokia, RAN4 #113, Orlando, US, Nov 18th – 22nd, 2024

- R4-2420103, WF on RRM requirements for NR_XR_Ph3, Nokia, RAN4 #113, Orlando, USA, November 18 – 22, 2024

- R4-2419327, Discussion on XR ph3 RRM requirements, Nokia, RAN4 #113, Orlando, US, Nov 18th – 22nd, 2024