3GPP TSG-RAN4 Meeting #114	R4-2502266

# Athens, GR, 17th –21th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | 0111 | rev |  | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Perf)Correction CR on SAN performance requirements in TS 38.108 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Perf |  |  |  |  |  | Date: |  |  | 2025-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The test parameters terminology of SAN PUCCH formats and multi-slot PUCCH are not aligned for First PRB after frequency hopping - The test parameter of SAN PRACH for channel model is not correct - Number of demodulation branches for SAN PRACH requirement should be updated as Number of Rx antennas considering the conducted test |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the test parameters terminology for SAN PUCCH formats and multi-slot PUCCH in section 8.3 - Update the channel model for SAN PRACH requirement - Update the number of demodulation branches for SAN PRACH requirement as number of Rx antennas |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SAN performance requirement can not be verified well |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3, 8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.181 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

Table 8.3.2.1-1: Test Parameters

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | the largest PRB index – (Number of PRBs– 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 for 2 symbols |

<End of Change 1>

<Start of Change 2>

Table 8.3.3.1.1-1: Test Parameters

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

<End of Change 2>

<Start of Change 3>

Table 8.3.7.2.1.1-1: Test Parameters for multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

<End of Change 3>

<Start of Change 4>

8.4.2	PRACH detection requirements

8.4.2.1	General

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For AWGN, NTN-TDLA100-200, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than the time error tolerance given in Table 8.4.2.1-1.

Table 8.4.2.1-1: Time error tolerance for AWGN, NTN-TDLA100-200

| PRACH | PRACH SCS | Time error tolerance |  |
| --- | --- | --- | --- |
| preamble | (kHz) | AWGN | NTN-TDLA100 |
| 0 | 1.25 | 1.04 us | 1.324 us |
| 2 | 1.25 | 1.04 us | 1.324 us |
| B4, C2 | 15 | 0.52 us | 0.804 us |
|  | 30 | 0.26 us | 0.544 us |

The test preambles are listed in table A.4 and the test parameter msg1-FrequencyStart is set to 0.

8.4.2.2	Minimum requirements

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in Tables 8.4.2.2-1 to 8.4.2.2-3.

Table 8.4.2.2-1: PRACH missed detection test requirements, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (annex D) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 2 |
| 1 | 1 | AWGN | 0 | -12.0 | -17.4 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | 0.7 | -9.7 |
|  | 2 | AWGN | 0 | -14.5 | -19.8 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | -6.8 | -14.9 |

Table 8.4.2.2-2: PRACH missed detection test requirements, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (annex D) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format B4 | Burst format C2 |
| 1 | 1 | AWGN | 0 | -14.6 | -9.2 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | -2.7 | 1.9 |
|  | 2 | AWGN | 0 | -16.8 | -12.5 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | -4.8 | -4.8 |

Table 8.4.2.2-3: PRACH missed detection test requirements, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (annex D) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format B4 | Burst format C2 |
| 1 | 1 | AWGN | 0 | -14.4 | -9.2 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | -4.3 | 0.1 |
|  | 2 | AWGN | 0 | -16.5 | -11.9 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | -10.0 | -5.8 |

<End of Change 4>