3GPP TSG-RAN WG4 Meeting #114                                                               R4-2502310

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.23.4

Source:	Samsung

Title:	Way Forward for [114][306] NR_duplex_evo_General

Document for:	Approval

# Introduction

This document is provided for capturing agreements and way forward on the thread [306] NR_duplex_evo_general based on the discussion in RAN4#114.

# Topic #1: General aspects (including RAN4 aspects for SBFD system parameters)

### Sub-topic 1-1: SBFD applicability for NR TDD band and CBW

##### Issue 1-1-1: FR2-1 CBW and SCS support for SBFD

Agreement:

- In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

  - RAN4 to further study on 100MHz for DUD case to identify if any issue.

##### Issue 1-1-2: Specification implementation of operating bands support for SBFD

Agreement:

- Text proposal below is endorsed:

=================== Start of Text Proposal ===================

5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation.

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31,  n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1.

Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Notes |
| --- | --- | --- | --- | --- |
| n1 | 1920 – 1980 | 2110 – 2170 | FDD |  |
| n2 | 1850 – 1910 | 1930 – 1990 | FDD |  |
| n3 | 1710 – 1785 | 1805 – 1880 | FDD |  |
| n5 | 824 – 849 | 869 – 894 | FDD |  |
| n7 | 2500 – 2570 | 2620 – 2690 | FDD |  |
| n8 | 880 – 915 | 925 – 960 | FDD |  |
| n12 | 699 – 716 | 729 – 746 | FDD |  |
| n13 | 777 – 787 | 746 – 756 | FDD |  |
| n14 | 788 – 798 | 758 – 768 | FDD |  |
| n18 | 815 – 830 | 860 – 875 | FDD |  |
| n20 | 832 – 862 | 791 – 821 | FDD |  |
| n24 | 1626.5 – 1660.5 | 1525 – 1559 | FDD | Note 7 |
| n25 | 1850 – 1915 | 1930 – 1995 | FDD |  |
| n26 | 814 – 849 | 859 – 894 | FDD |  |
| n28 | 703 – 748 | 758 – 803 | FDD |  |
| n29 | N/A | 717 – 728 | SDL |  |
| n30 | 2305 – 2315 | 2350 – 2360 | FDD |  |
| n31 | 452.5 – 457.5 | 462.5 – 467.5 | FDD |  |
| n34 | 2010 – 2025 | 2010 – 2025 | TDD | Note 12 |
| n38 | 2570 – 2620 | 2570 – 2620 | TDD |  |
| n39 | 1880 – 1920 | 1880 – 1920 | TDD |  |
| n40 | 2300 – 2400 | 2300 – 2400 | TDD |  |
| n41 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n46 | 5150 – 5925 | 5150 – 5925 | TDD | Note 3, Note 12 |
| n48 | 3550 – 3700 | 3550 – 3700 | TDD |  |
| n50 | 1432 – 1517 | 1432 – 1517 | TDD |  |
| n51 | 1427 – 1432 | 1427 – 1432 | TDD | Note 12 |
| n53 | 2483.5 – 2495 | 2483.5 – 2495 | TDD | Note 12 |
| n54 | 1670 – 1675 | 1670 – 1675 | TDD | Note 12 |
| n65 | 1920 – 2010 | 2110 – 2200 | FDD |  |
| n66 | 1710 – 1780 | 2110 – 2200 | FDD |  |
| n67 | N/A | 738 – 758 | SDL |  |
| n70 | 1695 – 1710 | 1995 – 2020 | FDD |  |
| n71 | 663 – 698 | 617 – 652 | FDD |  |
| n72 | 451 – 456 | 461 – 466 | FDD |  |
| n74 | 1427 – 1470 | 1475 – 1518 | FDD |  |
| n75 | N/A | 1432 – 1517 | SDL |  |
| n76 | N/A | 1427 – 1432 | SDL |  |
| n77 | 3300 – 4200 | 3300 – 4200 | TDD | Note 11 |
| n78 | 3300 – 3800 | 3300 – 3800 | TDD |  |
| n79 | 4400 – 5000 | 4400 – 5000 | TDD |  |
| n80 | 1710 – 1785 | N/A | SUL |  |
| n81 | 880 – 915 | N/A | SUL |  |
| n82 | 832 – 862 | N/A | SUL |  |
| n83 | 703 – 748 | N/A | SUL |  |
| n84 | 1920 – 1980 | N/A | SUL |  |
| n85 | 698 – 716 | 728 – 746 | FDD |  |
| n86 | 1710 – 1780 | N/A | SUL |  |
| n89 | 824 – 849 | N/A | SUL |  |
| n90 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n91 | 832 – 862 | 1427 – 1432 | FDD | Note 2 |
| n92 | 832 – 862 | 1432 – 1517 | FDD | Note 2 |
| n93 | 880 – 915 | 1427 – 1432 | FDD | Note 2 |
| n94 | 880 – 915 | 1432 – 1517 | FDD | Note 2 |
| n95 | 2010 – 2025 | N/A | SUL | Note 1 |
| n96 | 5925 – 7125 | 5925 – 7125 | TDD | Note 3, Note 4, Note 12 |
| n97 | 2300 – 2400 | N/A | SUL | Note 5 |
| n98 | 1880 – 1920 | N/A | SUL | Note 5 |
| n99 | 1626.5 – 1660.5 | N/A | SUL | Note 6 |
| n100 | 874.4 – 880 | 919.4 – 925 | FDD | Note 10 |
| n101 | 1900 – 1910 | 1900 – 1910 | TDD | Note 10, Note 12 |
| n102 | 5925 – 6425 | 5925 – 6425 | TDD | Note 3, Note 4, Note 12 |
| n104 | 6425 – 7125 | 6425 – 7125 | TDD | Note 8 |
| n105 | 663 – 703 | 612 – 652 | FDD |  |
| n106 | 896 – 901 | 935 – 940 | FDD |  |
| n109 | 703 – 733 | 1432 – 1517 | FDD | Note 2 |
| NOTE 1: This band is applicable in China only. NOTE 2: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. NOTE 3: This band is restricted to operation with shared spectrum channel access as defined in TS 37.213 [20]. NOTE 4: This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. NOTE 5: The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 6: UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48 [25]. NOTE 7: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order 20-51 [24]. NOTE 8: This band is applicable only in countries/regions designating this band for IMT licensed operation subject to country-specific conditions. NOTE 9: Operating band n200 is a reserved value. NOTE 10: This band is applicable to CEPT countries subject to ECC Decision (20)02 [21], for the RMR application. NOTE 11: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. In Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. NOTE 12: This TDD band is not applicable to SBFD operation. |  |  |  |  |

Table 5.2-2: NR operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- |
| n257 | 26500 – 29500 | TDD |
| n258 | 24250 – 27500 | TDD |
| n259 | 39500 – 43500 | TDD |
| n260 | 37000 – 40000 | TDD |
| n261 | 27500 – 28350 | TDD |
| n262 | 47200 – 48200 | TDD |
| n263 | 57000 – 71000 | TDD |

=================== End of Text Proposal ===================

##### Issue 1-1-3: Specification implementation of BS CBW support for SBFD

Agreement:

- Text proposal to Table 5.3.2-1 and Table 5.3.2-2 is endorsed:

=================== Start of Text Proposal ===================

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| Note: In FR1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |
| Note: The BS channel bandwidth for SBFD operation shall be no less than 100MHz. |  |  |  |  |

=================== End of Text Proposal ===================

Agreement:

- RAN4 shall aim to adopt the same Notes for Table 5.3.5-1 and Table 5.3.5-2 respectively. Take the table for FR2-1 as example:

=================== Start of Text Proposal ===================

Table 5.3.5-2: BS channel bandwidths and SCS per operating band in FR2-1

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n257 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n258 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n259 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n260 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n261 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n262 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| Note 1: For this bandwidth, it is not applicable for SBFD operation. |  |  |  |  |  |

=================== End of Text Proposal ===================

##### Issue 1-1-4: Declaration and flexibility for SBFD operation

Agreement:

- The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested.

  - This restriction shall not be applied to FR1 LA BS

  - FFS FR1 MR BS

### Sub-topic 1-2: DL/UL subband configurations support for SBFD

##### Issue 1-2-1: Restriction on the number of UL subband sizes supported

- No new agreement achieved.

##### Issue 1-2-2: Restriction on the number of DL subband sizes supported

- No new agreement achieved.

##### Issue 1-2-3: Restriction on guardband size for SBFD

- No new agreement achieved.

##### Issue 1-2-4: Other restriction on DL subband size

- No new agreement achieved.

### Sub-topic 1-3: Requirement Applicability for SBFD

##### Issue 1-3-1: Requirement set applicability table for SBFD-capable BS

Way Forward:

- RAN4 further study the applicability of RAN4 requirement for SBFD capable BS, by taking the table provided in R4-2501967 as example to be further study on. NOTE: The table does not imply that the BS can be declared 1-H for non-SBFD and 1-O for SBFD, or vice versa.

### Sub-topic 1-4: Fair coexistence between SBFD and TDD networks

##### Issue 1-4-1: Limiting Maximum UE Transmit power in SBFD symbols/slots.

Way Forward:

- No further RAN4 discussion is expected until RAN1 confirm the necessity of introducing the mechanism of limiting Maximum UE Transmit power in SBFD symbols/slots

##### Issue 1-4-2: Using threshold(s) to limit the scheduling of cell-edge UEs in SBFD symbols/slots

Way Forward:

- No further RAN4 discussion is expected until RAN1 confirm the necessity of introducing the mechanism of using threshold(s) to limit the scheduling of cell-edge UEs in SBFD symbols/slots

### Sub-topic 1-5: Other general issues for SBFD requirements

##### Issue 1-5-1: BS RF Specification structure for SBFD requirements

Agreement:

- RAN4 shall capture RF requirements in SBFD-capable BS in TS 38.104. To use a separate chapter for requirements for SBFD-capable BS in TS 38.104 is the baseline, while details can be FFS by taking below structure as the example.

Foreword

1	Scope

2	References

3	Definitions, symbols and abbreviations

4	General

5	Operating bands and channel arrangement

6	Conducted transmitter characteristics

7	Conducted receiver characteristics

8	Conducted performance requirements

9	Radiated transmitter characteristics

10	Radiated receiver characteristics

11	Radiated performance requirements

12	Requirements for SBFD-capable BS

# Topic #2: PUSCH with resource muting for CLI handling

### Sub-topic 2-1: RF requirement impact for PUSCH with resource muting

##### Issue 2-1-1: RF requirement impact for PUSCH with resource muting

Way Forward:

- RAN4 continue to study RF requirement impact for PUSCH with resource muting.

# Topic #3: Potentially new BS RF requirements for SBFD

### Sub-topic 3-1: Requirements for Transient period between SBFD and non-SBFD

##### Issue 3-1-1: Location of transient period between SBFD and non-SBFD

Agreement:

- RAN4 reach the conclusion of the location of transient periods:

  - Case-A: in TA_offset in SBFD symbol(s);

  - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols

    - FFS guard symbol(s) in this case is SBFD symbol or not.

  - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission:

    - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design.

    - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall

      - either send LS to RAN1 to ask for clarification,

      - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact.

  - Case-D: in TA_offset in non-SBFD UL symbol(s).

  - Note: The illustrative figure for Case A, B, C and D is provided below:

##### Issue 3-1-3: Transient period requirement applicability on SBFD subbands

Way Forward:

- Text proposal below is taken as the baseline for further discussion, with the understanding that the content can be further updated based on the future agreement on the location of transient period between SBFD and non-SBFD:

=================== Start of Text Proposal ===================

6.4.2B	Transmitter transient period for SBFD

6.4.2.1B	General

Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS.

The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases:

Table 6.4.2.1B-1: Transmitter transient period for SBFD

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | BS transmitter OFF-to-ON |

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

=================== End of Text Proposal ===================

### Sub-topic 3-2: In-channel adjacent subband requirements

##### Issue 3-2-1: In-channel adjacent subband leakage ratio: General

Agreement:

- New requirement for In-channel adjacent subband leakage ratio:

  - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104)

    - FFS the value for absolute PSD limit

    - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit.

##### Issue 3-2-2: In-channel adjacent subband leakage ratio: Configuration

Way Forward:

- Further study the configuration of the in-channel adjacent subband leakage ratio requirement, by taking the following table as the baseline:

| BS DL subband bandwidth BWSBFD,DL (MHz) | BS UL subband bandwidth BWSBFD,UL (UL) | Filter on UL subband and corresponding filter bandwidth | In-Channel Adjacent Subband Leakage Ratio limit |
| --- | --- | --- | --- |
| BWSBFD,DL as configured by gNB | BWSBFD,UL as configured by gNB | Square (BWSBFD,UL) | TBD |

##### Issue 3-2-4: In-channel adjacent subband blocking requirement

Way Forward:

- RAN4 continue to study In-channel adjacent subband blocking requirement, by following the agreement achieved in RAN4#113.