3GPP TSG-RAN WG4 Meeting #114					          R4-2502316

Athens, Greece, Feb 17 – 21, 2025

Source:	Nokia

Title:	TP to TR 38.908 clause 6.2 The calculation for EEIRP mask

Agenda item:       7.13.2	[Expected EIRP mask for upper 6GHz]

Document for:	Approval

- Introduction

This it TP to TR 38.908 for clause 6.2 The calculation for EEIRP mask. TP includes some updates and corrections of typos, to align with draftCR to core specification TS 38.104 and additional definitions for formula for EEIRP calculation.

- TP to TR 38.908

<Start of TP>

## 6.2	The calculation for EEIRP mask

6.2.1 General

The Expected EIRP calculation is based on two averaging processes:

- Averaging over the test beam directions in clause.

- Averaging over horizontal and vertical angles within specified vertical angles bins

These two averaging processes are inter-related and should not be viewed as the order in which the calculation is done.

The required sampling resolution for the EIRP pattern measured per test beam is described in clause 6.4.

    - 6.2.2  Angle definition

The angles in the spherical coordinate system used in this section is as shown in figure 6.2.2-1, and the arrow indicates the direction of increasing angle from the base station perspective:

Figure 6.2.2-1: Angle definition for EEIRP

    - 6.2.3 Averaging over horizontal and vertical angles

One of the average calculations for the Expected EIRP is averaging the EIRP over horizontal and vertical angles. There are seven elevation angular ranges with different elevation bin sizes. Each bin is effectively a spherical strip (or spherical cap for the last range) bounded by θHLi and θHHi as shown in Figure 6.2.3-1.

Figure 6.2.3-1: Spherical strip (blue shade)

The average EIRP for each elevation bin is defined as:

EIRPθHLi , θHHi=12πsinθHHi-sinθHLiθHLiθHHi-ππEIRPθ,φ cosθ dφ dθ        ,                1

The EIRPθ,φ is  measured in discrete steps and equation (1) needs to be written in discrete form, and the next section describes how this is considered.

It should be noted that all the averaging is done on linear units.

    - 6.2.4 Approximating the double integral in the EIRP averaging over an elevation angle range

The EEIRP evaluated over a spherical strip involves taking EIRP measurements, EIRPθ,φ, around the ‘point source’ (i.e. the Base Station) bounded by the elevation range θHLi and θHHi. Figure 6.2.4-1 is an illustration of the measured EIRP points and the area of the spherical strip for one of the elevation bin. Note that the measured EIRP points are over two discrete elevation angles with equal number of points in azimuth.

Figure 6.2.4-1: Measured EIRP points and the spherical strip

The double integral in equation (1) can be approximated in discrete form, and Figure 6.2.4-2 illustrates some of the relevant discrete parameters used in the approximation. It is important to note that this approximation is based on equal grid method.

Figure 6.2.4-2: Illustration of the parameters involved

For an equal angle grid method where the grid spacing is uniform in both the elevation θ (over the range of [θHLi to θHHi]) and azimuth φ (over the range of [-π to π]), with N and M points respectively, this average EIRP can be approximated to:

EIRPθHLi , θHHi=12π360°×sinθHHi-sinθHLiθLθH-π180°π180°EIRPθ,φ cosθ ∆φ ∆θ    ,                     (2)

By substituting the following:

dφ≈∆φ=2π360°M

dθ≈∆θ=(θHHi-θHLi)N

the overall equation (with input angles in degrees) can be written as:

EIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn    ,                       (3)

where

θHLi , θHHi	is the elevation angular range bounded by θHLi and θHHi in degrees, e.g. for bin 1, this will be 0⁰ and 5⁰ respectively.

θn	is the corresponding angle of the  nth midpoint between the elevation intervals, referenced from the horizon. Midpoint is the middle point between the intervals, e.g. for the interval 1°-2°, the midpoint is 1.5°.

φm	         is the corresponding angle of the mth midpoint between the azimuth intervals

nL	         is the lowest elevation index of sampling points within the θHLi , θHHi range

nH	         is the highest elevation index of sampling points within the θHLi , θHHi range

M	         is the number of azimuth points within the (-180°, 180°) range

N	         is the number of elevation points within the θHLi , θHHi range

Note: All input angles in the equation 3 are in degrees.

<Next section of TP>

## 6.4	Spatial sampling grid

For the maximum measurement/spatial sampling step size in the elevation angle, since the elevation angular range for some elevation bins bin is quite limited, especially for 1st, 2nd, 3rd and 4th elevation bin with only 5 degree angular range, in order to ensure the measurement accuracy of EEIRP measurement in the elevation angle to comply with WRC-23 resolution, RAN4 agree to use 1 degree as maximum measurement/spatial sampling step size in the elevation angle.

For the maximum measurement/spatial sampling step size in the azimuth angle, RAN4 agree to use the following approach similar as TRP measurement captured in Annex I in TS 38.141-2:

∆θ=1∆φ=180πλD

, where l is the wavelength and D is the physical length of the array antenna diagonal. The physical length of the array antenna diagonal can be determined by following expression:

D=w2+h2

, where w  is the array antenna width, and h is the array antenna length.

∆ϕref= BW

<End of TP>