3GPP TSG-RAN4 Meeting #114	R4-2502323

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0721 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_demod_enh3-Perf) Correct Measurement Channel for Requirements in Section 5.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-Perf |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | “Measurement channel” in Table 5.6.2.1.1.0-1 is incorrectly referencing TBS.1-1 and TBS.1-2 under Table A.4-2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct “Measurement channel” in Table 5.6.2.1.1.0-1 as per 38.101-4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement definition is not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.521-4 CR 0937 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 5.6.2	2RX requirements

#### 5.6.2.1	FDD

##### 5.6.2.1.1	Minimum requirements with Link Adaptation

The purpose of the requirements is to verify the PDSCH absolute physical layer throughput with link adaptation performance under 2 receive antenna conditions.

The performance requirements are specified in Table 5.6.2.1.1-2, with additional test parameters in Table 5.6.2.1.1-1 and the downlink physical channel setup according to Annex C.3.1.

Table 5.6.2.1.1-1: Test parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |
| Subcarrier spacing |  |  | kHz | 15 |
| Duplex Mode |  |  |  | FDD |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | 2×2 |
| Correlation configuration |  |  |  | ULA Low |
| Beamforming Model |  |  |  | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | (9) |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | (13) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 5 |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  |  | CodebookSubsetRestriction |  | 111111 |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 6 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Measurement channel |  |  |  | As specified in Table A.4-2 Rank 1: TBS.2-1 Rank 2: TBS.2-2 |
| Note1: For retransmission number 4 including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; follow the latest UE reported PMI whose rank is same as the initial transmission |  |  |  |  |

Table 5.6.2.1.1-2: Minimum performance requirement

| Test num. | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- |
| Test 1 | 35% | 19.3 |
| Note 1: The maximum throughout is defined as with TBS corresponding to CQI index 15 with rank 2. |  |  |

<< End of changes >>