3GPP TSG-RAN WG4 Meeting # 114	R4-2502371

Athens, GR, 17h – 21th Feb, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | Draft | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for TS 38.101-5, Introduction on RF requirements for NTN UE supporting less than 5 MHz channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce RF requirements for NTN UE supporting less than 5 MHz channel bandwidth |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce Reference sensitivity level for NTN UE supporting less than 5 MHz channel bandwidth To introduce Maximum input level for SAN supporting less than 5 MHz channel bandwidth |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RF requirements of NTN UE supporting less than 5 MHz channel bandwidth would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3, 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2501005 |  |  |  |  |  |  |  |  |

<Start of Change>

7.3	Reference sensitivity

7.3.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement:

-	In all bands, the UE shall be verified against those requirements by applying the REFSENS value in Table 7.3.2-1.

7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A3.2.2 of 3GPP TS 38.101-1 [5], with parameters specified in Table 7.3.2-1.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | [-101.2] | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | TBD | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | [-101.2] | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |  |

The reference receiver sensitivity (REFSENS) requirement specified in Table 7.3.2-1 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-2 and with default Tx-Rx carrier center frequency separation except for cases specified in Table 7.3.2-3.

Table 7.3.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  |  | 24 | 36 | 50 | FDD |
|  | 60 |  |  | 10 | 18 | 24 |  |
|  | 15 | 15 | 25 | 50 | 75 | 752 503 |  |
| n255 | 30 |  |  | 24 | 36 | 362 243 | FDD |
|  | 60 |  |  | 10 | 18 | 182 103 |  |
|  | 15 | 15 | 25 | 50 | 75 |  |  |
| n254 | 30 |  |  | 24 | 36 |  | FDD |
|  | 60 |  |  | 10 | 18 |  |  |
| NOTE: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. |  |  |  |  |  |  |  |

Table 7.3.2-3: TX – RX carrier centre frequency separation for REFSENS verification

| Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| n256 | 5 MHz | 165 MHz, 215 MHz |
|  | 20 MHz | 180 MHz, 200 MHz |
| n255 | 5 MHz | -72.5 MHz, -130.5 MHz |
|  | 20 MHz | -87.5 MHz, -115.5 MHz |

The minimum requirements specified in Table 7.3.2-1 shall be verified with the network signalling value NS_01 configured in Table 6.2.3.1-1.

< Next Change>

7.4	Maximum input level

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in 3GPP TS 38.101-1 [5] Annex A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1) with parameters specified in Table 7.4-1.

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 3, 5, 10, 15, 20 |
| Power in Transmission Bandwidth Configuration3 | dBm | -402 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-2 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in 3GPP TS 38.101-1 [5] Annex A.3.2.3 for 64 QAM. NOTE 3: Power in transmission bandwidth configuration value is rounded to the nearest 0.5dB value. |  |  |

<End of Change>