3GPP TSG-RAN WG4 Meeting #114	R4-2502574

Athens, Greece, 17th – 21st February 2025

Agenda item:			7.21.4

Source:	Nokia

Title:	Adhoc session minutes for FS_NR_AIML_Mob

Document for:	Approval

# Introduction

This document contains the ad-hoc meeting minutes for the discussion on AI/ML mobility SI. The ad-hoc meeting discussed some of the topics from the moderator’s summary [1].

Recommendations from the moderator for the topics to be discussed in the RRM session:

Topic #1: General Aspects

- Issue 1-1-1 Measurement event prediction use case

- Issue 1-1-2 Radio Link Failure (RLF) prediction use case

Topic #2: RAN4 Requirements

- Issue 2-1-2 Ground truth definition for RSRP accuracy

  - [For information] We have already reached an agreement in the Adhoc session to consider Option 1. Therefore, we just need to discuss Option 1-FR1 and Option 1-FR2.

- Issue 2-1-4 Impacts of measurement error on prediction accuracy

  - [For information] We discussed this issue during Adhoc session and reached a tentative agreement (in yellow). We need to discuss if a final agreement can be reached on this issue. If a final agreement is reached, we may need to discuss further the contents of LS (may be in another Adhoc session).

- Issue 2-1-3 Impacts on beam/cell level measurements

- Issue 2-1-9 Impact from observation and prediction windows

- Issue 2-1-6: LCM related requirements

Topic #3: Testability and Interoperability

- Issue 3-1-3 Considering multiple cells in the testing setup

- Issue 3-2-2 Principle of FR2 testing

- Issue 3-1-2: Prediction consistency in time domain

- Issue 3-1-1: Testing Setup in FR1

# Discussion.

## Topic#2: RAN4 Requirements

### Sub-topic 2-1: General and scope

#### Issue 2-1-1a: Relative RSRP Prediction Accuracy

- Proposals and Observations:

  - Proposal 1 (Oppo): Relative L3 Predicted RSRP Accuracy can be defined as the RSRP difference between any two reported predicted L3-RSRP results.

  - Proposal 2 (Ericsson):

    - RAN4 to consider both absolute and relative RSRP accuracy requirements for AI/ML based mobility.

    - Legacy definition for absolute RSRP accuracy and relative RSRP accuracy is adopted for AI/ML based mobility.

  - Proposal 3 (Huawei):  The absolute prediction accuracy for both intra-frequency and inter-frequency shall be prioritized, the relative perdition accuracy can be considered after more progress of measurement event prediction is achieved.

  - Proposal 4 (Qualcomm): RAN4 to consider a relative L3-RSRP as one of the performance metrics for AI/ML-based L3-RSRP prediction with the following definition:

    - Accuracy of predicted relative L3-RSRP = Reported predicted relative L3-RSRP - Ground truth of relative L3-RSRP, where the ground truth of relative L3-RSRP is defined as follows:

      - For intra-frequency L3-RSRP,

        - Difference between ‘the ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from one cell’ and the ‘ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from another cell on the same frequency’, or

        - Difference between the ground truths of two L3-RSRPs defined for the predicted absolute L3-RSRP accuracy from the same cell

      - For inter-frequency,

        - Difference between ‘the ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from one cell on a frequency’ and the ‘ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from another cell on a different frequency in the same FR

  - Proposal 5 (Xiaomi): RAN4 should clarify the definition of relative RSRP prediction accuracy before determining whether to include it as a requirement.

  - Proposal 6 (Samsung):

- For scenario 3, to accommodate the existing accuracy definition, it is plausible to define relative RSRP accuracy requirement besides the absolute RSRP accuracy requirement

- If to consider absolute and relative RSRP prediction accuracy for scenario 3, the accuracy definitions should be further discussed by taking at least the following into account:

    - Reference/baseline measurements related to 1st goal (measurement overhead reduction)

    - Difference of RSRP measured and RSRP predicted from different cells on different frequencies

    - Cell-specific and/or cluster prediction approaches

  - Proposal 7 (MediaTek):

    - Option1: relative RSRP accuracy is defined as legacy, i.e.:

      - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2)

      - extra margin should be added on top of the defined accuracy requirements in the test considering that transmitted power at TE may not be used as the ground truth directly.

    - Option 2: relative RSRP accuracy is defined based on the test setup, i.e.:

      - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (“ground truth” of RSRP of cell 1 – “ground truth” of RSRP of cell 2)

      - “ground truth” is the referenced RSRP to compare with in the test.

  - Proposal 8 (Nokia):

    - For at least temporal domain AI/ML mobility measurement prediction, RAN4 to define relative accuracy as “reported SS-RSRP level of one cell compared to another reported SS-RSRP level from another cell or the same cell, where the reported level is predicted or measured or a combination of both predicted and measured samples”.

    - RAN4 may study a different relative accuracy definition for frequency domain prediction aligning with definitions of RAN4 AI/ML for beam management.  One option is to define relative accuracy as ground truth SS-RSRP of (the best) predicted cell compared to the ground truth of the highest SS-RSRP of another cell or the same cell.

  - Proposal 9 (vivo): RAN4 to first consider absolute RSRP accuracy requirements. And for the relative RSRP accuracy, it can be further considered if the necessary scenarios are supported

- Candidate options:

  - Option 1: relative RSRP accuracy is defined as.:

    - relative RSRP = reported RSRP of cell 1 – reported RSRP of cell 2

  - Option 2: relative RSRP accuracy is defined as legacy, i.e.:

    - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2)

- Recommended WF

- Discuss above candidate Options.

- FFS: cell 1 and cell 2 are inter-frequency or intra-frequency.

- FFS: whether extra margin should be added on top of the defined accuracy requirements in the test

- FFS: whether ground truth can be reported or ideal (Follow issue 2.1.2)

- FFS: whether the reported RSRP can be predicted or measured or a combination of both predicted and measured samples.

Discussion:

QCM: For measured we already have legacy requirements.

Apple: There should be the measured part as well particularly in inter-frequency case. However this definition is ok as starting point.

OPPO: Based on current use cases, we don’t have

Agreement:

- Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported predicted RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2)

  - FFS: Beam 1 and Beam 2 can be from same cell or different cells.

  - FFS: whether ground truth can be reported or ideal

  - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

#### Issue 2-1-2: Ground Truth Definition for RSRP accuracy

- Proposals and Observations:

  - Proposal 1 (ZTE):

    - For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test.

    - For FR2, the definition of ground truth on R19 AI BM can be reused, that is, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP).

  - Proposal 2 (Huawei):

    - The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2.

    - The ground truth of L3 RPRP in FR1 is FFS, whether it is reported L3 RSRP measurement under sufficient high SNR or the transmitted RSRP directly from TE (conducted test).

  - Proposal 3 (Apple):

    - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy.

    - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test)

  - Proposal 4 (Qualcomm):

    - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

      - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE and the configured filter coefficient.

      - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

    - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition (UE-reported L3-RSRP with noise)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms.

        - [Testability: to avoid UE from cheating the test, TE can create another cell having the same radio condition to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction]

      - The definition of the ground truth does not differentiate FR1 and FR2.

    - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough.

        - [Testability: another round of tests would be needed for the L3-RSRP measurement report]

      - The definition of the ground truth does not differentiate FR1 and FR2.

  - Proposal 5 (Xiaomi):

    - If the TE is capable of calculating the ideal cell-level L3-RSRP, RAN4 should discuss the specific conditions and information required for the TE to accurately determine this ideal value.

    - If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports.

  - Proposal 6 (MediaTek):

    - For scenario 2 and 3, the “ground truth”, i.e., referenced RSRP to compare with in the test, can be derived based on the transmitted power at TE and the assumed L1 filtering.

  - Proposal 7 (Nokia):

    - RAN4 may consider UE reported L3 RSRP measurement as the approximate of ideal measurement of L3-RSRP and the ground truth for FR2 L3 prediction

    - RAN4 should consider that the test equipment in FR1 has the information of ground truth regarding L3-RSRP (transmit power).

  - Proposal 8 (vivo): For testing, the ground truth definition of L3-RSRP for FR1 and FR2 can be further studied based on the following candidate Options:

    - Option 1: To define the ground truth of L3 RSRP as the reported RSRP measurement under certain SNR

    - Option 2: To define measured RSRP from t1 to t2 as the ground truth under the condition that the channel from t0 to t1(prediction window) is the same as the channel from t1 to t2

  - Proposal 9 (CATT):

    - The definition of ground truth in FR1 can be the ideal L3 filtered cell-level measurement results for both temporal and frequency domain predictions

    - RAN4 to postpone the discussion on the definition of ground truth in FR2.

  - Proposal 10 (Tejas Network Limited)

    - For FR1 conducted tests, the ground truth should be derived from the test equipment.

  - Proposal 11 (OPPO)

    - ground truth is defined as the actual L3-RSRP measurement results based on legacy measurement procedure for both FR1 and FR2.

  - Proposal 12 （CMCC）

    - For FR2, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP)

    - For FR1, ground truth of L3-RSRP is the transmitted power, which is obtained as the legacy way of conducted test

  - Proposal 13 (Samsung): Two directions can be considered to define ideal RSRP

    - 	Direction 1: Settled by TE. The ideal RSRP is of the transmit power

    - 	Direction 2: Settled by measurement report. The ideal RSRP can be derived based on the RSRP measurements reported by UE for the noise-free or very high SNR conditions

For the both directions above, emulation of noise-free/high SNR test conditions and assumption of single signal source are supported

- Candidate options:

  - Option 1: The definition of the ground truth may differentiate between FR1 and FR2.

  - Option 2: The definition of the ground truth does not differentiate between FR1 and FR2.

  - Option 1-FR1:

    - Option 1-1-FR1: For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test.

      - FFS: If certain high SNR is required and what may be the SNR level required?

      - FFS: The power level at which SS-RSRP measurement is conducted.

    - Option 1-2-FR1: The ground truth of L3 RSRP as the reported RSRP measurement under certain SNR

    - Option 1-3-FR1: To define measured RSRP from t1 to t2 as the ground truth under the condition that the channel from t0 to t1(prediction window) is the same as the channel from t1 to t2

    - Option 1-4-FR1: The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X

      - FFS: If certain high SNR is required and what may be the SNR level required?

      - FFS: Further study factors impacting Y, such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc

    - Option 1-5-FR1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

      - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE and the configured filter coefficient.

      - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

  - Option 1-FR2:

    - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2.

    - Option 1-2-FR2: The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X

      - FFS: If certain high SNR is required and what may be the SNR level required?

      - FFS: Further study factors impacting Y, such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc

    - Option 1-3-FR2: : Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

      - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE and the configured filter coefficient.

      - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

    - Option 1-4-FR2: RAN4 to postpone the discussion on the definition of ground truth in FR2.

  - Option 2-1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

    - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE and the configured filter coefficient.

    - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

  - Option 2-2: Ground truth based on measurement report from the UE under the SAME radio condition (UE-reported L3-RSRP with noise)

    - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

      - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

      - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms.

      - [Testability: to avoid UE from cheating the test, TE can create another cell having the same radio condition to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction]

    - The definition of the ground truth does not differentiate FR1 and FR2.

  - Option 2-3: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free)

    - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

      - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

      - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough.

      - [Testability: another round of tests would be needed for the L3-RSRP measurement report]

    - The definition of the ground truth does not differentiate FR1 and FR2.

- Recommended WF

  - First Discuss Option 1 and Option 2.

  - If Option 1

    - Discuss Option 1-FR1 and Option 1-FR2

  - If Option 2

    - Discuss Option 2-1, Option 2-2 and Option 2-3.

  - If no agreement on Option 1 and Option 2, the moderator suggests to start the discussion with Option 1-FR1 and Option 1-FR2.

Discussion:

Qualcomm: In the study phase, we are not sure if we are ready to downselect any of these Option 1 and Option 2. We prefer Option 2 depending on the feasibility.

ZTE: We prefer Option 1.

CMCC: We prefer Option 1. Since there is RX beamforming gain issue in FR1.

Apple: is it for requirement or testing?

Qualcomm: Requirements also need some reference to the ground truth.

Huawei: For FR2, it may be reported while in FR1, it may be the transmit power from TE. We prefer Option 1 for testing.

Oppo: For testing, we share the same view as Apple and Huawei. From requirements perspective, there is no difference.

Agreement:

- The way the ground truth is extracted in the testing may differentiate between FR1 and FR2.

#### Issue 2-1-4: Impacts of measurement error on prediction accuracy

- Background

Following candidate options were identified in RAN4#113 meeting’s moderator’s summary.

- Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase - Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4) - Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event.

- Proposals and Observations:

  - Proposal 1 (ZTE):

    - The methodology of adding measurement error to AI mob should be similar to AI BM. To avoid redundant discussions, the simulation results of AI BM in RAN4 should be utilized as much as possible.

    - Measurement errors will affect the accuracy of predictions, and the larger the BB error, the greater the impact on prediction performance.

  - Proposal 2 (Huawei):

    - RAN4 to discuss the measurement impairment considered for verifying the prediction accuracy enabled by AI.

  - Proposal 3 (Apple):

    - RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. Additionally, RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility.

  - Proposal 4 (Qualcomm):

      - RAN4 should not start simulation-based performance evaluation on measurement prediction due to measurement inaccuracy in the observation window during the study item phase. Instead, the group can further discuss additional aspects that might need to be taken into account in the WI phase, if approved, in addition to the outcome of the evaluations on AI/ML-based BM, e.g.

        - Whether and how to accommodate the impact of measurement inaccuracies for a cluster-based approach.

        - Whether and how to model the impact of imperfect RF calibration for inter-frequency RRM prediction.

        - Whether different aspects need to be considered for Case A and Case B in intra-cell/intra-frequency RRM prediction.

        - Note that the effective differences between Case A and Case B, from a UE measurement and prediction perspective, might be limited to the measurement rate within the observation window and the prediction range into the future. Case A and Case B may have different impacts on RRM requirements, particularly about measurement/scheduling restrictions.

  - Proposal 5 (Nokia):

    - RAN4 should send an LS to RAN2, suggesting that RAN2 considers measurement errors in its evaluation

    - RAN4 should agree that both RF and baseband errors follow a Gaussian distribution

  - Proposal 6 (vivo):

    - RAN4 to evaluate the prediction accuracy performance with consideration of the impact of measurement error in WI .

  - Proposal 7 (Oppo):

    - Whether and how to evaluate the impacts of measurement error on performance requirement should be considered within RAN4 scope in WI stage.

  - Proposal 8 (CMCC):

    - if it is agreed that RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event, at least how to model measurement error (e.g. distribution of baseband/RF error) need to be provided to RAN2

  - Proposal 9 (Samsung):

    - RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error.

      - If additional simulations are needed, suggest to do the required simulation in WI phase

    - Suggest to wait for more progress/evaluation outcomes in AI/ML BM (especially for FR2 temporal domain prediction) to study the impact of measurement error on the prediction accuracy requirements for AI mobility

  - Proposal 10 (Ericsson):

    - RAN4 to send an LS to RAN2 to at least consider impact of error on measurement (for example due to interference from another cell, inaccuracy of the measurement etc.) used for prediction on the accuracy of the predicted measurement/event.

- Candidate options:

  - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase

  - Option 2: RAN4 should study the impact of measurement error on the prediction accuracy during WI phase

  - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4)

  - Option 4:  RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event.

    - RAN4 to discuss and identify specific simulation parameters (such as interfering cells, side conditions, RF margin etc.) that are not considered by RAN2 which may have an impact on the performance of AI/ML based mobility.

    - Option 4-a: Both RF and baseband errors follow a Gaussian distribution.

    - Option 4-b: Baseband and RF errors follow different distributions.

      - Baseband error is modeled as:

        - Option A: Gaussian distribution

        - Option B: link-level simulation results

      - RF error is modeled as:

        - Option A: uniform distribution

        - Option B: Gaussian distribution

        - Option C: truncated Gaussian distribution

        - Option D: fixed value

      - How to apply RF error:

        - Option A: Independent for each TX beam

        - Option B: Same for each Tx beam

        - Option C: Independent for each module/panel

        - Option D: Same for each module/panel

    - Option 4-c: Others

- Recommended WF

  - First Discuss Option 1 to Option 4.

  - If Option 4 is agreed

    - Discuss Option 4-a, Option 4-b and Option 4-c.

Discussion:

Samsung: Option 2 should be OK.

Apple: in some of the cases, to decide on ideal measurements may not be the best way. Option 4.

Qualcomm: Ideally that should be considered. We don’t oppose sending an LS.

Oppo: Evaluation in RAN2 is from another angle, which is different from RAN2. Measurement error is RAN4 business.

MediaTek: We agree with Oppo. We can continue how to model measurement error in RAN4 during SI phase, however that will only be considered in WI phase.

Vivo: We share the same view as MediaTek and Oppo.

Nokia: In our view, we should select the simplest option to consider measurement errors during SI phase.

Apple: If we are not confident on the result of the study, how we can be ready to define requirements during WI phase.

Qualcomm: We may share our assessment with RAN2 but we don’t have strong views on if RAN2 should consider it in their evaluations.

Oppo: We should let the evaluation of the feasibility study to RAN2. RAN4 should focus on the definition of requirements.

E//: This is unfortunate to not ask RAN2. We should send an LS to RAN2 and then let them decide.

Apple: The study should evaluate the feasibility considering measurement errors.

Samsung: We support Option 4

Agreement:

RAN4 to suggest to RAN2 to consider measurement errors in their evaluation. RAN2 to decide if to take them into account.

FFS: provide input for measurement error models.

- If RAN4 doesn’t agree on error model in RAN4#114, RAN4 will not send an LS.

# References

[1] R4-2500533, “Topic summary for [114][219] FS_NR_AIML_Mob”, Moderator(Nokia), RAN4#114