# 3GPP TSG-RAN WG4 Meeting #114	R4-2502588

# Athens, GR, 17th – 21st February, 2025

Agenda item:			7.8.4

Source:	Moderator (Apple)

Title:	WF on RRM requirements for NR_LBCA_Sw

Document for:	Approval

# Topic #1: RRM requirement for R19 LB CA via switching (7.8.3)

Issue 1-1-1: RRM work scope

  - Interruption requirement

    - Alt 1: Needs interruption requirement for carrier switching between FDD CC and SDL CC (Apple, Xiaomi, CATT, HW, Samsung)

      - Capabilities for UEs with and without interruption (Samsung)

    - Alt2:	RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional interruption or scheduling restriction in RRM

    - Session Chair (RRM session): check the progress in RF session, and discuss if any aspects are not covered by the RF discussion.

  - Switch delay requirements

Agreement:

    - The switching delay between operating the FDD and SDL carrier is to be discussed in the RF session. RAN4 RRM not to define additional switch delay in RRM.

  - Synchronization requirements for FDD and synchronization accuracy at BS antenna connectors or radiated interface boundaries

Agreement:

    - No need to define new cell phase synchronization requirement in RRM spec for R19 LB CA via switching.

  - SDL SCC related RRM requirement

- activated SDL SCell measurement

Agreement:

For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used.

For R19 LB CA via switching, FFS:

- Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used.

- de-activated SDL SCell measurement

Agreement:

For R19 LB CA via switching, RAN4 to not define SDL deactivated SCell L3 measurement requirement if SSB-less based SCell operation is used.

For R19 LB CA via switching, FFS:

      - Whether and how RAN4 to define SDL deactivated SCell L3 measurement requirement if SSB based SCell operation is used.

- other SDL SCell requirements

Agreement:

- FFS: Whether and how

  - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA

  - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA

- FFS: Whether and how

  - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA

  - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA

Agreement:

- Not to change definition of intra-frequency/inter-frequency measurement with/without gap measurement

- FFS on the use case for MG application.

  - Potential impacts to the PCell related requirement

    - FFS

Issue 1-1-2: Assumptions for R19 LB CA via switching

- (1) FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both?

Agreement:

- For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting.

- (2) FFS: MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session?

Agreement:

As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session.

- Option 1: 4.67us

- Option 2: 3 us