3GPP TSG-RAN WG4 Meeting # 114											R4-2502603

Athens, Greece, 17/02/2025 to 21/02/2025

Title: 	WF on RRM requirements for XR_Ph3

Agenda Item:	7.28.3

Source: 	Nokia

Document for:	Approval

# Topic #1: Scenarios for measurement skipping

## Issue 1-1: Deployment scenarios

< Agreement>:

- Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

<Way forward>:

- Further discussion needed for

  - NR-DC

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions

## Issue 1-4: L1 measurements with scheduling restrictions and procedures

## Issue 1-5: L1/L2 triggered mobility measurements and procedures

<Agreement>

- During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions.

- Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported.

- Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

# Topic #2: Measurements with gaps

## Issue 2-1: Types of measurement gaps

<Way forward>:

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Companies may bring proposals regarding the priorities of each gap type for RAN4 to work based on the following table

| Gap types | First priority | Second priority | Don’t include |
| --- | --- | --- | --- |
| Type 1 |  |  |  |
| Type 2 |  |  |  |
| NCSG |  |  |  |
| MUSIM |  |  |  |
| Positioning |  |  |  |
| Rel 18 concurrent gap |  |  |  |
| PreConfigured |  |  |  |

# Topic #3: XR skipping requirements impact

## Issue 3-1: Principles for measurement delay calculation

<Agreement>:

- Follow NR-U-like approach for designing the formulas for measurement delay extension.

## Issue 3-2: Formulas for measurement delay extension

<Way foward>: The purpose of the options provided below is for guidance and not to preclude any other options.

- Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms × CSSFinter, Ceil( 8  * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

- Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

## Issue 3-3: Conditions for measurement skipping

<Way forward>:

- Option 1: conditions are needed

  - Option 1a: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified.

  - Option 1b: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133

  - Option 1c: Maximum acceptable time separation between two measurement occasions available for RRM measurements

- Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

# Topic #4: Time offset for measurement skipping processing

## Issue 4-1: Time offset requirement for measurement gap skipping

<Way forward>:

- Option 1: 3 ms optional capability only apply when UE is only configured with NR measurement.

- Option 2: No need to define additional condition for the optional capability of 3 ms

## Issue 4-2: Time offset requirement for cancelling skip indication

<Way forward>:

- Option 1: The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI at least X ms before the MG occasion, where

  - X=5ms as baseline and X=3ms as optional UE capability.

  - [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

# Topic #5: Need/feasibility of UE assistance information

## Issue 5-1: General on UAI

<Agreement>:

- Agree to define UAI based on the following assumptions, and further discuss the content of UAI

- RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective)

- Not define UE behaviour and the corresponding requirements based on UAI

- Supporting of UAI is optional for UE with optional UE capability.

<Way forward>:

  - Proposals related to measurement occasions

    - Option 1: Do not define UAI related to measurement occasions.

    - Option 2: The UE indicates a pattern of unused gaps

    - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension

    - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation.

    - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability.

  - Proposals related to channel conditions

    - Option 1: Provide information on channel conditions

    - Option 2: do not provide information on channel conditions

# Topic #6: Additional signalling aspects

## Issue 6-1: Semi-static schemes

<RAN1 agreement for information>

Agreement Confirm the Working assumption in RAN1#118 for supporting Alt. 1-1: Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.. Conclusion It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction. - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

<Way forward>:

- Further discussion based on recommended options

  - Option 1: Do not consider semi-static solutions

  - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution.

    - Option 2a: RRC pattern-based

    - Option 2b: MAC CE bitmap for skipping indication

    - Option 2c: RRC configuration patterns with MAC CE activation

  - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

## Issue 6-2: Skipping indication on frequency domain

<Way forward>:

- Proposals

  - Proposal 1: For the gap/restriction skipping impact in frequency domain,

    - For the gap skipping, depend on whether the UE capable of per-FR gap

      - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

      - Otherwise, no need to distinguish different FR.

    - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

  - Proposal 2:

    - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

    - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

  - Proposal 3 (Huawei): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

  - Proposal 4 (vivo): RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

# Topic 8 List of impacted clauses and worksplit

| Clause | Topic | Volunteer |
| --- | --- | --- |
| 8.xr (new) Or 9.1 | Time offset for DCI based measurement skipping | Nokia |
| 9.2.6 | Intra-freq measurements with gaps | ZTE |
| 9.3.4 | Inter-freq measurements with gaps | Ericsson |
| 9.4.2 | Inter-RAT measurements E-UTRAN FDD measurements | Huawei |
| 9.4.3 | Inter-RAT measurements E-UTRAN TDD measurements | Apple |
| NOTE: The full list of clauses with potential impact due to XR skipping is found in the Ad-hoc minutes R4-2502580 |  |  |