3GPP TSG-RAN WG4 Meeting # 114	             R4-2502604

# Athens , GR, 17th – 21st Feb, 2025

Agenda Item:	7.29.5

Source: 	Moderator (CATT)

Title: 	WF on RRM requirements for NR_NTN_Ph3_Part1

Document for:	Approval

0 Notes:

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other options can be proposed.

- < Tentative Agreement > will be changed to < Agreement > if no comments are received.

1 Topic #1: General consideration on RRM requirements

    - Sub-topic 1-1 What RRM requirements are defined for (e)RedCap UE with FR1-NTN

Issue 1-1-1: Whether to define measurement requirement for inter-RAT (NR NTN to E-UTRAN TN) in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN?

Online agreement:

Agreement:

- RAN4 to define measurement requirements in cell re-selection for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN.

Issue 1-1-2: SDT requirements

Online agreement:

Agreement:

- For SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and RAN4 can refer to the requirements on TA validation for the PUR requirements for IoT-NTN, that is,

      - Define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN.

- Further discuss whether to send RAN2 LS

Issue 1-1-3: MDT requirements

Online agreement:

Agreement:

- For MDT requirements in RRC_IDLE and RRC_INACTIVE state,

      - Deprioritize to define Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN.

    - Sub-topic 1-2 Features to be considered for (e)RedCap UE with FR1-NTN

Issue 1-2-1: Relaxed measurement

Online agreement

Agreement:

- For the relaxed criteria,

      - Only ‘low mobility’ and ‘not at cell edge’ relaxation criteria defined for R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) will be supported from RAN4 requirement perspective.

- For the thresholds of relaxation criterion,

      - Existing thresholds defined for relaxed measurement in R17 NTN are re-used, and threshold value is up to network configuration.

- For the relaxation factor K,

      - Reuse the parameters used in R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation), which is 3 for UE only fulfill one of the relaxation criterion, and no requirement when both relaxation criterion are fulfilled for up to 1 hour.

Issue 1-2-2: eDRX configuration

For information:

The agreement is based on the assumption of the serving time of satellite is about several minutes(5min~10min) for quasi-earth fixed cell LEO deployment. The approximate serving time also can be calculated according to the minimum elevation angle received by UE (25°，[TR 38.863]), the altitude of LEO(600km) and the speed of the satellite(7.56km/s, [TR 38.821]). The following calculation is just for reference: Tsering≈ (2*600）/ tan 25°/7.56≈340s

Online agreement:

Agreement:

For quasi-earth fixed LEO scenario:

- From RAN4 requirement perspective, the requirements are applicable for up to 10.24s eDRX cycle, larger than 10.24s eDRX cycle cannot be applied for (e)RedCap UE with FR1-NTN.

Issue 1-2-3: SMTCs and 'in parallel' related

- Background

R17 NTN UE capabilities related to 'in parallel' are copied as follows:

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_NTN_solutions | 25-1 | Parallel measurements on multiple SMTC-s for a single frequency carrier | Support of measurements on target cells belonging to 4 SMTC-s on a single frequency carrier |  | parallelSMTC-r17 | MeasAndMobParametersCommon | FDD only | FR1 only | UE is mandatory to support 2 and can optionally support 4 if the feature is supported | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-2 | Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell | Support of measurements on cells belonging to different satellite as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. The feature is applicable only when the serving satellite is NGSO. If the serving cell belongs to GSO satellite, the scheduling restriction is not applied on the premise that a mixed type of satellites on the same frequency layer is not supported in this release (Rel-17). |  | parallelMeasurementWithoutRestriction-r17 | BandNR | FDD only | FR1 only | For UEs not able to perform measurements in parallel with normal operation of serving cell scheduling restrictions shall apply. | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-3 | Parallel measurements with multiple measurement gaps | Support of 2 measurement gaps |  | parallelMeasurementGap-r17 | MeasAndMobParametersCommon | FDD only | FR1 only | UE is mandatory to support 1 measurement gaps | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-5 | Parallel measurements on multiple NGSO satellites within a SMTC | Support of simultaneously measurements on target cells belonging to different NGSO satellites within a SMTC |  | maxNumber-NGSO-SatellitesWithinOneSMTC-r17 | BandNR | FDD only | FR1 only | Candidate values for the number of NGSO satellites are 1,2,3, or 4 |  |
| 25. NR_NTN_solutions | 25-7 | The number of target LEO satellites the UE can monitor per carrier including serving satellite | On serving carrier, it indicates the number of target LEO satellites the UE can monitor per carrier including serving satellite On non-serving carrier, it indicates the number of target LEO satellites the UE can monitor per carrier. |  | maxNumber-LEO-SatellitesPerCarrier-r17 | BandNR | FDD only | FR1 only | Candidate values for the number of NGSO satellites are 2,3, or 4. The value shall be larger than or equal to the reported value on FG 25-5. | Optional with capability signaling |

The agreements in the last meeting:

RAN4#113: <Agreements>: - RAN4 to define RRM requirements for multiple SMTC-s for a single frequency carrier for (e)RedCap UEs. - For whether to support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs? - Option 1 (MTK, Xiaomi, Samsung, ZTE, Ericsson, LG, QC): Not support - Option 2 (HW, Nokia): FFS - Further discuss the supported number of SMTCs, the number of cells and SSB, the number of supported satellites, the proximity distance between two SMTCs. - Further discuss the specific measurement requirements, i.e, whether and how to extend measurement period. - Further discuss other R17 NTN UE capabilities (i.e, Feature 25-2/25-3) related to 'in parallel'.

Issue 1-2-3-1: Feature 25-1(parallelSMTC)

Online discussion

<Way Forward>:

Options for further discussion:

If not differentiate 1Rx and 2Rx RedCap UEs, the baseline for UE capability is:

- Option 1: [1 SMTC or] 2 SMTCs not in parallel

- Option 2: 2 parallel SMTCs

  - ‘Parallel’ means the SMTCs are TDM with proximity distance.

Issue 1-2-3-1a: The proximity distance between two SMTCs

Ad-hoc agreement

Agreement:

- RAN4 to define the proximity distance between two SMTCs for (e)RedCap UEs in NTN.

- The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

- For whether to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier,

  - Option 1: No. RAN4 to reuse the legacy collision condition.

  - Option 2: Yes. RAN4 to consider relaxation for proximity, i.e.,

- When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

- the two SMTC occasions are fully or partially overlapping in time domain, or

- the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x can be FFS. (1ms as starting point)

Issue 1-2-3-2: Feature 25-5(maxNumber-NGSO-SatellitesWithinOneSMTC)

Ad-hoc agreement

Agreement:

- For whether to define RRM requirements for parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5, maxNumber-NGSO-SatellitesWithinOneSMTC-r17) for (e)RedCap UEs,

  - The UE capability is applicable, and whether to support this optional feature is up to UE implementation. RAN4 to follow the same principle of the existing non-RedCap requirements.

Issue 1-2-3-3: The number of cells/SSBs/supported satellites

<Way Forward>:

- For the number of supported satellites,

  - Proposal 1: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for RedCap UE.

  - Proposal 2:

    - RedCap UEs shall support at least one NGSO satellite per SMTC configured in a frequency layer in addition to the serving satellite.

    - The total number of supported satellites for measurement in a frequency layer is equal to N+1 where N is the number of configured SMTCs, if the serving satellite is to be measured in this frequency layer and N if the serving satellite is not to be measured in this frequency layer.

- For the number of cells and SSB,

  - Proposal 1: RedCap NTN reuses the number of cells and SSB for NR NTN, i.e.,

    - Proposal 1a: For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

- 8 identified cells, and

- 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

- 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

- In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

Issue 1-2-3-4: Feature 25-2(parallelMeasurementWithoutRestriction)

Ad-hoc agreement

Agreement:

- For whether to define RRM requirements for Feature 25-2(parallelMeasurementWithoutRestriction-r17) for (e)RedCap UEs,

  - The UE capability is applicable, and whether to support this optional feature is up to UE implementation. RAN4 to follow the same principle of the existing non-RedCap requirements.

Issue 1-2-3-5: Feature 25-3(parallelMeasurementGap)

- Background

The agreements on concurrent gap related issue in the last meeting:

RAN4#113: < Agreement >: - Deprioritize parallel measurement gap for (e)RedCap UEs in NTN. - Come back to this issue at Apr 2025 meeting.

<Way Forward>:

- For whether to define RRM requirements for Feature 25-3(parallelMeasurementGap-r17) for (e)RedCap UEs,

  - Proposal 1: Support to define.

    - Proposal 1a: The UE capability is applicable, and the support of the optional features is left to UE implementation following the same principle of the existing non-RedCap requirements.

    - Proposal 1b: The capability can be optionally supported by (e)RedCap UEs with 2Rx, can not be supported by (e)RedCap UEs with 1Rx.

Issue 1-2-4: The number of searchers and CSSF

< Agreement >

- Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

Issue 1-2-5: FH in PRS measurement for NW verified location requirements

< Agreement >

- Not to consider FH in the PRS measurement requirements for NW verified location for (e)RedCap UE with FR1-NTN.

Issue 1-2-6: NCD-SSB

<Way Forward>:

- Proposal 1:

  - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

- Proposal 2:

  - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

Issue 1-2-7: Other features

<Way Forward>:

- For SMTC adjustment in idle/inactive,

  - Option 1: Keep mandatory for RedCap UE.

  - Option 2: Keep optional for RedCap UE.

- For Location-based measurement trigger,

  - Option 1: Keep optional for RedCap UE.

  - Option 2: RAN4 not to further discuss applicability of location-based measurement report trigger.

2 Topic #2: Reduction in the number of UE Rx branches

    - Sub-topic 2-1: RRC_IDLE/RRC_INACTIVE mode mobility

- Background

The agreement in the last meeting:

RAN4#113: < Agreement >: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - RAN4 to reuse the legacy mobility requirements in RRC_IDLE / RRC_INACTIVE state defined for normal UEs with FR1-NTN in high level. - Depend on the supported UE capabilities (e.g., multiple SMTC), the existing NR NTN requirements may need be revised. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - RAN4 to reuse the same offset of 1Rx RedCap UEs in TN compared to 2Rx (e)RedCap UEs. - For Qrxlevmin and Qqualmin: -1 dB offset can be reused. - For the measurements of intra- and inter-frequency NR cells, - +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline. - For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked. - In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria. - For 1 Rx RedCap by a margin of at least: - 6 dB in FR1 for reselections based on ranking or - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities. - RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring. - 6 NR inter-frequency carriers can be reused. - The parameters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)RedCap UE, and the requirements for both will be the same. - For SDT requirements, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx RedCap UE. - Define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands. - FFS: How to define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to further discuss how to define the cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx and 2 Rx (e)RedCap UE after issue1-1-1.

Issue 2-1-1: Enhanced cell reselection measurement requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

<Way Forward>:

  - Proposal 1: Enhanced RRM requirements for measurements in IDLE and INACTIVE modes are kept optional in Redcap over NTN.

    - Proposal 1a: Define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX_IDLE cycle, the requirement for normal UEs can be reused.

    - Proposal 1b: To define enhanced cell reselection requirements for RedCap UE,

      - Re-use the requirements for normal UE except that Tdetect is extended by [2] samples

      - Same delay requirements for 1Rx and 2Rx UEs.

    - Proposal 1c:

      - For 2 Rx (e)RedCap NTN UE, RAN4 to reuse the existing enhanced cell reselection measurement requirements

      - For 1 Rx (e)RedCap NTN UE, RAN4 to

- Option 1: Reuse the existing enhanced cell reselection measurement requirements

- Option 2: Consider increasing measurement samples based on the current cell reselection measurement enhanced requirements. The actual number of samples can be further determined.

    - Proposal 1d: For enhanced cell re-selection for (e)RedCap UE with FR1-NTN bands,

      - For 2Rx/1Rx, the Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh are reused the legacy FR1-NTN requirements. In addition, The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

Issue 2-1-2: Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN

<Way Forward>:

- Proposal 1: The existing requirements for measurements of inter-RAT E-UTRAN cells with TN carrier can be used as baseline.

- Proposal 2: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements, reuse the legacy requirements defined for normal UEs for (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle.

- Proposal 3: For cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for (e)RedCap UE with FR1-NTN bands,

  - For 2Rx UE, the existing cell reselection requirements in Rel-18 (e)NTN can be reused for (e)RedCap 2Rx UE with FR1-NTN bands.

  - For 1RX UE, reuse the same margin of RSRP/RSRQ as for 2Rx.

Issue 2-1-3: SDT requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

<Way Forward>:

- Proposal 1: For SDT requirements on Redcap NTN in RRC_INACTIVE state, RAN4 to revert the agreement reached in RAN4#113 and modify it as:

    - For SDT requirements on Redcap NTN, the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

    - Sub-topic 2-2: RRC_CONNECTED mode mobility

Issue 2-2-1: Satellite switching with re-sync for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Ad-hoc agreement

Agreement

- For hard and soft Satellite switching with re-sync requirements,

    - For 1Rx (e)RedCap UEs with FR1-NTN bands, the delays for Satellite switching with re-sync should be extended compared to those defined for 2Rx (e)RedCap UEs.

      - Define the relaxation on Tsearch by extending one more SSB periodicity on the top of Tfirst_SSB:

- Tsearch = Tfirst_SSB + TSSB ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the source satellite.

    - Sub-topic 2-3 Measurement Procedure for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Issue 2-3-1: L1/L3 Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Ad-hoc agreement

Agreement:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the legacy FR1-NTN measurement requirements defined for normal UEs in high level.

    - Depend on the supported UE capabilities (e.g., multiple SMTC), the existing NR NTN requirements may need be revised.

- For 1Rx (e)RedCap UEs with FR1-NTN bands,

    - The time period for PSS/SSS detection defined for intra-/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

    - The time period for SSB index identification (PBCH decoding) defined in intra-/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

- Extend the lower boundary from ‘120 ms’ to ‘160ms’.

    - The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Reuse the same extensions for 1Rx RedCap UE in TN network.

- Extend the lower boundary from ‘200 ms’ to ‘400ms’

    - The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands are the same. i.e. NOT differentiate 2Rx vs. 1Rx in terms of measurement period requirement

    - The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands are the same.

Issue 2-3-2: Network verified UE location for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Ad-hoc agreement

Agreement:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred, and the same relaxation (as the measurement accuracy requirements defined for 1Rx TN-RedCap ) for accuracy should be reused.

    - Sub-topic 2-4 L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Issue 2-4-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

  - For 2Rx (e)RedCap UEs with FR1-NTN bands,

    - Proposal 1: RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands,

    - Proposal 1: RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

      - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

- L1-RSRP: relax by 3dB.

3 Topic #3: The impact of HD-FDD

    - Sub-topic 3-1 General HD-FDD related impact

Issue 3-1-1: General HD-FDD related impact

<Way Forward>:

  - Proposal 1: The legacy requirements and applicable conditions can be as the baseline for defining requirements of RedCap UE in NTN scenario.

    - Proposal 1a: To following the TN, the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

      - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

      - Handover interruption time

      - Random access

      - SA: RRC Connection Release with Redirection

      - Minimum requirement for L1 indication for RLM and LR

      - MAC-CE/DCI/RRC based uplink spatial relation switch delay

      - Scheduling availability of UE performing intra/inter measurements

      - Scheduling availability of UE during L1-RSRP measurement

    - Proposal 1b: For scheduling availability, need further RAN1 progress for detailed requirements of HD-FDD UEs.

    - Proposal 1c:

      - Adopt TN requirements as baseline for clarifying the requirements listed in P1a for HD-FDD RedCap UE except for

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE during L1-RSRP measurement

      - Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period.

    - Proposal 1d:RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reach further conclusions.

  - Proposal 2: For NTN UEs with HD-FDD, the requirements are met provided that the SSB of at least one of the satellites to be measured in this SMTC period is available at the UE side.

  - Proposal 3: For HD RedCap and eRedCap UEs, RAN4 to adopt a high level statement that the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification.

  - Proposal 4: RAN4 should further discuss measurement behavior for collision between SMTC and UL when network indicates UL overriding DL in HD-FDD.

    - Sub-topic 3-2 The specific impact of HD-FDD on NR Handover

Issue 3-2-1: The impact on NR Handover due to HD-FDD

<Way Forward>:

- Proposal 1:

  - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

    - For RedCap NTN UE with HD-FDD, the requirements are met provided that

      - SSB is available at the UE once every SMTC period during Tsearch

      - One SSB is available during T∆

      - One SSB is available during TIU.

  - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

    - For RedCap NTN UE with HD-FDD, the requirements are met provided that

      - One SSB is available during T∆

      - One SSB is available during TIU

4 Topic #4: Others

    - Sub-topic 4-1 Specification structure

Issue 4-1-1: Specification structure

Agreement

- Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands.

- Utilize the reference method for the requirements that can be reused. FFS details.

    - Sub-topic 4-2 UE capabilities for (e)RedCap UE with FR1-NTN bands

Issue 4-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

<Way Forward>:

- Proposal 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Proposal 2: Introduce new capability to declare support over RedCap over NTN.

- Proposal 3: Both proposals, i.e., a new capability declaring (e) RedCap for NTN or combining (e)RedCap and NTN, are fine with us. It may be determined later.