3GPP TSG-RAN WG4 Meeting #114		  																											R4-2502858

Athens, Greece, 17– 21 February, 2025

Agenda item:			7.24.4

Source:	CMCC

Title:	WF on requirements for system parameters and A-IOT device

Document for:	Approval

Introduction

This way forward captures the agreements on A-IOT system parameters, device RF requirements and OTA test method. The summary document for RAN4#114 is R4-2500686.

System parameters

- Topic 2-1: System parameters

Issue 2-1-1: Operating band

Agreement:

- NR band n8 can be used as an example band, and other FDD bands below 1GHz are not precluded.

Issue 2-1-2: R2D transmission bandwidth definition

Agreement:

- Reuse the following definition of R2D transmission bandwidth in TR38.769

  - Transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting R2D.

- RAN4 will re-visit the two agreed bullets above, if RAN1 has the different conclusion.

Issue 2-1-3: R2D channel bandwidth definition

Agreement:

- Define R2D channel bandwidth as:

  - Channel bandwidth, BCH,R2D from a reader perspective: The frequency resources used for transmitting R2D, and potential guard band.

Issue 2-1-4: Values of R2D bandwidth

Agreement:

- Define R2D transmission bandwidth configuration:

  - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS

  - FFS on other RB numbers

- Further discuss the R2D channel bandwidth considering additional guard band.

- If RAN1 has the agreement different than above, RAN4 will revisit the agreements

Issue 2-1-5: R2D channel raster and channel spacing

Agreement:

- Do not define channel spacing for R2D transmission

- Define channel raster for R2D transmission.

  - Details need further discussion.

Issue 2-1-6: D2R bandwidth

Agreement:

- D2R channel bandwidth for A-IoT device

  - Define D2R channel bandwidth for A-IoT device considering the following aspects:

    - [1SB or] 2SB

    - D2R maximum data rate (depending on RAN1 conclusion)

    - SFO

    - FDMA, i.e., considering the different device 1 has different frequency shift

    - Reference Filter performance

    - Other aspects are not precluded

Issue 2-1-7: D2R channel raster and channel spacing

Agreement:

Do not define channel raster and channel spacing for D2R

- Topic 2-2: Others

Issue 2-2-1: CW spec

Agreement:

- Add CW transmission in the new specification title: “TS 38.xxx Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception”

Issue 2-2-2: Deployment scenario

Agreement:

- To specify RAN4 RF requirements based on the assumption that NR UE is only outdoor in Rel-19.

  - FFS on whether and how to capture the side condition of outdoor UE assumption in the specifications

Device RF requirements

- Topic 3-1: General

Issue 3-1-1: General

Agreement:

- Only define radiated RF requirements for device 1 in Rel-19 WI. The details will be further discussed.

- Topic 3-2: Tx

Issue 3-2-1: Transmit output power

Agreement:

- Measurement configuration including input CW power and potential other conditions need to be defined.

Issue 3-2-2: Modulation quality requirements

Agreement:

- Further discuss whether and how to define the modulation signal quality requirements

Issue 3-2-3: Occupied bandwidth

Agreement:

- Further discuss whether to define the occupied bandwidth for device 1.

Issue 3-2-4: Emission requirements

Agreement:

- Consider one composite emission requirements covering the emission frequency range of SEM and spurious emission as starting point.

- Further discuss whether to define TRP or EIRP requirement for unwanted emission.

Issue 3-2-5: Other requirements

Agreement:

- Do not define output power dynamics and intermodulation requirements for device 1.

Issue 3-2-6: Other requirements (frequency error)

Agreement:

- Do not define frequency error requirements.

- Small frequency shift error will be discussed in modulation quality requirements.

Issue 3-2-7: Other requirements (Transmit ON/OFF time mask)

Agreement:

- Do not define transmit ON/OFF time mask for device 1

- Topic 3-3: Rx

Issue 3-3-1: Reference sensitivity

Agreement:

- For R2D reference sensitivity, consider following options:

  - Option 1: define reference sensitivity using legacy approach, e.g. PREFSENSdBm=-174dBm+10×log10BW+NF+IM+SNR

  - Option 2: define reference sensitivity based on link budget and/or physical limits of the receiver circuitry.

  - Other options are not precluded.

Issue 3-3-2: Maximum input power

Agreement:

- Further discuss the maximum input power for A-IOT device for R2D signal.

Issue 3-3-3: Interference rejection

Agreement:

- Do not define interference rejection requirements.

OTA test method

- Topic 4-1: General

Issue 4-1-1: Scope of OTA test methodology

Agreement:

- For OTA test, the starting point of test methodology for Ambient IoT includes

  - Chamber setup

  - Position of Device

  - Test procedures

  - Minimum range/ QZ (Quiet Zone) size/QZ calibration

  - Preliminary MU assessment

- Note: The performance metrics to be discussed in RF (RRM/Demodulation if any) requirements. And the test methodology takes sensitivity and backscatter power as example for discussion to develop test methodology.

Issue 4-1-2: Measurement campaign

Agreement:

- Measurement campaign is not possible in this work item. Refinement in future work items based on measurement campaign is not precluded.

Issue 4-1-3: chamber

Agreement:

- Use anechoic chamber as baseline for device1 test in Rel-19

Issue 4-1-4: Energy source

Agreement：

- Further discuss the energy source issue for device 1.

- Topic 4-2: Test method and configurations

Issue 4-2-1: Test configuration

Agreement：

- Test configuration is for a single A-IOT device under free space condition

Issue 4-2-2: Device positioning guidance

Agreement：

Take following as a starting point and further discuss the details.

- The traditional test system of AC for handheld UE radiated testing can be considered as the starting point for A-IoT device testing.

- The following test setup parameters shall be defined for A-IoT device OTA testing:

  - DCW2D: distance between CW node and DUT

  - DMA2D: distance between measurement antenna and DUT

  - IsoCW2M: Isolation between CW node and measurement antenna