3GPP TSG-RAN WG4 Meeting #114	R4-2502863

Athens, Greece, 17-21 Feb, 2025

Title: 	           WF on TN HPUE

Agenda Item:	7.1.1.1

Source: 	Samsung, Skyworks, OPPO, LGE, Apple

Document for:	Approval

1. PC1.5 for intra-band contiguous and non-contiguous UL CA

1.1 MPR values for intra-band contiguous ULCA with contiguous RB allocation (with TxD)

Agreement:

- Define separate MPR/A-MPR requirements for handheld and FWA

- Companies are encouraged to bring A-MPR values in following meetings

- The MPR values of intra-band contiguous ULCA PC1.5 are supposed to be higher than single CC PC1.5, as well as intra-band contiguous ULCA PC2

- Having the following PC1.5(2Tx) MPR values for equal PSD as starting point to be further checked in upcoming meetings

  - Companies can further evaluate the unequal PSD (6dB) impacts in next meeting.

  - The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference

| Modulation |  | PC3 (1Tx) |  | PC2 (2Tx) |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | Handheld |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | QPSK | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | 16QAM | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | 64QAM | 5 | 7 | 6 | 8 | [7] | [9] | [6.5] | [8.5] |
|  | 256QAM | 7 | 7.5 | 8 | 8.5 | [9] | [9.5] | [8.5] | [9] |
| CP-OFDM | QPSK | 3.5 | 8 | 4 | 8.5 | [5-5.5] | [9.5] | [4.5-5] | [9] |
|  | 16QAM | 3.5 | 8 | 4 | 8.5 | [5-5.5] | [9.5] | [4.5-5] | [9] |
|  | 64QAM | 5 | 8 | 5.5 | 8.5 | [6-6.5] | [9.5] | [5.5-6] | [9] |
|  | 256QAM | 7 | 8 | 7.5 | 8.5 | [8.5] | [9.5] | [8.5] | [9] |
|  |  |  |  |  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

1.2 MPR values for intra-band contiguous ULCA with non-contiguous RB allocation (with TxD)

Agreement:

- Further discussion in upcoming meetings

1.3 MPR values for intra-band non-contiguous ULCA (with dualPA)

Agreement:

- Further discussion in upcoming meetings

1.4 MPR applicability for intra-band contiguous and non-contiguous ULCA

Agreement:

- The MPR requirements defined are carrier power and are PSD balance/imbalance agnostic

- The MPR values are derived with equal PSD condition with margin reserved to accommodate unequal PSD cases

- Send LS to RAN5 on PSD condition RAN4 assumed to derive MPR requirements, if needed

1.5 PCMAX,c and PCMAX for intra-band NCCA (with dual PA)

Agreement:

- PCMAX,C is upper bounded by 26dBm

  - Detailed spec change is discussed based on dCR provided in future meeting, CR leader is encouraged to take companies’ input into account

- For PCMAX, no change of current spec

2. 2-band Inter-band UL NR-CA/EN-DC with 2Tx and/or 3Tx

  - Configurations

Agreement:

- RAN4 to develop general requirements based on at least Table 1, Table 2, Table 3, and Table 4 in Rel-19

- Table 1, Table 2, Table 3, and Table 4 are for information purpose, and are not expected to be captured into RAN4 TS.

| 1st band | 2nd band | 2Tx or 3Tx |
| --- | --- | --- |
| PC3 TDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC3 TDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC3 FDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC3 FDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC5 TDD 1Tx | PC2 TDD 1Tx | 2Tx |
| PC5 TDD 1Tx | PC2 TDD 2Tx | 3Tx |
| PC5 TDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC5 TDD 1Tx | PC2 TDD 2Tx | 3Tx |

Table 1 Potential new UL configurations for PC2 inter-band UL CA with 2Tx or 3Tx

| E-UTRA band | NR band | 2Tx or 3Tx |
| --- | --- | --- |
| PC3 TDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC3 TDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC3 FDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC3 FDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC2 FDD 1Tx | PC3 TDD 1Tx | 2Tx |
| PC2 FDD 1Tx | PC3 FDD 1Tx | 2Tx |
| Note: Per the WID, only PC3 is considered for LTE FDD band in EN-DC configuration in this Rel-19 HPUE work under this WI |  |  |

Table 2 Potential new UL configurations for PC2 inter-band EN-DC with 2Tx or 3Tx

| 1st band | 2nd band | 2Tx or 3Tx |
| --- | --- | --- |
| PC2 TDD 1Tx | PC2 TDD 1Tx | 2Tx |
| PC2 TDD 1Tx | PC2 TDD 2Tx | 3Tx |
| PC3 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx |
| PC2 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx |
| PC2 TDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC2 TDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC2 FDD 1Tx | PC2 TDD 2Tx | 3Tx |
| PC2 FDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC2 FDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC2 FDD 1Tx | PC1.5 TDD 2Tx | 3Tx |
| PC5 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx |

Table 3 Potential new UL configurations for PC1.5 inter-band UL CA with 2Tx or 3Tx

| E-UTRA band | NR band | 2Tx or 3Tx |
| --- | --- | --- |
| PC3 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx |
| PC3 FDD 1Tx | PC1.5 TDD 2Tx | 3Tx |
| PC2 TDD 1Tx | PC2 TDD 1Tx | 2Tx |
| PC2 TDD 1Tx | PC2 TDD 2Tx | 3Tx |
| PC2 TDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC2 TDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC2 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx |
| PC2 FDD 1Tx | PC2 TDD 1Tx | 2Tx |
| PC2 FDD 1Tx | PC2 TDD 2Tx | 3Tx |
| PC2 FDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC2 FDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC2 FDD 1Tx | PC1.5 TDD 2Tx | 3Tx |
| Note: Per the WID, only PC3 is considered for LTE FDD band in EN-DC configuration in this Rel-19 HPUE work under this WI |  |  |

Table 4 Potential new UL configurations for PC1.5 EN-DC with 2Tx or 3Tx

  - Re-organize 2Tx/3Tx Notes in clause 5 and clause 6

Agreement:

- Companies are encouraged to continue the discussion on the necessity and possibility to re-organize the Notes pertaining to power class in future meetings

  - Release independence of 3Tx NRCA/EN-DC

Agreement:

- For FWA, Rel-17 (agreed in RAN4#113)

- For handheld UE, Rel-18

3. Increasing UE transmission power

Moderator Note: No open issue

4. General aspect

4.1 MSD rules

Moderator Note: It is covered by R4-2502864

4.2 Duty cycle solution for SAR compliance

Agreement:

- Detailed spec changes are checked based on corresponding dCRs

5. Annex: dCR/LS splitting information

| dCR | author | Note |
| --- | --- | --- |
| Intra-band contiguous ULCA w and w/o MIMO | Skyworks | Including all the necessary spec changes |
| Intra-band NC ULDA (w/o MIMO) | Xiaomi | Including all the necessary spec changes |
| Inter-band ULCA/EN-DC with 2Tx, for TS 38.101-1/3 (New MSD rules, and potential Notes reorganization are not covered in these dCRs) | Vivo | It is observed that only needed spec change is duty cycle SAR solution. Target to endorse -1 and -3 dCRs in Feb meeting |
| Inter-band ULCA/EN-DC with 3Tx, for TS 38.101-1/3 (New MSD rules, potential Notes reorganization change are not covered in these dCRs) | LGE | It is observed that only needed spec change is duty cycle SAR solution. Target to endorse -1 and -3 dCRs in April meeting |
| Increasing higher power limit feature for NRCA/EN-DC, for TS 38.101-1/3 | Samsung, Nokia, Qualcomm, Apple, Ericsson, vivo, ZTE, OPPO, Huawei, Hisilicon, LG Electronics, CHTTL, Skyworks, xiaomi | Endorsed already |
| New MSD rules+ potential Notes re-organization, for TS38.101-1 | Skyworks |  |
| New MSD rules+ potential Notes re-organization, for TS38.101-3 | Skyworks |  |
| Ts 38.307 | ZTE | Suggest to postpone ZTE’s dCR(R4-2500950), companies are encouraged to provide comments offline/online |
| LS to RAN5 on MPR applicability for intra-band ULCA (if needed) | OPPO |  |
| LS to RAN5 on new MSD rules | Nokia |  |
| Note 1: 1st dCR author is encouraged to cooperate with other companies Note 2: The above dCR splitting/LS splitting does not preclude other companies to provide inputs as valuable reference |  |  |