3GPP TSG-RAN WG4 Meeting # 114															R4-2503006

Athens, GR, Feb 17 – Feb 21, 2025

Agenda item:			7.10.5

Source:	vivo

Title:						WF on HPUE UE requirement for IoT-NTN

Document for:		Approval

- Tx Requirements

- Tx requirements

Issue 1-1-1: ACLR for NB-Iot based Iot-NTN for PC1/2;

Agreement:

- Re-use current PC3 ACLR (UTRA_ACLR = 37dBc) for PC1/2.

Issue 1-1-2: ACLR for eMTC based Iot-NTN for PC1/2;

Agreement:

o	Option 1: Re-use PC3 eMTC based Iot-NTN ACLR 30dBc.

o	Option 2: 31dBc ACLR

Issue 1-1-3: SEM for NB-Iot based NTN for PC1/2

Agreement:

- Reusing SEM requirements defined for PC3 for NB-Iot based Iot-NTN

Issue 1-1-4: SEM for eMTC based NTN for PC1/2

Agreement:

- Reusing SEM requirements defined for PC3 for eMTC based Iot-NTN

Issue 1-1-5: MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC1/2

Agreement:

- For MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC1/2

  - For NB-IoT PC2, for A-MPR evaluations assume 25 dBc IQ-image and LO-leakage and 60 dBc CIM3

  - For NB-IoT PC1, for MPR and A-MPR evaluations assume [28~34]dBc IQ-image and LO-leakage and 60 dBc CIM3.

  - For NB-IoT PC2, as a starting point consider re-use of PC3 MPR

    - Editor’s Note: In RAN4#113 there was also a related agreement: “As the starting point, consider the center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation.”

Issue 1-1-6: MPR assumptions for eMTC based Iot-NTN for PC1/2

Agreement:

- For MPR/A-MPR assumptions for eMTC based Iot-NTN for PC1/2

  - For Cat M1, in simulations calibrate PA to just meet ACLR with [30~31] dBc ACLR and 1 dB MPR with 100 RB DFT-s-OFDM waveform.

Issue 1-1-7: P-MPR definition for NB-Iot based Iot-NTN.

- Proposals

  - For NB-Iot based Iot-NTN, discuss the proposal, confirm it is applicable for all the power classes, and discuss whether it can be defined from Rel-18.

  - Discuss whether P-MPR should be applied to handheld UEs only and whether this needs to be specified or not.

- WF

  - Further discuss P-MPR definition

Issue 1-1-8: The Basic MOP tolerance (Including eMTC and NB-Iot based)

- Proposals

  - For PC2:

    - Option 1: +2 / -3dB

    - Option 2: +2 / -2dB

  - For PC1:

    - Option 1:  +2 / -3dB

- WF

FFS

Issue 1-1-9: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation.

- Proposals

- Option 1: Relaxation of TX MOP tolerance upper limitation for NTN HD-FDD

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26 | +2x, -2 | 23 | +2x, -2 |
| 255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 2: Additional NW signal to control relaxation of TX MOP upper limitation for NTN HD-FDD.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26x | +2, -2 | 23x | +2, -2 |
| 255 | 26x | +2, -2 | 23x | +2, -2 |
| NOTE X: The power limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 3: New sub power classes (e.g., PC2NTN-HD-FDD) to relax the TX output power upper limitation.

- Option 4: Other ways are not excluded

  - FD-FDD can also be considered.

- WF

  - FFS.

- Feasibility & Requirements Application

Issue 1-2-1: Whether PC1 requirements can be applied to handheld UE for NB-Iot based Iot-NTN.

- Proposals

  - Option 1: Yes

  - Option 2: No

- WF

  - FFS.

Issue 1-2-2: PC1 Nominal MOP for NB-Iot based Iot-NTN

- Proposals

  - Considering lower nominal output power for NB-IoT based Iot-NTN PC1.

    - Not necessarily aligned and further improve feasibility

- WF

  - FFS.

Issue 1-2-3: The reference components need to be considered.

Agreement:

- The reference components need to be considered.

  - For NB-Iot based Iot-NTN, both GSM PA and NR 1Tx PC2 PA are taken into consideration in developing unified for one power class;

  - For eMTC-based IoT-NTN, HPUE for PC2 and NR 1Tx PC2 PA

Issue 1-2-4: Whether PC1 requirements can be applied to handheld UE for eMTC based with half duplex.

- Proposals

  - Option 1: Yes

  - Option 2: No

- WF

  - FFS

Current Status: (For Information)

Table: Current status for requirements consideration and feasibility for Iot-NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | FFS (non-handheld already agreed) |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | FFS (non-handheld already agreed) |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

- SAR Related issues

- SAR Related issues

Issue 2-1-1: Whether and how to assume some scenarios like satellite orbits and traffic model etc. for the study of SAR issue.

- Proposals

  - Option 1: No need.

    - [E.g. The scheme has to be general enough to cover various scenarios;]

  - Option 2: Needed.

    - [E.g. If typical scenarios do not have an issue may be the discussion is not needed.]

  - Option 3: Others

WF (Ad-hoc agreement):

- Option 1

Issue 2-1-3: Whether and how to introduce duty cycle related method.

- Proposals (Not necessarily mutually exclusive):

  - Scheme 1: (vivo)

Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful.

  - Scheme 2: (oppo)

UE indicate whether the following Tx time duration should be counted into the UL duty cycle statistics in addition to current maxUplinkdutycycle capability. How the information is reported is up to RAN2.

  - Scheme 3: (Samsung)

To use beam hopping information as the UE uplink duty ratio in time domain if duty cycle-based SAR solution is to be specified.

And to discuss following impacts to existing duty cycle solution, considering our target is for UE to maintian its power class

Scheme 3-1: UE to fall back power classes based on information above ( legacy duty cycle solution, not quite aligned with purpose to transmit as much and as long as possible)

Scheme 3-2: UE to fall back several dB but not entire power class, based on information above ( technically UE can reach more power than Scheme 3-1)

Scheme 3-3: UE to not fall back any power, but to monitor SAR by itself. ( In this case, UE may fail SAR compliance.)

- Scheme 3-3a: UE to enter scheme like a ’DTx’, or Tx UL Gap for a certain duration.

- Scheme 3-3b: UE to give up a certain ratio of uplink opportunities for a certain duration to recover its margin for SAR compliance. (Like above scheme 1 from vivo)

- Scheme 3-3c: UE to fall back to lower power class for a certain duration to recover its margin for SAR compliance.

  - Scheme 4: (Ericsson, Sony)

Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value. (Ericsson, Sony)

If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details: (Ericsson, Sony)

    - If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

      - DPC = ‘00’: the criteria to report DPC is not met

      - DPC = ‘01’: ΔPPowerClass = 0 dB

      - DPC = ‘10’: ΔPPowerClass = 3 dB

      - DPC = ‘11’: ΔPPowerClass = 6 dB

    - If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

      - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

      - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

      - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

      - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

- WF

  - Further discuss the proposed schemes related to duty cycle related SAR scheme.

  - Other schemes or no duty cycle related scheme are not precluded.

- Rx Requirments

- RSD (for full duplex eMTC based)

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based)

- Proposals

  - 256:

    - 0 dB (vivo, oppo, Samsung, Ericsson)

    - 1.1 dB (Qualcomm)

  - 255:

    - 0 dB (oppo)

    - 1.2 dB (Qualcomm)

    - FFS (vivo)

  - 254:

    - 0 dB (Samsung, Ericsson)

- WF (Ad-hoc agreement)

  - 256: FFS

  - 255: FFS

  - 254: 0dB

Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based)

- Proposals

  - Option 1: 0dB for n254/n255/n256 (Ericsson,…)

  - Option 2: non-zero value (MediaTek)

  - Option 3: others

- WF (Ad-hoc agreement)

  - Keep earlier agreement [0] dB.

  - Further study needed