3GPP TSG-RAN WG4 Meeting # 114	R4-2501436

# Athens, Greece, 17th – 21st February, 2025

Source:	Huawei, HiSilicon, Xiaomi

Title:	TP for TR 37.719-21-11 on introduction of DC_8A-28A_n77A and DC_8A-28C_n77A

Agenda item:	6.2.3

Document for:	Approval

1 Background

This contribution provides text proposal on the NR band combination DC_8A-28A_n77A and DC_8A-28C_n77A.

2 Text Proposal

---Start of changes---

6.X	DC_8-28_n77

6.X.1	Configurations for DC

Table 6.X.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE X) |
| --- | --- |
| DC_8A-28A_n77A DC_8A-28C_n77A | DC_8A_n77A DC_28A_n77A |

6.X.2	Co-existence analysis for DC

Table 6.X.2-1: Band 8 and Band n77 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 | 3320 | 4180 | 5115 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 1470 | 5685 | 7520 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 | 6030 | 7480 | 9315 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1560 | 555 | 8985 | 11720 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6640 | 4770 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940 | 6945 | 10780 | 13515 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8360 | 10230 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15920 | 12285 | 360 | 680 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10840 | 8070 | 3855 | 5760 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 | 17715 | 6820 | 7860 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 | 14430 | 9240 | 11145 |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.x.2-1, IMD4 may fall into Rx frequency range of band 28 when both band 8 and n77 transmit signals.

Table 6.X.2-2: Band 28 and Band n77 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3497 | 4003 | 4948 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2794 | 1804 | 5852 | 7697 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5696 | 7303 | 9148 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2091 | 1056 | 9152 | 11897 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6994 | 5104 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6444 | 10603 | 13348 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8006 | 9896 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16097 | 12452 | 308 | 1388 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11194 | 8404 | 4356 | 6291 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 17548 | 6112 | 7192 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 14096 | 8709 | 10644 |

Based on Table 6.x.2-2, IMD5 may fall into Rx frequency range of band 8 when both band 28 and n77 transmit signals.

6.X.3	∆TIB and ∆RIB values

Referring to the DC_8A_n28A-n78A, the ΔTIB,c and ΔRIB,c can be specified below

Table 6.X.3-1: ΔTIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_8-28_n77 | 0.6 | 0.5 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

Table 6.X.3-2: ΔRIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_8-28_n77 | 0.2 | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

6.X.4	Analysis of MSD requirements

Referring to the DC_28A_n8A-n78A and DC_8A_n28A-n78A, the MSD due to IMD4 and IMD5 can be specified below.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-28A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 28 | N/A | 5 | N/A | 765 | 11.6 | IMD4 |
|  | n77 | 3495 | 10 | 50 | 3495 | N/A | N/A |
|  | 8 | N/A | 5 | N/A | 935 | 4.3 | IMD5 |
|  | 28 | 713 | 5 | 25 | 768 | N/A | N/A |
|  | n77 | 3787 | 10 | 50 | 3787 | N/A | N/A |

---End of changes---

Reference

[1]	RP-243257, “Revised WID for for Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands”, Nokia, CHTTL, LGE, Samsung