3GPP TSG-RAN WG4 Meeting #114	R4-2502694

Athens, Greece, 17 – 21 February, 2025

Title:	LS on neighboring cell measurement for Rel-18 SSB-less

Response to:

Release:	Release 18

Work Item:	Netw_Energy_NR

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:	Chenchen Zhang

E-mail Address:	zhang.chenchen@zte.com.cn

- Overall Description:

RAN4 is discussing the SSB based neighbour cell measurement on the carrier frequency of SSB-less SCell. From RAN4 understanding, the definition of intra-frequency/inter-frequency measurement, whether MG is needed and which requirements apply depends on whether servingCellMO is present or not.

Based on the agreements in RAN4#114 meeting, RAN4 provides analysis for the following cases:

- If the servingCellMO indication is configured:

      - Intra-frequency/inter-frequency: The SSB based neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB frequency configured in the MO indicated by servingCellMO; otherwise, it is defined as inter-frequency.

      - UE Requirements:

- The requirements of intra-frequency without measurement gap are defined in Clause 9.2.5 TS 38.133

- The requirements of intra-frequency with measurement gap are defined in Clause 9.2.6 TS 38.133

- The requirements of inter-frequency with measurement gap are defined in Clause 9.3.4 TS 38.133

- The requirements of inter-frequency without measurement gap are defined in Clause 9.3.9 TS 38.133

- If the servingCellMO indication is not configured:

      - Intra-frequency/inter-frequency: Based current RAN4 framework on Intra-frequency/inter-frequency definition, the SSB based neighbor cell measurement is defined as inter-frequency.

      - UE Requirements:

- The requirements of inter-frequency with measurement gap are defined in Clause 9.3.4 TS 38.133

- The requirements of inter-frequency without measurement gap are defined in Clause 9.3.9 TS 38.133

Based on above information, it is up to RAN2 to decide whether servingCellMO can be present or not and detailed signalling design.

The potential conflict (if any) when servingCellMO is present is up to RAN2 to handle.

Note: The servingCellMO is based on the current RAN2 specification, it does not preclude any RAN2 discussion or signalling design for the same purpose.

2. Actions:

To RAN2:

RAN4 respectfully asks RAN2 to take the above into account.

3. Date of Next TSG-RAN4 Meetings:

RAN WG4 Meeting #114-bis		April 07 – April 11, 2025	         Wuhan, China

RAN WG4 Meeting #115    		May 19 – May 23, 2025	         Malta, MT