3GPP TSG-RAN WG4 Meeting #114	   R4-2503015

Title:	LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27

Response to:	R4-2503079 on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27

Release:	Rel-19

Work Item:	FS_NR_IMT_4400_7125_14800MHz

Source:	RAN4

To:	RAN

Cc:

Contact Person:

Name:	Man Hung Ng

Email:		man_hung.ng AT nokia DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN WG4 received the incoming LS from ITU-R Working Party 5D on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27 (doc 5D/TEMP/273) and would like to provide clarification on this topic.

2. Actions:

To RAN:

ACTION: 3GPP RAN WG4 asks RAN to consider forwarding the clarification provided in the Annex to ITU-R Working Party 5D.

3. Date of Next RAN4 Meetings:

RAN4#114-bis	7th – 11th April 2025				Wuhan, China

Annex

Title:	LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27

Response to:	5D/TEMP/273 on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27

Release:	Rel-19

Work Item:	FS_NR_IMT_4400_7125_14800MHz

Source:	RAN

To:	ITU-R WP 5D

Cc: 	RAN4

Contact Person:

Name:	Man Hung Ng

Email:		man_hung.ng AT nokia DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN WG4 received the incoming LS from ITU-R Working Party 5D on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27 (doc 5D/TEMP/273) and would like to thank for the opportunity to clarification on this topic.

As correctly observed by ITU-R Working Party 5D, the spectral mask provided in doc 5D/428 for AAS IMT base station in the frequency range 14 800 to 15 350 MHz is similar to the spectral mask of AAS IMT base station in the frequency range 7 125 to 8 400 MHz for 0 MHz ≤ ∆f < 50MHz, and similar to the non-AAS IMT base station in the frequency range 7 125 to 8 400 MHz for ∆f ≥ 50MHz.

RAN WG4 has discussed and agreed that the spectral mask of AAS IMT base station in the frequency range 14 800 to 15 350 MHz for ∆f ≥ 50MHz should also be similar to the spectral mask of AAS IMT base station in the frequency range 7 125 to 8 400 MHz for ∆f ≥ 50MHz. Accordingly, the spectral mask of AAS IMT base station in the frequency range 14 800 to 15 350 MHz is updated in the table below.

TABLE 1A

AAS BS Spectral mask (Operating band unwanted emissions limits) for 
14 800-15 350 MHz operation (Category A)

| Frequency offset of measurement filter ‒3dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz £ Df < 50MHz | 2dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz £ Df < min(100 MHz, Dfmax) | ‒5 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | ‒4 dBm | 1 MHz |
| NOTE: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

Furthermore, RAN WG4 has discussed and agreed that the spectral masks as provided in Tables 1B, 1C, and 1D below should be used as the spectral masks of AAS IMT base station for micro cell with rated carrier total radiated power (Prated,c,TRP) £ 47 dBm and small cell indoor with rated carrier total radiated power (Prated,c,TRP) £ 33 dBm for 14 800 to 15 350 MHz. Moreover, RAN WG4 would like to clarify that the AAS BS spurious emissions for 14800 - 15350 MHz operation in Table 2A in doc 5D/428 can also be used for micro cell and small cell indoor.

TABLE 1B

AAS BS Spectral mask (Operating band unwanted emissions limits) for Micro cell for
14 800-15 350 MHz operation, 40 < Prated,c,TRP  47 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | Prated,c,TRP-53dB-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | Prated,c,TRP - 60dB | 100 kHz |
| 100 MHz f fmax | 100.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP - 60dB, -16dBm) | 100 kHz |
| NOTE 1: Prated,c,TRP is rated carrier total radiated power declared per radiated interface boundary. NOTE 2: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |  |

TABLE 1C

AAS BS Spectral mask (Operating band unwanted emissions limits) for Micro cell for
14 800-15 350 MHz operation, Prated,c,TRP  40 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -13dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -20 dBm | 100 kHz |
| 100 MHz f fmax | 100.05 MHz f_offset < f_offsetmax | -20 dBm | 100 kHz |
| NOTE 1: Prated,c,TRP is rated carrier total radiated power declared per radiated interface boundary. NOTE 2: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |  |

TABLE 1D

AAS BS Spectral mask (Operating band unwanted emissions limits) for Small cell indoor for
14 800-15 350 MHz operation

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -21dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -28 dBm | 100 kHz |
| 100 MHz f fmax | 100.05 MHz f_offset < f_offsetmax | -28 dBm | 100 kHz |
| NOTE: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |  |

2. Actions:

To ITU-R WP5D:

ACTION: 3GPP RAN WG4 asks ITU-R WP 5D to consider the updates provided above for 14800 to 15350 MHz.

3. Date of Next RAN4 Meetings:

RAN4#114-bis	7th – 11th April 2025				Wuhan, China