3GPP TSG-RAN WG4 #114bis	R4-2503127

Wuhan, China, April 7-11, 2025

Agenda item:	7.22.3.1		Enhancement for UE-initiated/Event-driven Beam Management

Source: 	Qualcomm Incorporated

Title: 	Views for enhanced UE-initiated/event-driven beam management

Document for:	Discussion

# Introduction

In RAN4 #112 the work on Rel-19 MIMO Phase 5 was started. The following agreements were made in RAN4 #114 for UE-initiated/Event-driven beam management and additional open issues have been identified in [1].

Issue 1-13: Whether to enhance beam sweeping in event triggered L1 beam reporting?

Agreement: Reuse current beam sweeping factor for UE/event triggered beam reporting.

Issue 1-10: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM

Agreement:

- Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

Issue 1-16: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

Agreement

  - The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

In this paper we provide our views for the open issues identified in the way forward in [1] in RAN4 #114.

# Discussion

In RAN4 #114 good progress was made on the definition of event triggered. It was agreed to reuse the current beam sweeping factor without additional enhancements in Rel-19 MIMO Phase 5. Additionally, it was agreed that no new accuracy requirements are defined, and existing L1-RSRP measurement accuracy requirements can be applied.

The agreement for Issue 1-2 as shown below shows that the measurement period TL1-RSRP_event triggered_measurement_Period_SSB for SSB and the corresponding measurement period for CSI-RS is reused as much as possible. The parameters for TReport and N are already agreed while other factors like M and TSSB are still under discussion. M  {1, 3} is configured by the RRC parameter timeRestrictionForChannelMeasurement. The way forward in [1] states that it is FFS pending further RAN1 agreements which value to assume in defining the requirements for the measurement period. However, it is unclear whether RAN1 is still discussing the time restriction for channel measurement.

Observation 1: It is unclear whether RAN1 is still discussing the time restriction for channel measurement M  {1, 3}. No agreement has been made in RAN1 #120.

Also, the parameter P is still open, although it is not mentioned in the way forward. P ≥ 1 is a value for the SSB resources to be measured in case some of the SSB are not available due to collisions with measurement gaps.

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1)?

Agreement:

The agreements under this issue is for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1.

TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS

  - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4

    - TReport = 0 or be removed from the measurement period

    - N: Reuse the legacy definition

    - M: FFS pending RAN1 further agreement

    - TSSB (taking SSB as example)

      - For the same periodicity supported in RAN1, TSSB_current, or TSSB_new

      - RAN4 to further discuss the case of different periodicities if different periodicities will be [supported or not excluded] in RAN1

        - Option A: Max{TSSB_current, TSSB_new}

        - Option B: When the RS periodicity is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new.

        - Option C: others are not precluded

In our previous paper [4], we analyzed whether the existing measurement period for N = 8, M = 1, P = 1 can be reused and concluded that this can be done if for the non-detecting beam one sample is used in the measurement process before the event happened OTA. This principle can be extended for M > 1 and P > 1 and is shown in Figure 2-1 for P > 1. The circles in red indicate reference signals that are not available for measurements due to collisions with gaps causing P to become larger than one. In this approach the measurement period for the detecting beam is simply extended by a factor of P while for the non-detecting beam potentially more samples are taken before the event happened OTA. This is the same approach that we already agreed for the beam sweeping factor. It can simply be extended for P > 1 and M > 1.

Figure 2-1: Measurement Period for Event-2 for P > 1

We also do not see why RAN4 needs to wait for RAN1 to make further progress on timeRestrictionForChannelMeasurement. In our opinion, RAN4 can continue the definition of the delay requirements for the measurement period taking both possible values of M into account.

Proposal 1: For the case M > 1 and P > 1, the measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam.

Proposal 2: RAN4 should define delay requirements for the measurement period of UE-initiated/event-driven beam management taking both possible values of M into account.

In RAN1 #119 it was agreed that at least the case is supported in Rel-19 that the evaluation periodicity for event-2 is identical for the current and the new beam. Also, it was agreed in RAN1 #119 that the evaluation periodicity equals the periodicity of the RS of the current and the new beam, i.e., both RS of the current and the new beam have the same periodicity as well. A LS by RAN1 to RAN4 was received in RAN4 #114 confirming this agreement [3].

Issue 1-3: Same or different periodicities of current beam and new beam(s)?

<Way forward >

- Option 1: (Qualcomm, Apple, CMCC, Samsung, vivo)

  - The same periodicities of current beam and new beam(s) to define the requirements, until further RAN1 agreements on different periodicities.

  - According to RAN1 LS (R1-2410914)

- Option 2: (Huawei)

  - From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations. Send LS to RAN1

- Option 3: (vivo)

  - RAN4 may further discuss RRM requirements for Alt-2 and the potential restrictions to the requirement applicability if time allows, based on RAN1 further agreements.

The case of different periodicities of the current beam RS and the new beam RS(s) is still open in RAN1 after RAN1 #120. Hence, RAN4 should focus on the case of same periodicities of the current beam RS and the new beam RS(s) to define measurement reporting delay requirements.

Observation 2: In RAN1 #119 it was agreed that at least the case is supported in Rel-19 that the evaluation periodicity for event-2 is identical for the current and the new beam and that the evaluation periodicity equals the periodicity of the RS of the current and the new beam.

Observation 3: The case of different periodicities of the current beam RS and the new beam RS(s) is still under discussion in RAN1 but has not been agreed yet.

Proposal 3: RAN4 should focus on the case of same periodicities of the current beam RS and the new beam RS(s) to define measurement reporting delay requirements.

Issue 1-4 deals with the components of the event triggered measurement reporting delay

Issue 1-4: Components of Event triggered measurement reporting delay?

<Way forward >

- Option 1: (Apple, vivo, Samsung, E///, MTK)

  - The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel

    - FFS on whether other components are needed.

    - FFS the definition of Tfirst UL channel

- Option 2: (CMCC, Xiaomi)

  - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering.

- Option 3: (Nokia)

  - The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (TL1-RSRP_measure_period).

Figure 2-2 illustrates the different contributions to the measurement reporting delay. It can be seen from the figure that there are several contributions to the overall measurement reporting delay, i.e., Tgap, Tmeas and TPUCCH.

Figure 2-2 shows a contribution to the overall measurement reporting delay Tgap. Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS or SSB resource for measuring. Tgap represents a delay uncertainty and is bounded by the periodicity of the CSI-RS or SSB resources. It is a similar uncertainty as TEvent_DU in the conditional handover core requirements in TS 38.133. In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted, see for example the test case definition for conditional handover in section A.6.3.3 in TS 38.133.

Observation 4: Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS resource for measuring.

Observation 5: In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted.

The next contribution to address is TPUCCH in Figure 2-2. Once the triggering event has been identified by the UE, the first PUCCH can be sent by the UE when the UL resources are available. After the event has been detected by the UE, there is a waiting time, indicated as TPUCCH in the figure, till the UE can sent the first PUCCH. The situation is like the case of event-triggered periodic reporting of L3 measurements as described in sections 9.2.4.2 and 9.2.4.3 of TS 38.133. Here the measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. […] This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

Figure 2-2: Illustration of Contributions to Measurement Reporting Delay

Observation 6: For event-triggered periodic reporting of L3 measurements delay uncertainty occuring when inserting the measurement report to the TTI of the uplink DCCH and the case of no UL resources are excluded from the reporting delay.

The same principle should be reused for UE-initiated/event-triggered beam reporting. Requirements should be defined in such a way that the case of no uplink resources is excluded and the waiting time till UL resources are available is also excluded.

Proposal 4: Delay uncertainty till the first PUCCH can be sent after the event has been identified by the UE and the case of no UL resources should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting.

Option 3 in Issue 1-4 states that the UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (TL1-RSRP_measure_period). We think that this option provides a feasible way forward. It could be added to the core requirements that the UE shall send the beam report in the next available PUCCH resource to ensure that the reporting delay is kept as small as possible.

Proposal 5: Option 3 in Issue 1-4 should be added to the core requirements that the UE shall send the beam report in the next available PUCCH resource to ensure that the reporting delay is kept as small as possible.

One of the main open discussion points after RAN4 #114 is whether to define requirements for the window-based approach with configurable M (Issue 1-6). Various proposals were made in RAN4 #114 how to define requirements for the window-based approach, see options 2, 2a, 2b, 2c below, however it seems that none of them is aligned with the current status in RAN1.

Issue 1-6: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M?

<Way forward >

- Option 1: (Qualcomm, Apple, Ericsson)

  - Only define requirements for the number of events is met at once

- Option 2: (China Telecom, ZTE, Huawei, Samsung, MediaTek)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable M

- Option 2a: (ZTE)

  - Event1&Event 2: Case 2: multiple samples M and timer X should be considered

  - Event1&Event 7: FFS

- Option 2b (Samsung)

  - For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event.

- Option 2c (MediaTek)

  - For multiple events is met by counter:

    - Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window

    - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS

    - C: event instance counter is configured by NW.

In RAN1 #120, the following agreement was reached.

Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, further study the following options for possible down-selection

- Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured.

- Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured.

  - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s).

- Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

- Note: T_window is the agreed time window parameter for measurement.

Note: Other options are not precluded.

It is seen that the topic how to define the details of the window-based approach was still under discussion in RAN1 #120, but no conclusion could be reached in RAN1. It seems also in RAN2 it has not yet been discussed how to define the window. Hence, options 2a, 2b and 2c in Issue 1-6 above are premature in the RAN4 discussion. Since RAN4 has agreed that the existing Rx beam sweeping factor N is reused for UE-initiated/Event-driven beam management, the window-based approach needs to take the beam sweeping factor N, the factor P and the factor M  {1, 3} into account in case it is decided that requirements in RAN4 are defined. However, it seems that RAN1 is not yet aware of the agreements reached in RAN4. It may help RAN1 and RAN2 to make further progress in defining the windows-based approach if RAN4 sends an LS to RAN1 and RAN2 summarizing the RAN4 agreements for the measurement periods.

Observation 7: How to define the details of the window-based approach was still under discussion in RAN1 #120, but no conclusion could be reached in RAN1. It seems in RAN2 the topic how to define the window has also not been discussed.

Proposal 6: All further discussion on the details of the measurement procedure of the window-based approach in RAN4 should be postponed till RAN1 and/or RAN2 has concluded.

Proposal 7: RAN4 should discuss whether a LS should be sent to RAN1 and RAN2 summarizing the RAN4 agreements for the measurement periods.

However, it has been agreed in RAN4 #113 that the UE does not do any filtering of L1-RSRP measurements if a configurable time window is applied. If no filtering is applied, it is not obvious whether additional reporting delay requirements for the configurable time window needs to be defined. The measurement would simply extend by a factor based on the configured value M. Hence, we do not see that defining additional requirements tests much more functionality of the UE.

Observation 8: In case that no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested.

Proposal 8: RAN4 should not define requirements for the window-based approach whether the event has occurred M times.

Issue 1-9 is related to the question whether CSI-RS based L1-RSRP should be defined for inter-cell beam management.

Issue 1-9: CSI-RS based L1-RSRP for inter-cell BM

<Way forward >

- Option 1: (Qualcomm, Nokia, MediaTek(descoped))

  - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

- Option 2: (Apple, Huawei, Ericsson)

  - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

  - Option 2a (Apple): The measurement delay is the same as that in serving cell in legacy L1-RSRP

  - Option 2b (Ericsson): reuse the principle of sharing agreed for SSB based measurement.

- Option 3: (ZTE)

  - RAN4 needs to identify whether additional clarifications on the following points are necessary:

    - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

    - The prerequisite on L3 measurement on NSC.

    - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Option 4: (Ericsson)

  - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2

  - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP. We propose not to introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

Observation 9: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-based L1-RSRP.

Proposal 9: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

In RAN4 #114 it was agreed that the existing conditions for known and unknown TCI states can be reused for UE-initiated / event-triggered beam management. It still needs to be discussed whether new delay requirements for unified TCI state switching should be introduced (Issue 1-11).

Issue 1-11: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- Option 1: (Qualcomm, Apple, China Telecom, Samsung (For known conditions for TCI states), Ericsson, MediaTek)

  - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

- Option 2: (Xiaomi, Nokia)

  - Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

- Option 3: (LGE)

  - For unified TCI switching delay requirements, RAN4 needs to discuss whether active TCI state switching delay can be reduced or not

- Option 4: (ZTE)

  - For MAC-CE based DL TCI switching delay: no need to consider TL1-RSRP and Tfirst-SSB+TSSB-proc

- Option 5: (Samsung)

  - Not consider unknown case.

After the gNB receives the beam report and executes a TCI state switch to a target TCI state, this target TCI state can be known or unknown since there is no guarantee how quickly the gNB would trigger the switch after the report has been received. Also, the target TCI state does not need to be in the active TCI state list for PDSCH. In section 8.10.3, 8.10.4 and 8.10.5 of TS 38.133, the TCI state switch delays are defined for MAC CE based, DCI based and RRC based switching, taking all the conditions mentioned above into account. Based on current RAN1 agreements, we believe that these TCI state delay switches can also be reused for UE-initiated/even-triggered beam management.

Proposal 10: The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114. Our observations and proposals are summarized as follows:

Proposal 1: For the case M > 1 and P > 1, the measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam.

Observation 1: It is unclear whether RAN1 is still discussing the time restriction for channel measurement M  {1, 3}. No agreement has been made in RAN1 #120.

Proposal 2: RAN4 should define delay requirements for the measurement period of UE-initiated/event-driven beam management taking both possible values of M into account.

Observation 2: In RAN1 #119 it was agreed that at least the case is supported in Rel-19 that the evaluation periodicity for event-2 is identical for the current and the new beam and that the evaluation periodicity equals the periodicity of the RS of the current and the new beam.

Observation 3: The case of different periodicities of the current beam RS and the new beam RS(s) is still under discussion in RAN1 but has not been agreed yet.

Proposal 3: RAN4 should focus on the case of same periodicities of the current beam RS and the new beam RS(s) to define measurement reporting delay requirements.

Observation 4: Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS resource for measuring.

Observation 5: In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted.

Observation 6: For event-triggered periodic reporting of L3 measurements delay uncertainty occuring when inserting the measurement report to the TTI of the uplink DCCH and the case of no UL resources are excluded from the reporting delay.

Proposal 4: Delay uncertainty till the first PUCCH can be sent after the event has been identified by the UE and the case of no UL resources should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting.

Proposal 5: Option 3 in Issue 1-4 should be added to the core requirements that the UE shall send the beam report in the next available PUCCH resource to ensure that the reporting delay is kept as small as possible.

Observation 7: How to define the details of the window-based approach was still under discussion in RAN1 #120, but no conclusion could be reached in RAN1. It seems in RAN2 the topic how to define the window has also not been discussed.

Proposal 6: All further discussion on the details of the measurement procedure of the window-based approach in RAN4 should be postponed till RAN1 and/or RAN2 has concluded.

Proposal 7: RAN4 should discuss whether a LS should be sent to RAN1 and RAN2 summarizing the RAN4 agreements for the measurement periods.

Observation 8: In case that no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested.

Proposal 8: RAN4 should not define requirements for the window-based approach whether the event has occurred M times.

Observation 9: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-based L1-RSRP.

Proposal 9: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

Proposal 10: The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management.

# References

- R4-2502595, WF on RRM requirements for NR_MIMO_Ph5_Part1, RAN4 #114, February 2025

- R1-2501556, Moderator Summary #4 on UE-initiated/event-driven beam management, RAN1 #120, February #120

- R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management, RAN1 #119, November 2024

- R4-2500054, Further considerations for enhanced UE-initiated/event-driven beam management, Qualcomm, RAN4 #114, February 2025