3GPP TSG-RAN WG4 #114bis	R4-2503128

Wuhan, China, April 7-11, 2025

Agenda item:	7.22.3.2		Other RRM Requirements

Source: 	Qualcomm Incorporated

Title: 						Views on additional RRM requirements for NR MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #112, the work on Rel-19 MIMO Phase 5 was started. Several agreements were made to identify impacts on RRM requirements, and several open issues were outlined in [1].

In this paper we provide our views for the open issues identified in the way forward in [1] in RAN4 #114.

# Discussion

## UE Reporting Enhancements for CJT Deployments

In the previous RAN4 meetings first discussions about the accuracy requirements for delay and frequency offset reporting were started. Background for this discussion is whether one or multiple samples are needed to provide sufficient accuracy for the reporting, since, depending on the number of required samples, core or performance requirements are impacted in TS 38.133. To investigate the measurement performance, simulation assumptions for delay and frequency offset reporting have been agreed in [2] and been refined in [1].

CJT reporting enhancements support NTRP  {1, 2, 3, 4} TRPs. The reports are relative to a reference TRP nref that is selected by the UE. Overall, the UE is configured with NTRP NZP-CSI-RS resources/resource sets with NTRP  {1, 2, 3, 4}. Further it has been agreed in RAN1 that the delay offset is uniformly spaced between 0 and AD and the frequency offset is uniformly spaced between between 0 and AFO. Since the reported delay and frequency offsets cannot be lower than zero, the UE needs to select the TRP with lowest estimated delay and lowest estimated frequency offset relative to the carrier frequency as the reference TRP nref. The upper limits AD and AFO and the number of uniform steps MD and MFO can be configured by the network. RAN1 agreed on AD = {0.5CP, CP}, MD = {32, 64, 128, 256} and AFO = {0.1ppm, 0.2ppm}, MFO = {16, 32, 256}.

Before we start looking into simulation results, RAN4 should define what accuracy requirements should be achieved. Based on the simulation assumptions in [2], a TDD system with a carrier frequency of 3.5 GHz can be assumed. Under these assumptions, the quantization step sizes are Δt = 0.5 CP / 255 = 0.5 * 2.3 ms / 255 = 4.5 ns and Δf = 0.1ppm of 3.5 GHz / 255 = 1.37 Hz for the most accurate case of MD = 256 and MFO = 256.

For DL, no requirements for coherency are defined. However, requirements for coherent UL MIMO are defined in [3] as the maximum allowable difference of relative phase error over a time window. The allowable difference of relative phase error is defined as 40 over a time window of 20 ms. We think that this requirement can be also used as a reference for phase coherency in time and frequency domain for delay and frequency offset reporting.

Observation 1: Requirements for coherent UL MIMO are defined in TS 38.101-1 as the maximum allowable difference of relative phase error over a time window. The allowable difference of relative phase error is defined as 40 over a time window of 20 ms.

Proposal 1: RAN4 should discuss whether 40 can be used as a reference for phase coherency in time and frequency domain for delay and frequency offset reporting.

Assuming 40 as reference for phase coherency, we derive the accuracy requirements for delay and frequency offset reporting.

- Assuming τn0 [seconds] is the timing misalignment between TRPs. For a SCS = ∆f and a subcarrier index k, the phase rotation in frequency domain is then given by 2πk∆fτn0.

- Assuming en [ppm] is the frequency offset between TRPs. For a symbol duration T, a symbol index l and a carrier frequency fc, the phase rotation in time direction is given by 2πenfclT.

For a phase rotation of 40 in time domain, we can now do the following calculation:

- 40 corresponds to 0.35 rads. T=130 kHz, l=14 for one slot = 0.5 ms. If we want to achieve a phase coherency across 20 ms, then l=560.

- 0.35=2πenfclT=2πenfcl30 kHz=2πenfc56030 kHz⇒enfc=0.35∙30 kHz560 ∙ 2π≈3 Hz

For a phase rotation of 40 in frequency domain, we can do the following calculation:

- 40 corresponds to 0.35 rads. ∆f=30 kHz, k=12 for one PRB. If we want to achieve a phase coherency of one sub-band of 8 PRB, then k=96.

- 0.35=2πk∆fτn0=192π∙30 kHz ∙ τn0⇒τn0=0.35192π∙30 kHz ≈19 ns

Observation 2: If 40 is assumed as a reference for phase coherency across a time duration of 20 ms and a sub-band size of 8 PRBs, the required accuracy requirements for delay offset and frequency offset reporting are 19 ns and 3 Hz, respectively.

Proposal 2: RAN4 should discuss whether an accuracy of 19 ns and 3 Hz for delay offset, and frequency offset reporting can be considered as accuracy requirement to derive how many samples for measurement are required.

## Asymmetric DL sTRP / UL mTRP

In this contribution we provide our view to the open issues identified in RAN4 #114. In RAN4 #114, two scenarios to be covered by RAN4 have been identified in Issue 2-1:

- Scenario #1: in this scenario, the UE is not expected to receive SSB from UL TRP(s)

- Scenario #2: in this scenario, the UE expects to receive SSB from UL TRP(s)

Issue 2-1: Whether RAN4 need to consider the scenario when UE is/isn’t configured with pathloss offset?

Agreement:

For Scenario#1: RAN4 define the timing requirements for FR1, FFS on FR2.

For Scenario#2, FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not).

RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.” are currently only applied for Scenario #1.

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s).

Note 1: the scenario 1 and 2 are based on RAN1 LS.

Note 2: The above agreements apply when 2TA are configured.

For scenario #1, the uplink timing has been agreed in Issue 2-2 for both TAGs.

Issue 2-2: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

- For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state.

  - The above agreement can be revisited if not aligned with RAN1 further agreement.

For scenario #2, RAN4 needs to wait whether RAN1 extends the existing agreement of a single downlink timing reference. For that purpose, a LS has been sent to RAN1 to ask about their view whether a second downlink timing reference should be introduced [3].

Proposal 3: For scenario #2, RAN4 should wait for RAN1 to reply to the LS whether the existing agreement for a single downlink timing reference is extended, and a second downlink reference timing is introduced.

One of the open questions for scenario #1 is how to handle autonomous timing adjustment. This topic was discussed in RAN1 #118bis and RAN1 #119 without reaching a clear agreement. However, it seems clear that there is no RAN1 spec impact. In our understanding the above-mentioned autonomous timing adjustment is the gradual timing adjustment defined in TS 38.133. RAN4 should discuss how gradual timing adjustment should be applied in scenario #1 when there is just one downlink reference signal but two uplink TAGs. Our view is that the same gradual timing adjustment should be applied to both TAGs.

Proposal 4: For scenario #1 gradual timing adjustment should be applied with the same value for both TAGs.

Regarding the UL TCI switching requirements, there was a discussion in the previous meetings whether to define UL TCI switching requirements based on SRS. The current status of the discussion is summarized in Issue 2-3. The purpose of uplink TCI state switching is to define whether additional time is needed in the UE to apply the new uplink TCU state. For a known TCI state, the switching time is just given by the MAC CE timeline. Additional time is needed in case the TCI state is not known that is different for FR1 and FR2. In case of FR2, additional time for Rx beam refinement is required. In case that there is no downlink pathloss reference signal as it is the case for scenario #1, the network tells the UE either what uplink TCI state to use or it tells the UE that SRS beam sweeping is needed for beam refinement. In both cases, the UE just follows the command by the network and timeline is fully specified by RAN1/RAN2. From a RRM point of view, no additional time is needed to execute the TCI switch or to execute the SRS beam sweeping. Potentially, additional requirements may be needed for the RF and/or the network for explicit uplink beam adjustment since there is no downlink correspondence when UL TRP does not transmit a downlink reference signal. But these additional requirements are outside the scope of RRM. Hence, we do not see a need to define RRM requirements for uplink TCI switching based on SRS.

Issue 2-3: Whether to define UL TCI state activation requirement based on SRS?

Way forward:

Further discuss the two proposals and down-select one proposal in the next meeting:

  - Proposal 1 (Qualcomm, Huawei, MediaTek, Samsung, Apple, QC): No RRM impacts on TCI state switching requirement.

  - Proposal 3 (Xiaomi, Nokia):

    - Legacy DCI based UL TCI state activation requirement can be re-used for UL TRP.

    - RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline.

    - For DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. no need to define SRS based ULTCI state activation delay.

    - Further refine the wording for P3 in the WF, if needed

Observation 3: For SRS based TCI state activation, the UE follows the command by the network and applies the timelines being defined by RAN1/RAN2. There is no need for defining RRM tests for SRS based TCI switching since no additional time is needed by the UE.

Proposal 5: RAN4 should not define RRM TCI state switching requirements based on SRS since no additional RRM capability in the UE is tested.

In case that the UL TRP does not transmit SSB (scenario #1), SRS beam sweeping is required for FR2 in the UE to let the gNB find out the best uplink beam and switch the uplink beam accordingly. Although we see no need to define TCI state switching requirements based on SRS in RRM, there are several challenges for SRS beam sweeping for FR2 in RF. In addition, how to support testing SRS beam sweeping from a TE point of view, may require further discussions in RAN5.

Observation 4: Whether or whether not additional requirements for SRS beam sweeping are needed, should be discussed in RF session or in RAN5.

Generally, it is not clear how well the system would work in case that the UL TRP does not transmit SSB (scenario #1) and continuous SRS beam sweeping is required. Hence, we propose that RAN4 focuses for FR2 only on the case that UL TRP transmits SSB (scenario #2). This is in line with the discussions in the RF room.

Proposal 6: RAN4 should focus for FR2 only on the case that UL TRP transmits SSB (scenario #2).

In case that the UL TRP transmits SSB (scenario #2), there is a downlink pathloss reference signal. In this case existing requirements for uplink TCI state switching can be reused, and no additional requirements need to be defined.

Proposal 7: In case that the UL TRP transmits SSB (scenario #2), existing requirements for uplink TCI state switching can be reused, and no additional requirements need to be defined.

For the case that the UL TRP transmits SSB (scenario #2), it should be noted that the UL TRP may not transmit SIB1. Hence, the SSB transmitted by the UL TRP should be an NCD-SSB but not a CD-SSB. RAN4 should only focus on this case that the UL TRPs transmit NCD-SSBs

Observation 5: Since the UL TRP may only transmit SSB but no SIB1, the SSB from the UL TRP should be an NCD-SSB.

Proposal 8: RAN4 should only focus on the case that the UL TRPs transmit NCD-SSBs.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114. Our observations and proposals are summarized as follows:

UE reporting enhancement for CJT deployments

Observation 1: Requirements for coherent UL MIMO are defined in TS 38.101-1 as the maximum allowable difference of relative phase error over a time window. The allowable difference of relative phase error is defined as 40 over a time window of 20 ms.

Proposal 1: RAN4 should discuss whether 40 can be used as a reference for phase coherency in time and frequency domain for delay and frequency offset reporting.

Observation 2: If 40 is assumed as a reference for phase coherency across a time duration of 20 ms and a sub-band size of 8 PRBs, the required accuracy requirements for delay offset and frequency offset reporting are 19 ns and 3 Hz, respectively.

Proposal 2: RAN4 should discuss whether an accuracy of 19 ns and 3 Hz for delay offset, and frequency offset reporting can be considered as accuracy requirement to derive how many samples for measurement are required.

Asymmetric DL sTRP/UL mTRP

Proposal 3: For scenario #2, RAN4 should wait for RAN1 to reply to the LS whether the existing agreement for a single downlink timing reference is extended, and a second downlink reference timing is introduced.

Proposal 4: For scenario #1 gradual timing adjustment should be applied with the same value for both TAGs.

Observation 3: For SRS based TCI state activation, the UE follows the command by the network and applies the timelines being defined by RAN1/RAN2. There is no need for defining RRM tests for SRS based TCI switching since no additional time is needed by the UE.

Proposal 5: RAN4 should not define RRM TCI state switching requirements based on SRS since no additional RRM capability in the UE is tested.

Observation 4: Whether or whether not additional requirements for SRS beam sweeping are needed, should be discussed in RF session or in RAN5.

Proposal 6: RAN4 should focus for FR2 only on the case that UL TRP transmits SSB (scenario #2).

Proposal 7: In case that the UL TRP transmits SSB (scenario #2), existing requirements for uplink TCI state switching can be reused, and no additional requirements need to be defined.

Observation 5: Since the UL TRP may only transmit SSB but no SIB1, the SSB from the UL TRP should be an NCD-SSB.

Proposal 8: RAN4 should only focus on the case that the UL TRPs transmit NCD-SSBs.

# References

- R4-2502596, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4 #114, February 2025

- R4-2420209, Simulation assumptions for CJT calibration, RAN4 #113, November 2024

- 3GPP TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18), v18.7.0, September 2024

- R4-2502667, LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP / UL mTRP, RAN4 #114, February 2025