3GPP TSG-RAN WG4 Meeting #114bis	R4-2503142

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.1.3

Source:	MediaTek Inc.

Title:	Discussion on demodulation and CSI requirements for 6Rx UE

Document for:	Discussion

# Introduction

Based on the latest revised WID [1], RAN4 is tasked to define 6Rx performance requirements. In this contribution, we provide our views about requirements for 6Rx UE.

- Discussion

## 2.1 General

Scope

Based on the core part and performance part in WID, RAN4 should focus on introducing requirements for the scenario of FR1, single carrier, and TDD band. Furthermore, RAN4 should focus on PDSCH, CSI and SDR requirements for 6Rx.

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 ……. 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx General for all the areas - Specify release independence requirements for performance part in TS 38.307/36.307, if needed and feasible.

Observation 1: According to the WID, RAN4 should focus on introducing requirements for the scenario of FR1, single carrier, and TDD band.

Proposal 1: RAN4 defines the 6Rx UE requirements by assuming:

- FR1 single carrier only (do not consider carrier aggregation)

- TDD with SCS = 30 kHz, CBW = 40 MHz and 106 RBs

- FR1.30-1 TDD UL-DL pattern, given by 7DS2U and S: 6D+4G+4U

Observation 2: According to the WID, RAN4 should focus on discussing PDSCH, CSI and SDR requirements.

Proposal 2: RAN4 should focus only on discussing requirements for PDSCH, SDR, and CSI for 6Rx UE, without considering the requirements for PDCCH and PBCH.

TxEVM

TxEVM is a critical parameter that must be taken into consideration when evaluating the performance requirements. It quantifies the deviation between the ideal transmitted signal and the actual transmitted signal, which directly impacts the quality and reliability of the received signal. High TxEVM values can lead to significant signal distortion, thereby degrading the demodulation accuracy and overall performance. We propose to re-use same EVM values in Rel-15, i.e., EVM = 6% for QPSK/16QAM/64QAM and EVM = 3% for 256QAM.

Proposal 3: Consider the following EVM assumptions for introducing requirements for 6Rx UE:

- 6% for QPSK/16QAM/64QAM

- 3% for 256QAM

Static channel

The current specification, TS38.101-4 Annex B.1, outlines static propagation conditions for configurations with 2, 4, and 8 Rx, which are applied to PDSCH requirements (for LOS channels), CQI requirements (for AWGN channels), and SDR requirements. Consider a MIMO system with Nt transmit antennas and Nr receive antennas. The system can be represented by the following equation:

Y = HPX+N = HeffX + N

where:

  - Y is the Nr×1 received signal vector

  - N is the Nr×1 noise vector

  - H is the Nr×Nt static channel matrix

  - P is the Nt×L precoder

  - X is the L×1 transmitted signal vector

A multi-layer signal is conveyed through an effective channel matrix which is the result of combining the static channel with the precoding matrix. It is noteworthy that all effective channel matrices utilized in the current requirements are fully ranked, also ensuring that no combination of static channel and precoding matrix results in zero input at the receiving antenna. This guarantees exceptional orthogonality across the multiple signal layers. If RAN4 agreed to introduce either one of PDSCH requirements (LOS channel), CQI requirements (AWGN channel) or SDR requirements for 6Rx UE, RAN4 needs to introduce new static channels for 6Rx UE.

Observation 3: In current TS38.101-4, static channels are applied in PDSCH requirements (LOS channel), CQI requirements (AWGN channel) and SDR requirements.

Proposal 4: RAN4 should introduce new 6Rx static channel if RAN4 agreed to introduce either one of PDSCH requirements (LOS channel), CQI requirements (AWGN channel) or SDR requirements for 6Rx UE.

Observation 4: In current TS38.101-4, the effective channel matrix, which is a combination of the static channel and precoding matrix, can maintain good orthogonality across multi-layers. All effective channel matrices used in existing requirements are of full rank and leading no zero input at the receiving antenna.

Proposal 5: If 6Rx static channels are agreed to be introduced, the effective channel matrix, which is a combination of the introduced 6Rx static channel and precoding matrix, should maintain good orthogonality across multi-layers. The effective channel matrices should be full rank and leading no zero input at the receiving antenna.

## 2.2 PDSCH requirements

Handheld UE and FWA differ significantly in terms of form factor and mobility. Handheld UE, such as foldable phones or tablets, are designed to be compact and portable, allowing users to carry them easily and use them on the go. These devices are characterized by their high mobility, enabling seamless connectivity across various locations and supporting features like handovers between cells. In contrast, fixed wireless access equipment is typically larger in form factor and designed for stationary use, providing a stable and reliable internet connection to fixed locations such as homes, offices, and buildings. These devices are often mounted on buildings or installed in specific locations. We provide our views about MIMO correlation and propagation channel assumptions for defining requirements for handheld and FWA.

MIMO correlation

For the assumptions of MIMO correlation, we propose ULA low for FWA as the large form factor of FWA plays a significant role in reducing MIMO correlation by allowing for greater antenna separation. As for handheld UE with smaller form factor compared to FWA, we suggest leveraging legacy pragmatic approach used in RAN4. The spatial correlation coefficient in correlation matrices between the i-th and j-th antenna elements is expressed as rij=ρdijΔ2, where dij=(i-j)Δ(N-1) represents the separation between the antenna elements, and ρ denotes the correlation at a reference element separation of Δ. Additionally, N is the total number of antennas.

To be more specific, we suggest using β = 0.00718 for 6Rx (N = 6) to attain the same correlation value for 4Rx in Medium Correlation A (defined in Annex B.2.3.1.2 of TS38.101-4) between two adjacent antennas, i.e., β4Rx1/9=β6Rx1/25.

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1

Propagation channel

For the assumptions of propagation channel, high mobility (e.g., when the UE is moving at high speeds such as in a vehicle or train) leads to fast changes in the channel characteristics, which can make it difficult for the system to maintain the signal quality required for high ranks. This variability can reduce the effective rank because the channel may not support multiple independent streams simultaneously. As we are now considering 6 and 4 layers, we suggest using low mobility propagation channel to define requirements.

Observation 5: Handheld UE and fixed wireless access equipment differ significantly in terms of form factor and mobility

Proposal 6: Considering the different form factor assumptions for handheld UE and FWA, we propose using distinct MIMO correlation to define specific requirements for each.

- For FWA: ULA low

- For handheld UE: considering legacy pragmatic approach used in RAN4 to introduce new UE correlation matrix with β = 0.00718

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1

Observation 6: High mobility results in rapid changes in channel characteristics, making it challenging for the system to maintain stable and reliable communication for high-rank transmission.

Proposal 7: For 6- and 4-layer requirements, RAN4 should consider low mobility in the propagation channel. we propose using distinct MIMO correlation to define specific requirements for each.

- For FWA: TDLD30-5

- For handheld UE: TDLA30-10

DMRS configuration

As Rel-18 enhanced DMRS needs additional UE capabilities, we propose to use Rel-15 2+2 and Rel-15 1+1 DMRS configurations to define requirements for 6R6L and 6R4L requirements, respectively.

Proposal 8: Use Rel-15 2+2 and Rel-15 1+1 DMRS configurations to define 6R6L and 6R4L requirements, respectively.

Based on above proposals, we suggest that RAN4 only consider introducing three new PDSCH requirements for 6Rx covering FWA and handheld UE.

Proposal 9: RAN4 define three new PDSCH requirements for 6Rx covering FWA and handheld UE.

| Device Type/Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| FWA/Rank 6 | 40 / 30 | TBD | TDLD30-5 | 8x6 ULA Low | 70 | TBD |
| FWA/Rank 4 | 40 / 30 | TBD | TDLD30-5 | 8x6 ULA Low | 70 | TBD |
| handheld UE/Rank 4 | 40 / 30 | TBD | TDLA30-10 | 8x6 ULA withβ = 0.00718 | 70 | TBD |

## 2.3 CSI requirements

Comparing 2Rx and 4 Rx, there is not much difference for CSI processing considering 6Rx. Therefore, we propose not to consider CSI requirements for 6Rx.

Proposal 10: Not to introduce CSI requirements for 6Rx.

## 2.4 SDR requirements

The objective of the SDR test is to ensure that Layers 1 and 2 can consistently process incoming packets at the maximum data rate specified by the UE capabilities. The MCS indexes for 6Rx SDR tests can be defined as the minimum between MCSupperboundj and MCSpracticalj. MCSupperboundj is the highest MCS according to UE capability calculated by Sec. 2.1 [2]. MCSpracticalj is the highest practical MCS considering feasible SNR levels. To find highest practical MCS, the simulations need to be conducted by considering TxEVM and impairments.

Proposal 11: Define SDR requirements for 6Rx UE based on the below table.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS upper bound | MCS practical | MCS requirement |
| --- | --- | --- | --- | --- | --- |
| 6 | 8 | 1 | 27 | TBD | TBD |
| 6 | 8 | 0.8 | 23 |  | TBD |
| 6 | 8 | 0.75 | 22 |  | TBD |
| 6 | 8 | 0.4 | 12 |  | TBD |
| 6 | 6 | 1 | 28 | TBD | TBD |
| 6 | 6 | 0.8 | 25 |  | TBD |
| 6 | 6 | 0.75 | 24 |  | TBD |
| 6 | 6 | 0.4 | 15 |  | TBD |
| 6 | 4 | 1 | 16 | TBD | TBD |
| 6 | 4 | 0.8 | 16 |  | TBD |
| 6 | 4 | 0.75 | 16 |  | TBD |
| 6 | 4 | 0.4 | 11 |  | TBD |
| 6 | 2 | 1 | 9 | TBD | TBD |
| 6 | 2 | 0.8 | 9 |  | TBD |
| 6 | 2 | 0.75 | 9 |  | TBD |
| 6 | 2 | 0.4 | 5 |  | TBD |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |  |

In SDR requirements, 6Rx static channel must be utilized. Also, the motivation for SDR test is to verify the maximum TBS UE can handle in an optimal channel condition, which is the reason why noise is not introduced during the SDR test. Therefore, RAN4 needs to carefully design static channel for 6Rx to facilitate the testing for SDR.

Observation 7: The objective of the SDR test is to ascertain the largest transport block size the UE can manage in perfect channel conditions, which explains the absence of noise introduction during the SDR testing phase.

Proposal 12: RAN4 needs to carefully design static channel for 6Rx to minimize the effect of inter-layer interference before conducting simulation alignments for SDR requirements.

# Conclusion

In this contribution, we provide our views about performance requirements for 6Rx UEs. Observations and proposals are summarized as follows.

Observation 1: According to the WID, RAN4 should focus on introducing requirements for the scenario of FR1, single carrier, and TDD band.

Proposal 1: RAN4 defines the 6Rx UE requirements by assuming:

- FR1 single carrier only (do not consider carrier aggregation)

- TDD with SCS = 30 kHz, CBW = 40 MHz and 106 RBs

- FR1.30-1 TDD UL-DL pattern, given by 7DS2U and S: 6D+4G+4U

Observation 2: According to the WID, RAN4 should focus on discussing PDSCH, CSI and SDR requirements.

Proposal 2: RAN4 should focus only on discussing requirements for PDSCH, SDR, and CSI for 6Rx UE, without considering the requirements for PDCCH and PBCH.

Proposal 3: Consider the following EVM assumptions for introducing requirements for 6Rx UE:

- 6% for QPSK/16QAM/64QAM

- 3% for 256QAM

Observation 3: In current TS38.101-4, static channels are applied in PDSCH requirements (LOS channel), CQI requirements (AWGN channel) and SDR requirements.

Proposal 4: RAN4 should introduce new 6Rx static channel if RAN4 agreed to introduce either one of PDSCH requirements (LOS channel), CQI requirements (AWGN channel) or SDR requirements for 6Rx UE.

Observation 4: In current TS38.101-4, the effective channel matrix, which is a combination of the static channel and precoding matrix, can maintain good orthogonality across multi-layers. All effective channel matrices used in existing requirements are of full rank and leading no zero input at the receiving antenna.

Proposal 5: If 6Rx static channels are agreed to be introduced, the effective channel matrix, which is a combination of the introduced 6Rx static channel and precoding matrix, should maintain good orthogonality across multi-layers. The effective channel matrices should be full rank and leading no zero input at the receiving antenna.

Observation 5: Handheld UE and fixed wireless access equipment differ significantly in terms of form factor and mobility

Proposal 6: Considering the different form factor assumptions for handheld UE and FWA, we propose using distinct MIMO correlation to define specific requirements for each.

- For FWA: ULA low

- For handheld UE: considering legacy pragmatic approach used in RAN4 to introduce new UE correlation matrix with β = 0.00718

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1

Observation 6: High mobility results in rapid changes in channel characteristics, making it challenging for the system to maintain stable and reliable communication for high-rank transmission.

Proposal 7: For 6- and 4-layer requirements, RAN4 should consider low mobility in the propagation channel. we propose using distinct MIMO correlation to define specific requirements for each.

- For FWA: TDLD30-5

- For handheld UE: TDLA30-10

Proposal 8: Use Rel-15 2+2 and Rel-15 1+1 DMRS configurations to define 6R6L and 6R4L requirements, respectively.

Proposal 9: RAN4 define three new PDSCH requirements for 6Rx covering FWA and handheld UE.

| Device Type/Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| FWA/Rank 6 | 40 / 30 | TBD | TDLD30-5 | 8x6 ULA Low | 70 | TBD |
| FWA/Rank 4 | 40 / 30 | TBD | TDLD30-5 | 8x6 ULA Low | 70 | TBD |
| handheld UE/Rank 4 | 40 / 30 | TBD | TDLA30-10 | 8x6 ULA withβ = 0.00718 | 70 | TBD |

Proposal 10: Not to introduce CSI requirements for 6Rx.

Proposal 11: Define SDR requirements for 6Rx UE based on the below table.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS upper bound | MCS practical | MCS requirement |
| --- | --- | --- | --- | --- | --- |
| 6 | 8 | 1 | 27 | TBD | TBD |
| 6 | 8 | 0.8 | 23 |  | TBD |
| 6 | 8 | 0.75 | 22 |  | TBD |
| 6 | 8 | 0.4 | 12 |  | TBD |
| 6 | 6 | 1 | 28 | TBD | TBD |
| 6 | 6 | 0.8 | 25 |  | TBD |
| 6 | 6 | 0.75 | 24 |  | TBD |
| 6 | 6 | 0.4 | 15 |  | TBD |
| 6 | 4 | 1 | 16 | TBD | TBD |
| 6 | 4 | 0.8 | 16 |  | TBD |
| 6 | 4 | 0.75 | 16 |  | TBD |
| 6 | 4 | 0.4 | 11 |  | TBD |
| 6 | 2 | 1 | 9 | TBD | TBD |
| 6 | 2 | 0.8 | 9 |  | TBD |
| 6 | 2 | 0.75 | 9 |  | TBD |
| 6 | 2 | 0.4 | 5 |  | TBD |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |  |

Observation 7: The objective of the SDR test is to ascertain the largest transport block size the UE can manage in perfect channel conditions, which explains the absence of noise introduction during the SDR testing phase.

Proposal 12: RAN4 needs to carefully design static channel for 6Rx to minimize the effect of inter-layer interference before conducting simulation alignments for SDR requirements.

# Reference

- RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, HiSilicon

- R4-1812165, NR DSR performance requirements, Intel Corporation