# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503342

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	On RRM impacts for SBFD operation

Source: 	OPPO

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

In last meeting, a WF on RRM requirements for NR_duplex_evo was approved [1].

In this contribution, we will provide our views on RRM impacts for SBFD operation.

Discussion

- Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - Option 3: Consider the UE measurement capability - FFS whether and how to account for the UE capability on CSI processing time. - Further discuss the accuracy requirements for the following cases - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). - Issue 2-1-2: Requirements for CSI-RS based L3 measurement - Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

In our view, the measurement performance including measurement delay and accuracy should consider different UE implementation. For the worst case, UE is not able to measure wideband RSSI from 2 DL sub-bands from a single resource occasion, but measure one DL sub-band on each CSI-RS resource occasion in TDM manner. So, we prefer to define the minimum requirements as option 2 by doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

Proposal 1: For CSI-RS based L1 measurement in SBFD symbols with DUD case, define the minimum requirements by doubling existing measurement period, assuming UE cannot measure both DL sub-bands simultaneously.

Conclusions

In this paper we provided our views on RRM impact for SBFD operation.

Proposal 1: For CSI-RS based L1 measurement in SBFD symbols with DUD case, define the minimum requirements by doubling existing measurement period, assuming UE cannot measure both DL sub-bands simultaneously.

Reference

- R4-2502699, WF on RRM requirements for NR_duplex_evo