3GPP TSG-RAN WG4 Meeting #114bis	R4-2503503

Wuhan, China, April 7 – 11, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	FR2-1 SSB based L3 measurement delay reduction with CSSF optimization

Agenda item:	7.17.2.2

Document for:	Discussion

Introduction

The discussion about RRM phase 5 enhancements for L3 measurement delay reduction started in the RAN4#110bis meeting. At RAN4#114 meeting, the optimized CSSF solutions were mostly concluded in [1]. In this contribution we will continue with the remaining issues and discuss the applicability of these solutions in FR1 scenarios.

Solutions for CSSF optimization

In last meeting, the tentative solution 1 and 3 were adopted for CSSF optimization. There are still some minor open issues on 3 searchers solution.

Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19.

Regarding to the scenario, we believe the 3-searcher solution can also be applied to NR-DC. Since all the cells in MCG and SCG are sharing the overall 2 searchers, the introduction of additional one searcher can improve the measurement delay on PSCC which is considered more useful than the SCCs measurement. Hence 3-searcher solution shall also be supported in NR-DC scenarios i.e. FR1 NR-DC and FR1+FR2 NR-DC scenarios.

- 3-searcher solution shall also be supported in NR-DC scenarios i.e. FR1 NR-DC and FR1+FR2 NR-DC scenarios.

Another issue is whether to introduce a network indication to use the 3rd searcher for a specific SCC. In our view, this provides flexible control of fast measurement on certain SCC which may help network request a fast measurement report. On top of the default sharing principle for the 3rd searcher, the support of the network indication can be optional and based on UE capability.

- Network may indicate to use the 3rd searcher for a specific SCC if UE supports the capability.

CSSF optimization in FR1 scenarios

At RAN4#113 meeting, it was agreed to further discuss if the CSSF optimization solutions can be extended to FR1 after it is concluded in FR2. In the discussion of 3 searchers, the scenarios have been extended to FR1. Herein we are discussing the applicability of tentative solution 1 in FR1 scenarios.

RAN4#113 meeting: Issue 2-7: Other WID scope discussion Agreement: - after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable

In Tentative solution 1, UE is allowed to measure one serving cell per FR2 band based on network indication. In the discussion of SCell activation, we have been assuming the same timing and channel status among contiguous carriers in one FR1 band. Hence we believe the same principle of Tentative solution 1 can be applied at least to FR1 intra-band contiguous CA. To allow flexible control on the fast measurement, network may indicate to enable or disable this CSSF enhancement for FR1 and FR2 separately.

- Extend Tentative solution 1 to at least FR1 intra-band contiguous CA.

- Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately.

In existing specifications, UE is already allowed to skip the neighbor cell measurement on some SCCs in one FR2 band as specified in TS 9.2.3.2. But this does not apply to the measurement on FR1 i.e. UE is required to measure both serving and neighbor cells in one FR1 carrier in the same way. When Tentative solution 1 is enabled for FR1, UE only needs to measure one serving CC per FR1 contiguous band which shall apply to both serving and neighbor cells. That is, UE is allowed to skip both the serving and neighbor cell measurement on the other CCs on the band for FR1 intra-band contiguous CA.

Observation 1: In existing specifications, UE is not allowed to skip the neighbor cell measurement on some SCCs in one FR1 band.

- When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA.

Conclusion

The following observations and proposals were made in this paper:

- 3-searcher solution shall also be supported in NR-DC scenarios i.e. FR1 NR-DC and FR1+FR2 NR-DC scenarios.

- Network may indicate to use the 3rd searcher for a specific SCC if UE supports the capability.

- Extend Tentative solution 1 to at least FR1 intra-band contiguous CA.

- Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately.

Observation 1: In existing specifications, UE is not allowed to skip the neighbor cell measurement on some SCCs in one FR1 band.

- When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA.

- References

- R4-2502592, Way Forward for NR_RRM_Ph5_Part1, Apple