# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503543

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.18.2

Source:	Apple

Title:	On UE demodulation performance requirements with 8RX

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#114 the UE demodulation enhancements in Rel-19 were discussed in detail and way forward [1] was agreed. In this contribution we share our views on UE demodulation requirements with 8RX.

# 2. Discussion

## General

The following agreements were made on general aspects:

Issue 1-1-1: Definition of MMSE-IRC receiver capability for 8Rx UE Agreement: - If agreed to change the receiver definition, it also needs to be changed in Rel-18 maintenance. - If receiver definition is agreed to be changed, an LS will be sent to RAN2 for updating TS38.306. Issue 1-1-2: 8Rx UE MMSE-IRC receiver coverage Agreement: - Criteria for decision if one or two sets of PDSCH requirements are to be introduced: - Secure the span is <=2.5 dB for each receiver type by removal of outliers - If difference in the average of each receiver type for a specific configuration is > [TBD:limit] then define two sets of requirements for the specific configuration.

On open issues related to general aspects, we provide our view below –

Issue 1-1-2: 8Rx UE MMSE-IRC receiver coverage

- Further discuss the actual value for the [limit].

  - Proposals:

    - Option 1:
Set [limit] to 0.5dB

    - Option 2:
Set [limit] to 1.0dB

    - Option 3:
Set [limit] to 1.2dB

    - Other options not precluded.

Based on the simulation results collection [2] and the agreed test cases for defining requirements [1] in RAN4#114, all the test cases meet all the options for limit discussed in the last meeting. We propose to capture that limit of 1.2 dB is used to define the limit to decide of 2 sets of requirements are defined.

For the test cases selected to define requirements, all options for limit value meet the criteria to define 2 sets of requirements.

- Finalize limit as 1.2 dB to decide if 2 sets of requirements are introduced.

Issue 1-1-1: Definition of MMSE-IRC receiver capability for 8Rx UE

Proposals:

- Option 1: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 2: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 3: 
Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE declares supporting SU-MIMO 8Rx receiver capability

Based on the observation in previous issue, it is inevitable that 2 sets of requirements will need to be defined for simplified and baseline receiver respectively. Hence, the receiver definition will need to be updated.

For 8RX ICI and MU-MIMO, we would need to define 2 sets of requirements.

- Introduce 2 sets of requirements for simplified and baseline receiver for PDSCH demodulation for ICI and MU-MIMO

- Update the receiver definition introduced in Rel-18 to be applicable to SU-MIMO and MU-MIMO with MMSE-IRC receiver.

For the wording update, we propose the following:

- Baseline 8Rx Receiver: 8Rx MMSE-IRC receiver for MIMO transmission with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx MMSE-IRC receiver for MIMO transmission with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx MMSE-IRC receiver for MIMO transmission with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx MMSE-IRC receiver for MIMO transmission with support of only up to 4 layers with two joint 4Rx MIMO detectors

## PDSCH requirements with intra cell inter user interference

For PDSCH requirements with intra-cell inter-user interference with 8 RX the following agreements were made:

Issue 1-3-1: MCS for Rank 2+2 Agreement: - For Rank 2+2, use MCS17 for requirement definition.

Simulation parameters for initial evaluation were agreed in [1] and our results are summarized in [3]. Rank 4+4 is feasible in simulation with 8RX baseline receiver, but we don’t think 4+4 rank combination is very practical in real deployments. Hence, we propose to only define requirements with rank combination 2+2 with 8RX.

- Configuration with rank 4+4 although feasible with 8RX baseline receiver is not a practical scenario in deployment.

- Do not define requirements with rank combination 4+4.

## Wideband CQI reporting requirements with inter-cell interference

On CQI reporting requirements in ICI the following agreements were made in [1]:

Issue 1-4-2: Rank for serving cell Agreement: - If simulation results show both rank 1 and rank 2 to be feasible for requirement definition, then Rank 2 will be selected. Issue 1-4-3: CSI-IM and NZP CSI-RS configurations Agreement: - Use the following CSI-RS configurations for FDD and TDD CQI requirements simulations:

Issue 1-4-1: CQI calculation algorithm

Proposals:

- Option 1: 
RAN4 to define CQI requirements for two receiver types:
- Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing. 
- Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing.

- Option 2: 
If the difference in gamma between baseline and simplified receiver is 0.1 or lower only one set of requirements shall be defined. Further discuss when more simulation results are available.

To determine if one or 2 sets of requirements are introduced for CQI reporting in ICI, we propose the following method:

- Choose the SINR for defining requirements based on TP gain with baseline receiver.

- At the chosen SINR, if the gamma value difference is > 0.1, then define 2 sets of requirements for simplified and baseline receiver.

- Adopt the following methodology and criteria to decide CQI reporting requirements:
- Choose the SINR for defining requirements based on TP gain with baseline receiver. 
- At the chosen SINR, if the gamma value difference is > 0.1, then define 2 sets of requirements for simplified and baseline receiver

## Applicability rule and release independence

On applicability rule and release independence the following agreements were made in [1]:

Issue 1-5-2: Test applicability for FDD/TDD Agreement: - Introduce applicability rules to limit the overall number of test cases so if UE supporting both TDD and FDD with 8Rx, UE will pass test cases with 2 interference cells (HomNet scenario) with FDD mode, and pass test cases with 1 interference cell (HetNet scenario) with TDD mode.

On the open issues –

Issue 1-5-1: Test applicability to skip 2/4Rx requirements

Proposals:

- Option 1: 
For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

- Option 2: 
For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.
TBD: Clarification of what “corresponding 8Rx requirements” covers.

We support option 1 above to reduce test burden.

- For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, irrespective of test configurations for 8RX and 2RX/4RX.

Issue 1-5-3: Release independency

Proposals:

- Option 1: 
Discuss release independence requirements at a later stage in the WI.

- Option 2: 
The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

- Option 3: 
The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

- Option 4: 
For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18.

- Option 5: 
The requirement should at least be released independent from Rel-18 if no new signalling or UE capability introduced.

With the testcases and parameters almost finalized for requirements, it is appropriate to talk about release independence. Given that 8RX UE was introduced in Rel-18, and requirements for ICI and MU-MIMO with 8RX don’t need additional UE capability signalling, the requirements for ICI and MU-MIMO with 8RX can be release independent since Rel-18.

- The requirements for 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario can be release independent since Rel-18.

# 3. Conclusion

In this paper, we provide our views on UE demodulation requirements with 8RX. Our observations and proposals are captured below:

General

- For the test cases selected to define requirements, all options for limit value meet the criteria to define 2 sets of requirements.

- Finalize limit as 1.2 dB to decide if 2 sets of requirements are introduced.

For 8RX ICI and MU-MIMO, we would need to define 2 sets of requirements.

- Introduce 2 sets of requirements for simplified and baseline receiver for PDSCH demodulation for ICI and MU-MIMO

- Update the receiver definition introduced in Rel-18 to be applicable to SU-MIMO and MU-MIMO with MMSE-IRC receiver.

- Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx MMSE-IRC receiver for MIMO transmission with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx MMSE-IRC receiver for MIMO transmission with support of only up to 4 layers with two joint 4Rx MIMO detectors

PDSCH requirements with intra cell inter user interference

- Configuration with rank 4+4 although feasible with 8RX baseline receiver is not a practical scenario in deployment.

- Do not define requirements with rank combination 4+4.

CQI reporting requirements with inter cell interference

- Adopt the following methodology and criteria to decide CQI reporting requirements:
- Choose the SINR for defining requirements based on TP gain with baseline receiver. 
- At the chosen SINR, if the gamma value difference is > 0.1, then define 2 sets of requirements for simplified and baseline receiver

Applicability rule and release independence

- For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, irrespective of test configurations for 8RX and 2RX/4RX.

- The requirements for 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario can be release independent since Rel-18.

# Reference

- R4-2502275, “Way Forward for [114][324] NR_demod_Ph5_Part2_UE”, Nokia.

- R4-2500267, “Simulation result summary for ICI and MU-MIMO with 8RX”, Apple

- R4-2503544, “Simulation results for ICI and MU-MIMO with 8RX”, Apple.