3GPP TSG-RAN WG4 Meeting #114bis		R4-2503668

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:		7.23.3.2

Source:		Samsung

Title:	Views on RRM requirements for Rel-19 SBFD operation

Document for:		Discussion

- Introduction

In last RAN4#114 meetings, the WF was approved in [1]. In this contribution, we continue to discuss the open issues of RRM requirements for SBFD operation.

- Discussion

In last RAN4#114 meeting, the potential issues for further discussion are captured in WF [1] as below:

### Sub-topic 2-1: RRM impacts of SBFD operation #### Issue 2-1-1a: CSI-RS L1 measurement – collision with UL #### Issue 2-1-1c: CSI-RS L1 measurement – different symbol types #### Issue 2-1-3: Requirements for scheduling restriction #### Issue 2-1-4: Requirements for RACH requirements #### Issue 2-1-5: Requirements for UL PC and or spatial relation update #### Issue 2-1-6: Requirements for MG and BWP switch Issue 2-1-7: Requirements for UL resource muting

CSI-RS L1 measurement – collision with UL

In RAN4#113 meeting, RAN4 sent an LS to RAN1 in [2].

In RAN1#120 meeting, a reply LS from RAN1 has been approved and sent to RAN4 in [3].

According to the answer for Issue#1 in the reply LS:

For collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) that involves CSI-RS resources used for L1-RSRP/SINR and RLM/BFD/CBD measurements, RAN1 would like to provide the following response: The intention of the mentioned RAN1 agreement is to follow the existing prioritization rules between semi-static DL channels/signals and dynamically scheduled UL transmission including semi-statically configured CSI-RS resources for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. The existing prioritization rules for serving-cell for collision Case 2 are as follows: - TS 38.213, Clause 11.1 “Slot configuration” “For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.” - TS 38.213, Clause 11.1.1 “UE procedure for determining slot format” “If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot.”.

This response is aligned with our understanding for RAN1 agreements and current spec in 38.213 v15. In a word, it will reuse the collision handling principle in single carrier unpaired spectrum, which is dynamic scheduled UL always is prioritized over the semi-persistent CSI-RS reception.

Proposal 1: For CSI-RS collision with dynamic UL transmission: dynamic UL transmission shall be prioritized over than L1 CSI-RS based on measurement.

CSI-RS L1 measurement – puncturing in frequency domain

In last RAN4#114 meeting, the agreement in [1] is listed as:

#### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - Option 3: Consider the UE measurement capability - FFS whether and how to account for the UE capability on CSI processing time. - Further discuss the accuracy requirements for the following cases - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

In legacy L1-RSRP based on CSI-RS, the minimum bandwidth of CSI-RS is 48PRBs. For RLM/BFR based on CSI-RS, the minimum bandwidth is 24PRBs.

The first issue is whether to reuse or extend the measurement period for DUD case.

According to RAN1 previous agreement/WA, the UE support non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. In this case, the SBFD-aware UE can support DL reception across two DL subbands. From UE implementation perspective, it is possible for receiving the DL signal from two subbands at the same time and punctured unused PRBs with less complexity. After that, the measurement of frequency punctured CSI-RS is quite similar as regular CSI-RS measurement.

Proposal 2: For measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case, reuse the existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands.

For the accuracy of this measurement in DUD case, according to agreements in RF session in RAN4#113 meeting:

##### Issue 1-1-4: Specification implementation of BS channel bandwidth support for SBFD Agreement: - The following note is introduced in Table 5.3.2-1 and Table 5.3.5-1 in TS 38.104: - In the bands applicable to SBFD operation, the channel bandwidth for SBFD operation shall be no less than 20MHz for band n39 and no less than 40MHz otherwise. ##### Issue 1-3-2: DL/UL subband configurations support for SBFD Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB

In the WF, there are 3 cases:

    - Case 1: X1 >= X3 and X2 >= X3

    - Case 2: X1 >= X3 and X2 < X3

    - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

For Case 1, it is obvious that the existing accuracy requirements can be applied.

For Case 2, according to the RF agreement, the minimum X2 is only X3-1. In such case, we suggest to reuse the existing accuracy requirements.

For Case 3, X1 and X2 are smaller than X3. In addition, X1 = X2+1 or X1= X2-1 or X1=X2. According to the agreed 20MHz channel bandwidth for band n39, to cover the scenarios of SCS = 30kHz/60kHz, 48 PRBs cannot be applied. In the RSRP measurement, if the total PRBs are the same and with no other condition changed, the simulation results should be quite similar in AWGN channel. In fading channel, there may be little difference. Thus, for case 3, the accuracy can be specified with the condition X4 = X3/2 for existing L1-RSRP or RLM/BFD/CBD requirements. The accuracy can be changed depends on simulation results difference.

Proposal 3: For accuracy requirements, for CSI-RS based L1 measurement in SBFD symbols with DUD case

- Case 1 and Case 2: reuse the existing accuracy requirements

- Case 3: The accuracy can be specified with the condition X4 = X3/2 for existing L1-RSRP or RLM/BFD/CBD requirements. The accuracy can be changed depends on simulation results difference.

CSI-RS L1 measurement – different symbol types

In [3], RAN1 reply LS was approved in RAN1#120 meeting.

Issue#2: Behaviour for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD measurements For resource configuration and/or reporting configuration for CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, RAN1 would like to provide the following response: - According to the following RAN1 agreements, Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. Agreement (RAN1#120) The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP. Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols. - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability. Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.. - The following RAN1 agreements for CSI reporting are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting. Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement (RAN1#120) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig..

For CSI-RS based L1-RSRP/SINR measurement/reporting, it is clear that there is no chance for the single L1-RSRP reporting by different symbol types such as mixed SBFD symbols and non-SBFD symbols.

Proposal 4: For L1-RSRP/SINR, the measurement and reporting are based on one type of SBFD symbols and non-SBFD symbols. No need to mixed the measurement in different types of symbols.

However, for CSI-RS based RLM/BFD/CBD measurement, according to RAN1 reply LS, these agreements are not applied for RLM/BFD/CBD for Configuration 1 and Configuration 2. By our understanding, the RLM/BFD/CBD means legacy configuration. There is no additional enhancement for SBFD for RLM/BFD/CBD. Thus, it means when the RadioLinkMonitoringRS (use RLM as example) is configured, it can be configured by NRLM RLM-RS resources across non-SBFD symbols and SBFD symbols.

Observation #1: For RLM/BFD/CBD, the configuration and measurement can across non-SBFD symbols and SBFD symbols.

And the power difference between SBFD and non-SBFD slot/symbol is due to different antenna configurations of SBFD BS. Such different implementations were documented in the SBFD TR 38.858[5] as in Appendix.

Observation #2: From TR 38.858 [5] and RF session WF [6], the SBFD BS antenna will have different number of antenna elements on its SBFD and non-SBFD symbols/slots. And due to the power limitation per antenna element from implementation, the actual DL output power from SBFD BS on its SBFD and non-SBFD symbols/slots can be different.

In the following figures, we try to illustrate the differences in two examples:

Figure 1. Example of SBFD BS utilize half of its panel for DL on SBFD symbol

In above Figure 1 exmaple, the SBFD BS will utilize same number of antenna element in its non-SBFD slot/symbols, same as a legacy TDD BS for its DL signals, including reference signals. But on SBFD symbol/slots, the SBFD BS can only utilize half numberf of its antenna elements.

The actual DL power level transmitted from the BS on the SBFD symbol is lower than the power transmitted on the non-SBFD symbols. The differences, if we are not considering any implementation or power boosting, will include 3dB loss from less antenna array in its beam forming gain and also another 3dB from the input power as the BS power is supplied per element or per PA before each or several elements in a sub-group.

Figure 2. Example of SBFD BS utilize a separate panel for SBFD operation

Another example in above figure 2 gives an implementation that the SBFD BS will utilize an additional antenna panel with same elements, and to use that panel for SBFD UL or DL on SBFD symbols/slots. With such implementation, the DL power can be kept same on the SBFD symbols/slots as it on non-SBFD symbols/slots.

Observation #3: For SBFD BS, which utilizes half of its antenna panel size for DL and UL on SBFD symbol, the DL output power from SBFD BS will have 3dB loss from antenna array gain and 3dB from input power.

For SBFD BS, which utilizes an additional antenna panel on SBFD symbols/slots, the DL output power level of this SBFD BS can be similar to its non-SBFD symbol/slots.

As mentioned in observstion 1, for RLM/BFD/CBD measurement, the power difference of RLM-RS (as example) due to different SBFD BS can cause different link quality of the measurement. It’s hard to say that UE always expect the link quality in the non-SBFD has better link quality than that in SBFD symbols without such power difference information. When considering the RLM/BFD resources across non-SBFD symbols and SBFD symbols, it is unfair and cannot reflect to the real quality for IS/OOS.

Proposal #5: For RLM/BFD/CBD measurement, measurement can across non-SBFD symbols and SBFD symbols, the power difference between SBFD symbols and non-SBFD symbols is needed.

CSI-RS based L3 measurement

In last RAN4#114 meeting, the agreement in [1] is listed as:

- Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

In last RAN4#114 meeting, it is agreed for DU case to use as legacy requirements. for DUD case, it is still FFS.

From the L3 measurement, there is no big difference between DU case and DUD case. For both cases NW deployments are responsible to ensure all the CSI-RS resource configuration. From the NW perspective, they are quite similar behaviors.

Proposal 6: For CSI-RS based L3 measurement in DUD case, use the same principle as agreed in last meeting for DU case.

Requirements for scheduling restriction

In RAN4#113 meeting WF[3], the scheduling restriction is shown as:

#### Issue 1-2-4: Requirements for scheduling restriction - Agreements (from Wed AH session) - For DL/UL transition, e.g. after SSB measurement, further check RAN1 agreements and potential impact to RAN4 scheduling restriction. - Further discuss the impact to inter-frequency SSB measurement without gap case.

For this issue, the measurement is inter-frequency SSB measurement without gap. According to RAN1 agreement, The SSB of serving cell is always in the DL subbands.

RAN1#118bis Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

The SSB of neighbor cell can be outside of the DL subbands which means fully overlapped or partially overlapped with UL subbands.

If the measurement is for the SSB with fully/partially overlapped with UL subbands, SSB is always prioritized over UL transmission.

Proposal 7: No additional scheduling restriction is needed for inter-frequency SSB measurement without gap.

- Conclusion

In this contribution, we provide our proposals for SBFD operation:

Proposal 1: For CSI-RS collision with dynamic UL transmission: dynamic UL transmission shall be prioritized over than L1 CSI-RS based on measurement.

Proposal 2: For measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case, reuse the existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands.

Proposal 3: For accuracy requirements, for CSI-RS based L1 measurement in SBFD symbols with DUD case

- Case 1 and Case 2: reuse the existing accuracy requirements

- Case 3: The accuracy can be specified with the condition X4 = X3/2 for existing L1-RSRP or RLM/BFD/CBD requirements. The accuracy can be changed depends on simulation results difference.

Proposal 4: For L1-RSRP/SINR, the measurement and reporting are based on one type of SBFD symbols and non-SBFD symbols. No need to mixed the measurement in different types of symbols.

Observation #1: For RLM/BFD/CBD, the configuration and measurement can across non-SBFD symbols and SBFD symbols.

Observation #2: From TR 38.858 [5] and RF session WF [6], the SBFD BS antenna will have different number of antenna elements on its SBFD and non-SBFD symbols/slots. And due to the power limitation per antenna element from implementation, the actual DL output power from SBFD BS on its SBFD and non-SBFD symbols/slots can be different.

Observation #3: For SBFD BS, which utilizes half of its antenna panel size for DL and UL on SBFD symbol, the DL output power from SBFD BS will have 3dB loss from antenna array gain and 3dB from input power.

For SBFD BS, which utilizes an additional antenna panel on SBFD symbols/slots, the DL output power level of this SBFD BS can be similar to its non-SBFD symbol/slots.

Proposal #5: For RLM/BFD/CBD measurement, measurement can across non-SBFD symbols and SBFD symbols, the power difference between SBFD symbols and non-SBFD symbols is needed.

Proposal 6: For CSI-RS based L3 measurement in DUD case, use the same principle as agreed in last meeting for DU case.

Proposal 7: No additional scheduling restriction is needed for inter-frequency SSB measurement without gap.

- Reference

[1] R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei

[2] R4-2420165, LS on CSI-RS measurement with SBFD operation, RAN4

[3] R1-2501560, Reply LS on CSI-RS measurement with SBFD operation, RAN1

[4] R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

[5] 38.858

[6] R4-2409958, Way Forward for [110bis][313] NR_duplex_evo, Samsung

- Appendix

38.858

For evaluation and comparison between SBFD and legacy TDD, three options can be used. - SBFD antenna configuration option-1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-3: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is half of the total number of TxRUs of the antenna array for legacy TDD. These options are further clarified with examples in the following: As shown in Figure A.5-2 (a), for legacy TDD with shared-Tx/Rx antenna array, assume the antenna configuration is M,N,P,Mg,Ng;Mp,Np. The total number of TxRUs is K=PMpNpMgNg, and the total number of antenna elements is L=PMNMgNg. For SBFD antenna configuration option-1, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg/2,Ng. The total number of TXRUs is K=PMpNpMgNg (same as legacy TDD), and the total number of antenna elements is L=PMNMgNg(same as legacy TDD). One method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols is illustrated as below. Other methods are not precluded and can be reported by companies. - Method 1 (see Figure A.5-2 (b)): - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Tx chains in TxRU group#2. - In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2. - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2. For SBFD antenna configuration option-2, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg,Ng. The total number of TXRUs is K=PMpNpMgNg (same as legacy TDD), and the total number of antenna elements is 2L=2PMNMgNg(two times of that for legacy TDD). Two methods on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies. - Method 2-1 (see Figure A.5-2 (c)): - In DL slots, L antenna elements on panel group#1 are connected to K Tx chains. - In UL slots, L antenna elements on panel group#2 are connected to K Rx chains. - In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains. - Method 2-2 (see Figure A.5-2 (d)): - In DL slots, L antenna elements on panel group#1 are connected to K Tx chains. - In UL slots, L antenna elements on panel group#1 are connected to K Rx chains. - In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains. For SBFD antenna configuration option-3, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg/2,Ng. The total number of TXRUs is K/2=PMpNpMgNg/2 (half of that for legacy TDD), and the total number of antenna elements is L=PMNMgNg(same as legacy TDD). The method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies. - Method 3-1 (see Figure A.5-2 (e)): - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains. - In UL slots, L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains. - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains. - Method 3-2 (see Figure A.5-2 (f)): - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1. - In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1. - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#1. (a) Legacy TDD with shared-Tx/Rx antenna array (b) SBFD antenna configuration Option-1 (Method 1) (c) SBFD antenna configuration Option-2 (Method 2-1) (d) SBFD antenna configuration Option-2 (Method 2-2) (e) SBFD antenna configuration Option-3 (Method 3-1) (f) SBFD antenna configuration Option-3 (Method 3-2) Figure A.5-2: Illustration of antenna configuration of legacy TDD and SBFD