3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503885

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussion on conditional Intra-CU LTM

Source: 	Huawei, HiSilicon

Agenda item:	7.27.2.3

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

This contribution provides analysis on intra-CU LTM from RAN4 RRM requirement perspective.

Discussion

Regarding the third objective conditional intra-CU LTM, according to the latest revised WID, both conditional LTM and subsequent conditional LTM are to be supported. The analysis on the RRM requirements for conditional LTM and subsequent conditional LTM are provided in the followings.

General aspects on conditional LTM

Issue 1-1-1: The applicable scenarios for conditional LTM < Agreement > - For the case which CLTM is triggered by L3 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included. - For the case which CLTM is triggered by L1 measurement, FFS deactivated SCell and the cells with same frequency as deactivated Scell are included.

As per R18 RAN2 signalling framework, UE can execute L1-RSRP measurement on the deactivated SCell/neighbour cell on the same frequency when the candidate cell is configured as a LTM candidate PCell. However as far there are no RRM requirements in RAN4. From defining requirements perspective, some aspects need to be considered. On the one hand, as the SCell is deactivated, to save power, UE may normally power off the SCell and turn on the RF chain when measurement is to be performed. In this case interruptions on activated serving cells are expected. Moreover whether to consider extended L1 measurement period for deactivated SCell (like L3 RSRP measurement) may also need to be discussed in RAN4. On the other hand, since the cell on deactivated SCC is configured as candidate PCell, UE can always open the RF chain. In other words, UE regards the deactivated SCell as an activated PCell. In this case, there may be no interruptions due to measurement on the target cell.

We are open to whether to define L1 measurement requirements on deactivated SCell and the cells with same frequency as deactivated SCell. If companies would like to define the corresponding requirements, the above aspects are to be discussed.

Proposal 1: If companies would like to define L1 measurement requirements on deactivated SCell and the cells with same frequency as deactivated SCell, the following aspects need to be considered:

- Whether interruptions exist on activated serving cells

- Whether to extended L1 measurement period for deactivated SCC.

Conditional LTM

Conditional LTM delay

Legacy conditional handover delay is defined as DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. R18 LTM cell switch is defined as DLTM = Tcmd  + TLTM-interrupt ( where TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms). In our understanding, conditional LTM delay can take legacy CHO delay as a reference.

Proposal 2: Conditional LTM delay can take legacy CHO delay as a starting point.

The starting point of legacy CHO delay is from UE receives a RRC message implying conditional handover. After UE successfully decodes the RRC signalling, UE waits for the time instance the conditional conditions are met and then performs handover to the target cell. For C-LTM, LTM cell switch related condition configurations are also signalled by RRC. In last meeting, for the issue of starting point of C-LTM delay, one bracket is remained, because some company spotted that RAN2 is discussing when UE starts to execute condition evaluation, e.g., upon it receives the conditional LTM command or an activation indication. As per the latest RAN2 agreement: the UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. Therefore it is not necessary to add the sentence in bracket.

Issue 1-3-2: The starting point of conditional LTM delay < Agreement > ad hoc - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM, [under assumption that UE will start evaluating the CLTM event after reception of the RRC for CLTM]. - Revisit if RAN2 has more progress.

RAN2 agreement The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

Proposal 3: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

In last RAN4 meeting it is agreed that the ending point of conditional LTM is the time the UE transmits the first UL transmission on the target cell [2]. So the ending point issue is closed.

In legacy CHO delay, the component TEvent_DU is the delay uncertainty from UE decodes a CHO command to the moment when the condition is met at measurement reference point (i.e., real channel quality satisfies the condition of CHO).

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover

As we know, RAN2 agreed to support early TCI state activation/deactivation and UL sync on candidate cells during conditional LTM procedure. In last RAN2 meeting, some progresses are made accordingly.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

Regarding the sequence of early DL (e.g., TCI state activation), UL sync (PDCCH ordered based RACH) and execution condition evaluation, we think early DL and UL sync procedures on candidate cells shall be performed before UE evaluating the execution conditions. Taking PDCCH order RACH as an example, in Rel-18 network triggered LTM, both PDCCH order PRACH and LTM cell switch command are sent from gNB, so gNB can totally control the order of the two procedures. However, in conditional LTM, the gNB does not know when the condition is met at UE side. If gNB sends a PDCCH order very early, the TA may be out of date when the UE triggers cell switching later. Conversely, there is also a case that when the conditions are met in UE side, UE starts to switch to the target cell, however at the same time, network is triggering PDCCH ordered RACH, then UE failed to receive the PDCCH order. To our knowledge, RAN2 think it is left to gNB implementation to decide when to trigger PDCCH order PRACH, as gNB has some information of UE measurement reports.  It means that if gNB failed to trigger the PDCCH order PRACH in time (e.g.,  UE failed to receive the command and has already switched to other cell), UE will still need to perform RACH based cell switch.

RAN2 agreements

When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

Similar, early TCI state activation before evaluating execution conditions is the similar status as PDCCH ordered RACH.

As per the existing definition of delay uncertainty of TEvent_DU (from decoding CHO command to a condition is met),  we think TEvent_DU definition can cover both cases in C-LTM: 1) UE completed early TCI state activation and/or early UL synchronization before the execution condition is met, 2) neither DL sync nor UL sync is successfully performed before condition is met.

Proposal 4: TEvent_DU in CHO can be reused in conditional LTM delay, as it can cover both cases:

1) UE completed early TCI state activation and/or early UL synchronization before the execution condition is met

2) Neither early TCI state activation nor early UL synchronization is successfully performed before the execution condition is met.

In legacy conditional handover, Tmeasure is the time duration when the condition event is really met at UE side (i.e., UE realize the condition of CHO is met).

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

As both L1 measurement and L3 measurement can be used to trigger conditions of C-LTM, Tmeas can be the L3 measurement period or L1 measurement period. Whether to support triggering condition based on  CSI-RS L3 measurement depends on RAN2 progress.

Proposal 5: In C-LTM delay, Tmeas is the L1 measurement period

- If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM.

- If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy.

- If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19.

- FFS on CSI-RS based L3-RSRP measurement.

In last meeting, one controversial issue is which components TCLTM-interrupt shall include.

Issue 1-3-3: The components of conditional LTM delay < Agreement > FFS the following options ad hoc - Option 1: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-processing + TLTM-IU ms - Option 2: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU ms - Option 3: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

Regarding TCHO_execution, in legacy CHO delay, TCHO_execution is the UE execution preparation time for conditional handover. This time duration originally called TRRC_2 for RRC processing time of the target cell. TCHO_execution is equivalent the term TLTM-RRC-processing in LTM delay (which is the time for ASN.1 decoding and validity/compliance check). In our understanding, TCHO_execution can be reused in conditional LTM. TCHO_execution can be up to 10ms.

Proposal 6: TCHO_execution (i.e., TLTM-RRC-processing) can be reused in conditional LTM.

In legacy CHO delay, Tinterrupt comprises Tprocessing + TIU + T∆ + Tmargin, and TCHO_execution is not be included. The reason is that UE is able to perform data reception/transmission when performing ASN.1 decoding and validity check of target cell. But in R18 LTM delay, TLTM-RRC-processing is regarded as one component of interruption. The reason is that serving cell probably won’t schedule the UE after sending cell switch command. For conditional LTM procedure, as network would not indicate cell switch command and has no information when UE is to switch to another cell. network can keep scheduling the UE during TCHO_execution. From UE side, the ASN.1 decoding and validity check of target cell has no impact on serving cell service as well. Therefore in C-LTM delay, TCHO_execution (i.e., TLTM-RRC-processing) is not be part of TCLTM_interrupt.

Proposal 7: TCHO_execution (i.e., TLTM-RRC-processing) is not be part of TCLTM_interrupt.

In C-LTM, upon condition is met, after UE finishes preparation operations and leaves the source cell, the steps which are defined in R18 LTM cell switch delay, i.e., TLTM-processing, Tfirst-RS, TRS-proc , TLTM-IU, shall be followed.

In R18 LTM cell switch delay:

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the uncertain time in transmit the first UL transmission.

The above components would result in interruptions on serving cells.

Proposal 8: After TCHO_execution, the component TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused for R19 C-LTM.

Proposal 9: TCLTM_interrupt includes TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU

In summary, the C-LTM delay can be depicted as

DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TCLTM_interrupt

Where TCLTM_interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

Proposal 10: C-LTM delay can be depicted as

DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TCLTM_interrupt

where TCLTM_interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

Subsequent conditional LTM

The requirements of subsequent CPA and subsequent CPC have been specified in R18. The delay for subsequent CPA/CPC is alike CHO delay in some extent. The difference is the starting point. In subsequent CPA/CPAC the starting point is from the time UE transmits RRC Reconfigurationcomplete message for the previous PSCell addition or change. In last meeting, the start point has reached conclusion for subsequent conditional LTM

Issue 1-4-2: The starting point of Subsequent conditional LTM delay < Agreement > ad hoc - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM, [under assumption that UE will continue evaluating the CLTM event after the completion of previous CLTM]. - Revisit if RAN2 has more progress.

Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Proposal 11: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Conclusion

The contribution provides analysis on intra-CU LTM from RAN4 RRM requirement perspective. The following proposals are proposed:

Proposal 1: If companies would like to define L1 measurement requirements on deactivated SCell and the cells with same frequency as deactivated SCell, the following aspects need to be considered:

- Whether interruptions exist on activated serving cells

- Whether to extended L1 measurement period for deactivated SCC.

Proposal 2: Conditional LTM delay can take legacy CHO delay as a starting point.

Proposal 3: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

Proposal 4: TEvent_DU in CHO can be reused in conditional LTM delay, as it can cover both cases:

1) UE completed early TCI state activation and/or early UL synchronization before the execution condition is met

2) Neither early TCI state activation nor early UL synchronization is successfully performed before the execution condition is met.

Proposal 5: In C-LTM delay, Tmeas is the L1 measurement period

- If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM.

- If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy.

- If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19.

- FFS on CSI-RS based L3-RSRP measurement.

Proposal 6: TCHO_execution (i.e., TLTM-RRC-processing) can be reused in conditional LTM.

Proposal 7: TCHO_execution (i.e., TLTM-RRC-processing) is not be part of TCLTM_interrupt.

Proposal 8: After TCHO_execution, the component TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused for R19 C-LTM.

Proposal 9: TCLTM_interrupt includes TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU

Proposal 10: C-LTM delay can be depicted as

DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TCLTM_interrupt

where TCLTM_interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

Proposal 11: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2502602, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom