3GPP TSG-RAN WG4 Meeting #114bis	R4-2504102

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.5.2

Source:            	Google

Title:                	Discussion on methods for reducing the number of UE Rx chains for fragmented carriers

Document for:	Approval

# Introduction

In RAN#106, the revised SID for “Study on NR FR1 DL Fragmented Carriers” was approved as one of Release 19 work items [1]. The objective in the SID is to identify methods for reducing the number of UE Rx chains for two CCs DL NCCA in the single band with frequency span smaller than 100MHz, which is shown as follows. In this contribution, we will focus on the non-RF requirement related yellow part for the fragment carriers study.

### 4.1 Objective of SI or Core part WI or Testing part WI The objective of this study is as follows: - Identify methods for reducing the number of UE Rx RF chains (e.g. from separate Rx RF chains per CC to one Rx RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering: - Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4]; - Other requirements for study purpose are not precluded, if identified - The ability to semi-statically switch hardware resources (i.e. Rx RF chains) [RAN4, RAN2 – See note 2]; - Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4]; - Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4] ; - Impacts on DL performance [RAN4] ; - Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2]. NOTE 1: No RAN1 impact is foreseen NOTE 2: RAN2 work, if necessary, will be triggered by RAN4 LS NOTE 3: This study starts from single DL band. One Rx RF chain is not considered among inter-band DL carriers. - Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

In RAN4#114, the methods for reducing the number of UE Rx chains for fragmented carriers have been discussed and some open issues are captured in the approved WF [2]. In the WF, the UE capability issue is still open which still needs further discussion. For semi-statically switch hardware resources, it is still does not have conclusion yet. In this paper, we would like to share our view for these issues in the following.

# Open issues ### How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers Option 1: Legacy band combination reporting scheme + Rx sharing capability indication - FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”. Other options are not precluded ### Discussion on semi-statically switch hardware resources procedure and fallback considerations ### Triggering condition to enable ”One RF Rx chain” FFS: - FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network. - Triggering condition for UE working in one Rx RF chain is needed - Triggering condition for UE working in legacy Rx RF chains is needed - The switching should be not be dynamic and be under network control with UE indication of condition. ### Discussion on applicability for PCell and SCell ### Discussion on the number of Rx and DL MIMO layer configuration for the UE supporting ”one Rx RF chain” mode

# Discussion

For fragmented carrier study item, the objective is to study whether the separated Rx RF chain for each CC can be replaced with fully shared Rx RF chain for two CCs in a single DL band under the frequency span smaller than 100MHz. In the previous meeting discussion, the meaning of the shared Rx RF chain is not very clear. Probably, it can be referred to partially-shared Rx RF chain, i.e., with the same BPF/external-LNA and separated internal-LNA/mixer/LO/analog-BB LPF, or fully-shared Rx RF chain, i.e., with the same BPF/LNA/mixer/LO/analog-BB LPF. After several meeting discussion, the fully shared Rx RF is identified as the reference architecture for fragmented carriers and the terminology is replaced with “one Rx RF chain” in the approved revised WID in RAN#106, where one Rx RF chain is shown as the following figure.

In our understanding, two CC NCCA is usually implemented by the shared antenna/BPF/external-LNA, different internal-LNA/mixer/LO/analog-BB LPF as the partially shared Rx RF chain. Supporting one Rx RF chain for fragmented carriers means that the UE can save more mixer/LO/analog-BB LPF for anther inter-band or intra-band carriers. In other words, the maximum supporting CC number for high order CA can be enhanced if the UE supports fragmented carrier feature. However, depending on the different RFIC implementation, the mixer/LO/analog-BB LPF allocation rule for a configured CA combination would be different. Hence, how many one Rx chain can be enabled in a configured CA combination is usually limited by the RFIC capability.

Signalling

One objective in the SID is to discuss the method for a UE to inform the network about which CA configuration it can support with the adjusted RF requirements, i.e., the RF requirement for one Rx RF chain which is adjusted from separated Rx RF chains. In our understanding, one bit indication to support one Rx RF chain for fragmented carriers can be per-UE capability signalling, per-band capability signalling, per-BC capability signalling, per-band per-BC capability signalling and per CC per-band per-BC capability signalling.

For per-UE or per-BC capability signalling, it would mean that the UE needs to support one Rx RF chain for all supported RF bands or for a configured band combination. Based on the above discussion, it may cause less flexibility for UE RFIC implementation. Take the following band combination CA_n3(2A)-n7(2A) in TS38.101-1 as an example, it is observed that there are two bands with the fragmented carriers in this NCCA band combination. Depending on the different optimization for the mixer/LO/analog-BB LPF allocation rule, there may be a possibility that the UE may use the one Rx RF chain for NCCA CA_n3(2A) and separated Rx RF chains for NCCA CA_n7(2A) in the configured CA band combination. Hence, in our view, it seems that the per-band or per-band per-BC capability signalling is more suitable since it can offer more UE implementation flexibility.

[38.101-1] [V19.0.0]

Table 5.5A.3.1-1c: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3(2A)-n7(2A) | CA_n3A-n7A | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | CA_n7(2A)_BCS0 |  |

Table 5.5A.3.4-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (five bands)

| NR CA configuration | Uplink configuration or single uplink carrier 2 | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3(2A)-n7(2A)-n8A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n8A CA_n1A-n78A CA_n3A-n7A CA_n3A-n8A CA_n3A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

In the other way, take the high order CA band combination CA_n1A-n3(2A)-n7(2A)-n8A-n78A as another example. It is observed that there are two NCCA CA_n3(2A) and CA_n7(2A) within this CA band combination. If per-band per-BC capability signalling is indicated, the UE can clearly indicate which NCCA is supported by fragmented carriers feature. The similar signalling indication can be also applied to the above example band CA_n3(2A)-n7(2A) to determine which NCCA or both supporting fragmented carriers feature. The advantage for per-band per-BC capability signalling is that the UE can have higher flexibility to decide whether the NCCA, e.g., CA_n3(2A), should be supported by fragmented carriers feature for both example band combination CA_n3(2A)-n7(2A) and CA_n1A-n3(2A)-n7(2A)-n8A-n78A.

For per-band capability signalling, although it can save multiple signalling bits for the indicated band NCCA to support fragmented carrier feature, the per-band signalling bit should be also applied to all high order CA combination which contains the indicated band NCCA. For example, if the per-band capability bit is indicated in NR band n3, the NCCA CA_n3(2A) is supported by fragmented carrier feature in both band combination CA_n3(2A)-n7(2A) and CA_n1A-n3(2A)-n7(2A)-n8A-n78A.

For per CC per-band per-BC capability signalling, although it can be more flexible to indicate which two CCs can support fragmented carrier feature among higher order band combination, e.g., CA_n41(3A) or CA_n41(4A), the signaling overhead would be increased due to reporting all CC permutation in NCCA supporting fragmented carrier feature, e.g., CC1/CC2 or CC1/CC3 or CC2/CC3 in CA_n41(3A). Hence, in our view, the per-band per-BC capability signalling is more compromised for the UE implementation. For example, if a UE reports per-band per-BC capability signalling for one band, it means that the UE can support one RF Rx chain for all CCs permutation on that band no matter how many CCs are activated by the network. Therefore, if the capability signalling for fragmented carriers feature is agreed to be introduced in RAN4, it is preferred to have one-bit indication per-band per-BC capability signaling for CA configuration supported fragmented carriers feature within a CA band combination.

Proposal 1: If the capability signalling for fragmented carriers feature is agreed to be introduced in RAN4, it is preferred to have one-bit indication per-band per-BC capability signaling for NCCA configuration supported fragmented carriers feature within a CA band combination.

Semi-statically switch hardware resources

Another objective in the SID is to discuss the semi-statically switching hardware resources, i.e., the switching from one Rx RF chain (e.g., FC-mode) to separate Rx RF chains (e.g., non-FC-mode). The discussion is raised in the previous meetings, but it is not very clear for the expected UE behavior in the FC-mode when the higher interference is detected inside the gap between two non-contiguous CCs. In our understanding, how to measure the interference inside the gap in the FC-mode and how to determine the interference threshold between FC-mode to non-FC mode needs to be further clarified.

From UE perspective, one possibility is that the UE in FC-mode can monitor interference measurement inside the gap by itself and the UE determine whether it should transfer to non-FC mode when there are still sufficient hardware resources, i.e., more RF Rx chains can be used for non-FC mode. If there is no hardware resource, the UE can continue staying in FC-mode and depend on the network decision by the legacy scheduling mechanism to perform any reconfiguration or fallback configuration. From network perspective, one possibility is that the network may configure the reference signal, e.g., NZP-CSI-RS or ZP-CSI-RS, for each CC in the NCCA configuration to request RSSI/RSRP/RSRQ/SINR/CQI reporting from the UE side. Based on the reported values, the network can decide whether to perform any reconfiguration or fallback to other configuration for the UE.

In our view, it is straightforward that the Rx RF performance in non-FC mode is generally better than in FC-mode. The UE may default prefer to apply non-FC mode for the NCCA configuration if the hardware resource is sufficient. Only when much higher order CA band combination is configured, the FC-mode may be forced to trigger to enhance CC number for DL transmission. Hence, the mandatory scenario to enable FC-mode may be limited to the high order band combination which is running out all hardware resource. For other scenarios, since the UE may have the sufficient hardware resource, the network may not know whether the UE is currently in FC-mode or non-FC-mode for a configured band combination if there is no signaling to be introduced to acquire the FC-mode status for the UE.

In our understanding, assuming that the network can know the UE is in FC-mode, it is possible for the network to determine the failure for the UE in one RF Rx chain by legacy methods, i.e., RSSI/RSRP/RSRQ/SINR/CQI reporting, or the new introduced in-gap interference measurement. By following legacy RRC reconfiguration procedure, whether to perform any fallback reconfiguration for the UE can depends on the network decision. Hence, it is proposed that if the UE in one Rx RF chain mode can be detected as failure, it is preferred to follow the network decision on whether to perform any CA reconfiguration or fallback configuration for the UE.

Proposal 2: If the UE in one Rx RF chain mode can be detected as failure, it is preferred to follow the network decision on whether to perform any CA reconfiguration or fallback configuration for the UE.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: If the capability signalling for fragmented carriers feature is agreed to be introduced in RAN4, it is preferred to have one-bit indication per-band per-BC capability signaling for NCCA configuration supported fragmented carriers feature within a CA band combination.

Proposal 2: If the UE in one Rx RF chain mode can be detected as failure, it is preferred to follow the network decision on whether to perform any CA reconfiguration or fallback configuration for the UE.

# Reference

- RP-243002, “Revised SI: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc., RAN#106

- R4-2420519, “WF on NR FR1 DL fragmented carriers study”, MediaTek Inc., RAN4#114

- R4-2501189, “Discussion on methods for reducing the number of UE Rx chains for fragmented carriers”, Google, RAN4#114