# 3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504395

# Wuhan, China,7th–11th April 2025

Source:		Ericsson

Title:			On WUR RF requirement testability

Agenda item:		7.26.2.4

Document for:		Approval

# Introduction

In this paper, we present our overview on the WUR RF testability issue.

# Discussion

In WF[1], there are several issues relating to the testing aspects.

Issue 4-1-2: General framework of interface between the MR and the LR to verify RF performance

Agreements:

  - Group discuss and conclude the general framework for LP-WUS performance verification, consider the following as starting point:

    - Option 1: Counter the detection rate without waking up the MR would be enough to verify LP-WUS RF performance.

    - FFS if the MR should only be woken up when data transfer the system simulator is needed.

    - Other options are not precluded.

Our understanding is that counting the detection rate without waking up the MR is good enough for MDR performance metric testing. The reason is that if UE detect the LP-WUS target to itself, the MR would be woken up and MR should continue to monitoring PDCCH to see if there is real paging for this UE. There are also MDR performance for PDCCH detection, then to count a LP-WUS detection successfully, the UE should also detect the PDCCH, otherwise, network still regards the LP-WUS is missed. So involving further the PDCCH detection into the RF verification needs RAN4 further discussion in such case.  For the FFS item using data transfer, this is done for PDCCH testing and PDCCH is specified similar metric as LP-WUS in 38.101-4:

- The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

The PDCCH test in demodulation is also require a certain SNR testing threshold. From test setup perspective, it is quite similar to what would be needed for LP-WUS REFSENS testing. The only issue of carrying the data transfer to test is that how to remove the impact of PDCCH detection from the LP-WUS test result. If this issue solved, then carrying the data transfer between system simulator and DUT seems not a bad idea. The point here is that also the impact of the test equipment should be discussed as test equipment vendor may prefer one way over another to simply their design.

Option 1 should be used for testing framework of RF requirement for WUR.

Another potential issue is that it is not clear that how the UE should be tested if both OOK WUR and OFDM WUR can be supported for the same band. Assuming the REFSENS for OFDM WUR is lower than REFSENS of OOK WUR. For a overlaid signal design of LP-WUS signal, UE can decode either OFDM symbol or OOK symbol and waking up the MR afterwards. This will create a ambiguous situation on the target of the testing, is the OOK WUR or OFDM WUR that responding to the LP-WUS? To solve this ambiguity, a different UE ID should be configured for OFDM WUR and OOK WUR, i.e. the DUT UE ID is set for OOK WUR to test OOK WUR REFSENS but other UE ID set for OFDM WUR.

Ambiguity exists for testing REFSENS of two typs of WUR, e.g OOK WUR vs OFDM WUR if the UE support both for the same band.

Configure the DUT UE ID on either OOK part or OFDM part of the LP-WUS when testing only one type of WUR.

Some companies have concern on the testing time, mainly because the it may take long time from transmitting the LP-WUS till MR waking up and monitorring PDCCH. RAN4 may suggest RAN5 a new test mode but we think this can be decided by RAN5 and RAN4 can send LS to RAN5 to trigger the discussion.

Send a LS to RAN5 when the test framework is agreed and leave the test mode discussion to RAN5.

# Conclusions

In this contribution, we present our view on WUR RF testability with below proposal:

Proposal-1: Option 1 should be used for testing framework of RF requirement for WUR.

Observation 1 Ambiguity exists for testing REFSENS of two typs of WUR, e.g OOK WUR vs OFDM WUR if the UE support both for the same band.

Proposal-2: Configure the DUT UE ID on either OOK part or OFDM part of the LP-WUS when testing only one type of WUR.

Proposal-3: Send a LS to RAN5 when the test framework is agreed and leave the test mode discussion to RAN5.

# References

- R4-2420528, WF on LP-WUS UE RF requirements, vivo