3GPP TSG-RAN4 Meeting  #114bis	R4-2504455

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.23.1

Source:	Apple

Title:	RF impact due to UL resource muting

Document for:	Discussion

# 1	Introduction

RAN4 received an LS from RAN1 regarding UL resource muting to check impact on transmit signal quality/MPR [1], as copied below:

On UL resource muting, RAN1 has made the following agreements Agreement If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Agreement If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS. Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. - FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.. Note that RAN1 is still discussing the boosting in symbols containing PT-RS. RAN1 would like to ask RAN4 to check impact on transmit signal quality/MPR, if any.

In this contribution, we share initial findings after performing simulation with UL resource muting.

# 2	Discussion

In the simulation, we use the following assumptions:

| Simulation assumptions | Values |
| --- | --- |
| Waveform | DFT-S-OFDM and CP-OFDM |
| Channel bandwidth | 100MHz with 30kHz SCS |
| Power class | Mainly PC2. Also simulated one RB case for PC3 |
| Slot format | In a slot, there are 3 DMRS symbols. Out of the remaining 11 PUSCH symbols, two consecutive symbols are with comb-2 RE muting |
| Modulation schemes | Pi/2 BPSK, QPSK, 16QAM, 64QAM |

The simulation results are shown in the following two tables (the first for DFT-S-OFDM and the second for CP-OFDM)ß:

Note we simulated 108RBs for DFT-S-OFDM and 111RBs for CP-OFDM, respectively. This is to consider the case where in a 100MHz channel, the UL sub-band size is 40MHz.

In all cases that have been simulated, UL muting has minimal impact on EVM or MPR. That said, please note the impact when 256QAM is used is to be investigated.

Observation 1: In all cases that have been simulated, UL muting has minimal impact on EVM or MPR.

# 3	Conclusions

In this contribution, we perform simulations for UL resource muting and make the following observations.

Observation 1: In all cases that have been simulated, UL muting has minimal impact on EVM or MPR.

# 4	References

R1-2410889, “LS on impact on transmit signal quality/MPR requirement,” RAN1