3GPP TSG-RAN WG4 Meeting #114-bis									                      R4-2504580

Wuhan, China, April 7 – 11, 2025

Agenda item:			7.1.1.3.4

Source:	Ericsson

Title:	On the SRS IL imbalance reporting for the SRS-AS

Document for:	Approval

# Introduction

In RAN#107, a revised WID has been approved [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” WI. One of the three main parts of the WI is to specify 6Rx UE RF requirements for both handheld and FWA UEs, and in this contribution we only focus on the study of the issue of insertion loss imbalance across SRS ports:

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - …

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

Due to divergent views from different companies, progress on this topic is slow. However, most of companies have expressed their interest in introducing some kind of reporting, often including the “assistance” part. There is a slow but steady convergence towards a candidate solution framework, where the exact details are still for further study (see the approved WF [2]).

Similar to our recent contributions, we will not repeat the detailed analysis on the general considerations for the SRS IL imbalance issue nor the justification for the introduction of reporting. The issue has been well justified from different perspectives: in theory, by simulation results and by lab testing. In this contribution, we will mostly focus on the lab testing part, which illustrates the best the observed issue.

Regarding the existing UE behavior, according to the current specifications, the UE shall compensate for the IL below the maximum configured output power for a resource, as additional Tx insertion loss is not accounted for in the SRS power control equations (see [3] for more details).

Regarding the impact of SRS IL imbalance on the NW performance degradation, based on system-level simulation results, we have demonstrated that the SRS IL imbalance without reporting/compensation results in a non-negligible DL MIMO performance loss (see [3] for more details).

There are also other issues which have been observed for multiple commercial UEs during the lab testing, including the Rx chains being switched off (presumably for power saving reasons) during the periods when there is no DL transmission or even when the DL data is very small. In such cases, the SRS has not been transmitted from the corresponding Rx ports, which breaks the specifications.

In general, UL RMCs and parameters for verifying SRS requirements defined in TS38.101-1, clause 6.2.4, with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching' or ‘codeBook’ are not specified. This means that UE conformance with SRS transmitter power requirements and the ΔTRxSRS for antenna switching cannot be verified. In our contribution for RAN4#114 [4] (as a part of TEI-17), we have proposed RMCs for SRS transmissions that should be introduced to TS38.101-1, where the rest could be handled by RAN5.

# Discussion

In the approved WF [2], the following candidate solution framework has been captured as baseline for further discussion, where the details of the assistance part are still FFS:

Way forward: RAN4 to take the candidate solution framework below as baseline for further discussion. - Rel-19 UE performs self-compensation of SRS IL up to its configured maximum output power. - FFS on RAN4 specification impacts. - No RAN1 specification impact. - FFS if UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting. - Others

In the following, we will first present the new results from the lab testing and then we will propose the details of the SRS IL self-compensation and assistance framework. Finally, we will discuss the impact on the different specifications (on high-level).

First, we make the following proposals:

Proposal 1: Introduce reporting for the SRS insertion loss imbalance issue in Rel-19.

Proposal 2: The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases.

## Tx/Rx non-reciprocal imbalance issue

To investigate the actual Tx and Rx imbalances in the field and the extent to which they are reciprocal, we have put couple of commercial UEs and mobile testing platforms under laboratory testing.

### UE #1

The testing has been performed under the following set-up:

| Operating band | n78 |
| --- | --- |
| DL channel model | 1x4 AWGN static |
| UL channel model | 4x16 AWGN static |
| Cell bandwidth | 20 MHz |
| UL MIMO support | Dual Tx |
| AS capability | 2T4R |
| UE Power Class | PC2 |

Table 1: Test set-up for UE #1 and #2 testing

First, to study the Rx imbalance, the same SSB signal was applied to all four antenna connectors/ports of the UE, where a power level calibration has been performed to obtain a balanced Rx power delivered by the RF cables connected to each antenna connector, i.e. no impact on different cables. In Figure 1 (a), we show the measured SS-RSRP (dBm) over time per antenna connector/port where RSRP0, …, RSRP3 correspond to Port0, …, Port3 respectively. Port0 and Port2 are the primary ports (used for PUSCH/PUCCH transmissions) while Port1 and Port3 are the diversity ports.

- (b)

Figure 1: (a) SS-RSRP (dBm) per UE RX port/connector over time; (b) SRS power (dBm) per UE antenna port/connector received by the gNB

We observe that for this particular UE implementation, ports 0 and 3 are balanced and have around 1.5dB larger SS-RSRP in average than ports 1 and 2 which are also quite balanced. Ideally, the Rx imbalance across all four ports should be minimal, but these measurements show a “small” Rx imbalance, which can only add to the many already identified uncertainties. Another aspect which should be further investigated is whether the Rx gain depend on the input power level, i.e. does the 1.5dB difference between ports 0/3 and 1/2 remain roughly the same for different input power levels.

In Figure 1 (b), we show the received SRS power from four UE antenna ports at the same RX port by the gNB.

We observe that for this UE, the Tx imbalance is considerably larger than the Rx imbalance, the differences between port 0 and port 1, port 0 and port 2, as well as port 0 and port 3 being approximately 6dB, 2dB and 3.5dB, respectively. The Tx imbalance we observe is non-reciprocal to the Rx imbalance (we see no correlation). Primary ports 0 and 2 have around 3.5 – 4.0 dB larger powers compared with the corresponding diversity ports 3 and 1, respectively, which is not unexpected.

Observation 1: Based on the lab testing results of UE #1, the Tx imbalance is much larger than and non-reciprocal to the Rx imbalance.

Since the SRS resources are two-ports (2T4R AS), according to TS38.213 and TS38.215 the power of the ports of the given SRS resource shall be the same, i.e. ideally, the total power at the antenna connectors should be split equally for 2T4R. However, according to Figure 2 that is not the case, which also adds to the overall uncertainty. In the current specifications, there is no defined test which verifies the expected behavior.

Observation 2: Based on the lab testing results of UE #1, contrary to TS38.213 and TS38.215, the total power at the antenna connectors is not split equally for multi-port SRS resources (e.g. 2T4R), and there is no specified test which verifies the expected behavior.

### UE #2

We have placed another UE under the same test set-up given by Table 1.

- (b)

Figure 2: (a) SS-RSRP (dBm) per UE RX port/connector over time; (b) SRS power (dBm) per UE antenna port/connector received by the gNB

We observe on Figure 2 (a) that for UE #2, the Rx imbalance goes up to 3dB (between port 1 and port 3), while the Tx imbalance is slightly smaller, up to 2.5dB between port 1 and port 3. However, the Tx-Rx imbalance is non-reciprocal since based on SS-RSRP measurement of port 3, the pathloss of port 3 should be the largest and thus the received SRS power at gNB from port 3 is expected to be the lowest. Nevertheless, it is the other way round and the received SRS power from port 3 is the largest even though the same SRS power was applied for all ports (much smaller than the PCMAX), which is probably a consequence of the non-reciprocal Tx insertion loss.

Observation 3: Based on the lab testing results of UE #2, the Tx imbalance is slightly smaller than the Rx imbalance, but it is non-reciprocal to it due to Tx insertion losses presumably.

### UE #3

We have placed UE #3 under the test set-up given by Table 2.

| Operating band | n41 |
| --- | --- |
| DL channel model | 32x4 AWGN static |
| UL channel model | 4x32 AWGN static |
| Cell bandwidth | 100 MHz |
| AS capability | 1T4R |

Table 2: Test set-up for UE testing

(a)                                                                                  (b)

Figure 3: (a) SS-RSRP (dBm) per UE RX port/connector over time; (b) SRS power (dBm) per UE antenna port/connector received by the gNB

On Figure 3 (a), the received SS-RSRP per sample is plotted, where the average values for each port are given in Table 3:

| RSRP​ | Median(dBm)​ |
| --- | --- |
| RSRP0​ | -77.67​ |
| RSRP1​ | -77.93​ |
| RSRP2​ | -79.97 |
| RSRP3​ | -78.33​ |

Table 3: The average SS-RSRP per port for UE #3

Note that the SRS power on Figure 3 (b) is not properly scaled, but the relative difference between the ports is correct.

We observe that for UE #3, the average Rx imbalance goes up to 2.3dB approximately (between port 0 and port 2). The average Tx imbalance is roughly the same, being slightly larger than 2dB between port 0 and port 2. For this particular UE implementation, we can observe that the Tx and the Rx imbalance are fairly reciprocal, since both the SS-RSRP and the SRS power for port 2 are the smallest, probably due to the largest pathloss for this port.

Observation 4: Based on the lab testing results of UE #3, the Tx and the Rx imbalance are roughly the same and reciprocal.

### UE #4

We have placed UE #4 under the test set-up given by Table 2.

(a)                                                                                  (b)

Figure 4: (a) SS-RSRP (dBm) per UE RX port/connector over time; (b) SRS power (dBm) per UE antenna port/connector received by the gNB

On Figure 4 (a), the received SS-RSRP per sample is plotted, where the average values for each port are given in Table 4:

| RSRP​ | Median(dBm)​ |
| --- | --- |
| RSRP0​ | -78.71 |
| RSRP1​ | -80.66 |
| RSRP2​ | -77.74 |
| RSRP3​ | -78.80 |

Table 4: The average SS-RSRP per port for UE #4

Note that also the SRS power on Figure 4 (b) is not properly scaled, but the relative difference between the ports is correct.

We observe that for UE #4, the average Rx imbalance goes up to 2.9dB approximately (between port 1 and port 2). The average Tx imbalance is larger in this case, going up to 4.3dB approximately between port 2 and port 3. For this particular UE implementation, we can observe that the Tx imbalance is larger than and non-reciprocal to the Rx imbalance.

Observation 5: Based on the lab testing results of UE #4, the Tx imbalance is larger than and non-reciprocal to the Rx imbalance.

### General conclusion from the UE lab testing

From the lab testing of four commercial/mobile testing platform UEs we can conclude that in general the Tx-Rx imbalance is unpredictable and, in most cases, non-reciprocal. In most cases, the Tx imbalance is larger than the Rx imbalance, even though there are some UEs in the field for which the imbalance is reciprocal.

Observation 6: In general, the Tx-Rx imbalance is unpredictable and, in most cases, non-reciprocal. In most cases, the Tx imbalance is larger than the Rx imbalance, even though there are some UEs in the field for which the imbalance is reciprocal.

Due to this unpredictable UE behavior in the field, we can conclude that there would be a very limited benefit from reporting of the actual Tx insertion losses – the reporting of the SRS output power would be more beneficial, which can be performed as presented in clause 2.2. The benefit of such reporting is the largest when the Tx imbalance is much larger than the Rx imbalance.

Observation 7: There would be a very limited benefit from reporting of the actual Tx insertion losses – the reporting of the SRS output power would be more beneficial, where the benefit of such reporting is the largest when the Tx imbalance is much larger than the Rx imbalance.

## Details of the proposed solution

### Details of the SRS IL self-compensation part of solution

As discussed in the introduction and captured by the WF [2], the UE shall perform self-compensation for the IL below the maximum configured output power for a resource, as additional Tx insertion loss is not accounted for in the SRS power control equations. Since PCMAX takes into account the insertion loss by allowing the maximum relaxation ΔTRxSRS in its lower bound and is defined for each SRS resource separately, each SRS transmission power below PCMAX shall be feasible. Thus, the UE should perform the self-compensation of the SRS IL up to the maximum configured output power for the given SRS resource.

Since this behavior is already captured in the specifications and no changes have been proposed, no additional RAN4 performance requirements would need to be introduced.

Proposal 3: No additional RAN4 performance requirement needs to be introduced to verify that the UE performs the IL self-compensation.

Proposal 4: For the SRS IL self-compensation part of solution, there is no impact on RAN1 specifications since the IL is not considered in the SRS power control equations.

### Details of the assistance framework part of solution

Since the UE cannot compensate for the IL at the maximum configured output power, it should provide assistance to the network on the SRS IL compensation.

One candidate reporting parameter is the “SRS insertion loss value (per SRS resource or per UE)”. In our understanding, this is related to the actual insertion loss of different branches/SRS ports. In general, this could be a promising solution as long as the imbalance between the actual SRS transmission powers is equal to the actual insertion loss differences, which should be the case if the UE transmits at the maximum configured output power PCMAX at each SRS port.

However, as it can be concluded from our lab testing results and the online discussions, the UE behavior in the field on the IL compensation may be unpredictable. If the network takes into account the reported actual “SRS insertion loss value” and the UE compensates for the insertion loss, that may introduce even more inaccuracy for the DL CSI estimation.

Even if the network knew that the UE behaves as it is supposed to, i.e. pre-compensating for all the powers below the maximum configured output power, there is no way for the network to know if the UE has been able to compensate for the insertion losses (UE being near the cell center vs. being near the cell edge). Only if the PHR were transmitted for each SRS resource together with the actual insertion loss, the network would know whether to take into account the information on the actual insertion loss (whether the SRS transmission output power is the equal on all SRS resources/ports or it differs by the amount of the actual insertion losses).

In addition, there is no guarantee that the UE will set the maximum configured output power according to the actual insertion loss (for example, the chosen PCMAX could be “overoptimistic”), and in that case only a partial compensation of the IL may occur.

Finally, the mapping between the SRS ports and the antenna connectors can change with the time, and the “SRS insertion loss value” may become outdated.

Observation 8: Reporting of the actual insertion loss (or the actual IL imbalance), i.e. “SRS insertion loss value reporting”, is in general insufficient for the accurate DL CSI estimation.

Our proposed solution for the “UE assistance to the network” part is that reporting should include the configured maximum output power per SRS resource and the power headroom (PH) per SRS resource, as well as the information on the power class fallback or the return to the supported power class (ΔPPowerClass). In that way the network can have a relatively precise information on each SRS output power at the connector. Note that the reporting of all three of these parameters is already part of “Single Entry PHR MAC CE” and “Multiple Entry PHR MAC CE”.

Observation 9: Reporting of the configured maximum output power per SRS resource, the power headroom (PH) per SRS resource and the information on the power class fallback or the return to the supported power class (ΔPPowerClass) is already part of “Single Entry PHR MAC CE” and “Multiple Entry PHR MAC CE”.

The configured maximum output power includes the actual insertion loss (Tx/Rx) imbalance (for resources for which this is allowed) on top of the MPR used for the SRS resource. If the MPR is the same between the different ports (which is expected), by inspecting the differences between the configured maximum output power per SRS port the information on the IL imbalance could be obtained. On the other hand, by transmitting in addition the power headroom (PH) report per SRS resource, the network can be aware of whether a UE operates at its maximum power and it can obtain the information on the actual output power per SRS resource by subtracting the power headroom from the configured maximum output power (in that way it can observe whether the IL compensation has been performed). Both cases when UEs perform or do not perform a compensation for the ILs are covered by the proposed solution. For the UEs which do not perform the self-compensation, the assumption is that they would still configure the power on the antenna connector, but the (relative) PH would also include the Tx insertion loss. However, that wouldn’t affect our proposed solution.

Observation 10: The proposed solution is beneficial for all the UEs, irrespective of whether the self-compensation of the IL is performed or not.

The expected benefit from such solution is the maximization of the accuracy of the DL CSI estimation, which is of key importance for the performance of the DL MIMO, both Single User and Multi-User.

Proposal 5: The following SRS IL imbalance reporting solution should be specified:

- UE performs self-compensation of SRS IL up to its configured maximum output power.

- UE provides dynamic assistance to the network on SRS IL compensation:

  - Per SRS resource power headroom reporting

  - Configured maximum output power per SRS resource reporting

  - ΔPPowerClass reporting

Observation 11: The expected benefit from the proposed solution is the maximization of the accuracy of the DL CSI estimation, which is of key importance for the performance of both SU and MU MIMO.

## Detailed impact on the specifications

### RAN4 specifications: TS38.101-1

By adopting Proposal 3 there would be no/very limited impact on TS38.101-1 specification.

### RAN1 specifications: TS38.213

The Type 3 power headroom report is defined in clause 7.7.3 of TS38.213[5].

For insertion loss reporting per SRS resource, the only required change would be to allow Type 3 power headroom report for the UE configured for PUSCH transmissions on carrier f of serving cell c (for both an actual and a reference SRS transmission). In the following, we highlight the exact places in the specifications where the applicability of Type 3 power headroom is limited only to “PUSCH-less” carriers, e.g. SDL. This restriction should be lifted at least for a new MAC-CE, named for example “SRS resource PHR” (see clause 2.2.3), which should be straightforward to incorporate into the text.

If a UE determines that a Type 3 power headroom report for an activated serving cell is based on an actual SRS transmission then, for SRS transmission occasion on active UL BWP of carrier of serving cell c and if the UE is not configured for PUSCH transmissions on carrier of serving cell c and the resource for the SRS transmission is provided by SRS-Resource, the UE computes a Type 3 power headroom report as [dB] where , , , , and are defined in clause 7.3.1 with corresponding values provided by SRS-ResourceSet. If the UE determines that a Type 3 power headroom report for an activated serving cell is based on a reference SRS transmission then, for SRS transmission occasion on UL BWP of carrier of serving cell c, and if the UE is not configured for PUSCH transmissions on UL BWP of carrier of serving cell c and a resource for the reference SRS transmission is provided by SRS-Resource, the UE computes a Type 3 power headroom report as [dB] where is an SRS resource set corresponding to SRS-ResourceSetId = 0 for UL BWP and , , and are defined in clause 7.3.1 with corresponding values obtained from SRS-ResourceSetId = 0 for UL BWP . is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB and TC =0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2], [8-3, TS 38.101-3] and [8-5, TS 38.101-5].

One possible way to do it is the following:

If a UE determines that a Type 3 power headroom report for an activated serving cell is based on an actual SRS transmission then, for SRS transmission occasion on active UL BWP of carrier f of serving cell c and if the UE is not configured for PUSCH transmissions on carrier f of serving cell c and the resource for the SRS transmission is provided by SRS-Resource [or if the UE is configured for PUSCH transmissions on carrier c of serving cell f and the resource for the SRS transmission is provided by SRS-Resource with usage antennaSwitching], the UE computes a Type 3 power headroom report as

### RAN2 specifications: TS38.321

Power Headroom (PH) framework specified in TS38.321 [6] could be used for the proposed reporting solution. PH used for the SRS resource can be a Type 3 but used for a new MAC-CE, for example called “SRS resource PHR”. A similar approach as for the “Multiple Entry PHR MAC CE” could be used (clause 6.1.3.9 in [8]), where multiple PH (power headroom), PCMAX (maximum configured output power) and DPC (ΔPPowerClass) fields (each set corresponding to a different SRS resource) could be transmitted in a single MAC CE. The maximum number of entries could be eight, which corresponds to 8 Rx case and 1T8R AS, where an SRS Resource Set consists of eight single port SRS resources. A new MAC-CE specified for SRS power reporting should be dynamic since SRS AS power levels can change.

The “SRS resource PHR MAC CE” could be defined similar to following:

-	Ci: This field indicates the presence of a PH field for the SRS resource with index i. The Ci field set to 1 indicates that a PH field for the SRS resource with index i is reported. The Ci field set to 0 indicates that a PH field the SRS resource with index i is not reported;

-	V: This field indicates if the PH value is based on a real transmission or a reference format. The V field set to 0 indicates real transmission on SRS and the V field set to 1 indicates that an SRS reference format is used. Furthermore, the V field set to 0 indicates the presence of the octet containing the associated PCMAX,f,c field and the DPC field, and the V field set to 1 indicates that the octet containing the associated PCMAX,f,c field and the DPC field is omitted;

-	Power Headroom (PH): This field indicates the power headroom level for the SRS resource i. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in TBD;

-	P: This field indicates whether power backoff is applied due to power management (as allowed by P-MPRc as specified in TS 38.101-1, TS 38.101-2, and TS 38.101-3). The MAC entity shall set the P field to 1 if the corresponding PCMAX,f,c field would have had a different value if no power backoff due to power management had been applied;

-	PCMAX,f,c i: If present, this field indicates the PCMAX,f,c (as specified in TS 38.213) for the SRS resource i used for calculation of the preceding PH field. The reported PCMAX,f,c and the corresponding nominal UE transmit power levels are shown in TBD (the corresponding measured values in dBm for the NR Serving Cell are specified in TS 38.133);

-	DPC: If dpc-Reporting-FR1 is configured, and the Serving Cell operates on FR1, this field indicates the ΔPPowerClass for SRS resource i, as specified in TS 38.101-1[14] and TS 38.101-3 [16]. This field indicates an index to Table 6.1.3.8-4 and the corresponding measured values of DPC levels in dB are specified in TS 38.133 [11], the DPC field is set to 0 if the criteria to report DPC is not met. The length of the field is 2 bits.

| C7 | C6 | C5 | C4 | C3 | C2 | C1 | C0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P | V | PH (Type 3, SRS Resource 0) |  |  |  |  |  |
| DPC0 |  | PCMAX,f,c 0 |  |  |  |  |  |
| P | V | PH (Type 3, SRS Resource 1) |  |  |  |  |  |
| DPC1 |  | PCMAX,f,c 1 |  |  |  |  |  |
| P | V | PH (Type 3, SRS Resource 2) |  |  |  |  |  |
| DPC2 |  | PCMAX,f,c 2 |  |  |  |  |  |

…

| P | V | PH (Type 3, SRS Resource 7) |
| --- | --- | --- |
| DPC7 |  | PCMAX,f,c 7 |

Figure 5: Possible configuration of “SRS resource PHR MAC CE” with the highest number of SRS resources equal to 8 (corresponding to ‘t1r8’ AS)

Such PHR configuration would be suitable even if the IL reporting were per SRS port, since the maximum total number of SRS ports over all SRS resources remains equal to eight (for 8Rx case).

The PH reporting with the proposed MAC-CE could be periodic (triggered by the expiration of timers, including a “prohibit” timer” to reduce reporting frequency) or by any modification of the SRS configuration.

Proposal 6: The PH reporting with the proposed MAC-CE could be periodic (triggered by the expiration of timers, including a “prohibit” timer” to reduce reporting frequency) or by any modification of the SRS configuration.

# Conclusion

In the previous sections, we made the following observations:

Observation 1: Based on the lab testing results of UE #1, the Tx imbalance is much larger than and non-reciprocal to the Rx imbalance.

Observation 2: Based on the lab testing results of UE #1, contrary to TS38.213 and TS38.215, the total power at the antenna connectors is not split equally for multi-port SRS resources (e.g. 2T4R), and there is no specified test which verifies the expected behavior.

Observation 3: Based on the lab testing results of UE #2, the Tx imbalance is slightly smaller than the Rx imbalance, but it is non-reciprocal to it due to Tx insertion losses presumably.

Observation 4: Based on the lab testing results of UE #3, the Tx and the Rx imbalance are roughly the same and reciprocal.

Observation 5: Based on the lab testing results of UE #4, the Tx imbalance is larger than and non-reciprocal to the Rx imbalance.

Observation 6: In general, the Tx-Rx imbalance is unpredictable and, in most cases, non-reciprocal. In most cases, the Tx imbalance is larger than the Rx imbalance, even though there are some UEs in the field for which the imbalance is reciprocal.

Observation 7: There would be a very limited benefit from reporting of the actual Tx insertion losses – the reporting of the SRS output power would be more beneficial, where the benefit of such reporting is the largest when the Tx imbalance is much larger than the Rx imbalance.

Observation 8: Reporting of the actual insertion loss (or the actual IL imbalance), i.e. “SRS insertion loss value reporting”, is in general insufficient for the accurate DL CSI estimation.

Observation 9: Reporting of the configured maximum output power per SRS resource, the power headroom (PH) per SRS resource and the information on the power class fallback or the return to the supported power class (ΔPPowerClass) is already part of “Single Entry PHR MAC CE” and “Multiple Entry PHR MAC CE”.

Observation 10: The proposed solution is beneficial for all the UEs, irrespective of whether the self-compensation of the IL is performed or not.

Observation 11: The expected benefit from the proposed solution is the maximization of the accuracy of the DL CSI estimation, which is of key importance for the performance of both SU and MU MIMO.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Introduce reporting for the SRS insertion loss imbalance issue in Rel-19.

Proposal 2: The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases.

Proposal 3: No additional RAN4 performance requirement needs to be introduced to verify that the UE performs the IL self-compensation.

Proposal 4: For the SRS IL self-compensation part of solution, there is no impact on RAN1 specifications since the IL is not considered in the SRS power control equations.

Proposal 5: The following SRS IL imbalance reporting solution should be specified:

- UE performs self-compensation of SRS IL up to its configured maximum output power.

- UE provides dynamic assistance to the network on SRS IL compensation:

  - Per SRS resource power headroom reporting

  - Configured maximum output power per SRS resource reporting

  - ΔPPowerClass reporting

Proposal 6: The PH reporting with the proposed MAC-CE could be periodic (triggered by the expiration of timers, including a “prohibit” timer” to reduce reporting frequency) or by any modification of the SRS configuration.

# References

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#107.

- R4-2503014, “WF on 6Rx requirements”, AT&T, RAN4#114.

- R4-2413360, “On need for SRS insertion loss imbalance reporting”, Ericsson, RAN4#112.

- R4-2501566, “(NR_RF_TxD-Core) Reference measurement channels for SRS transmissions”, Ericsson, RAN4#114.

- 3GPP TS38.213 V18.5.0, “NR; Physical layer procedures for control”.

- 3GPP TS38.321 V18.5.0, “NR; Medium Access Control (MAC) protocol specification”.