3GPP TSG-RAN WG4 Meeting #114bis	R4-2504599

Wuhan, China, April 7 – 11, 2025

Agenda Item:	7.26.4.2

Source:	MediaTek Inc.

Title:	Discussion on the RRM core requirements for LP-WUS

Document for:	Discussion

# Introduction

The RRM core requirements for Rel-19 LP-WUS/WUR was further discussed in the last RAN4 meeting, and the achieved agreements are captured in the WF [1]. In this paper, we continue the discussion on the open issues in the RRM core part and provide our view. The WID [2] is also provided as below for reference.

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

# Discussion

## Cases/states of RRM relaxation for LP-WUS capable UE

In the last meeting, RAN4 further discussed the possible RRM offloading/relaxation cases for LP-WUS capable UE to be considered in Rel-19 LP-WUS.

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading - Proposals Regarding case 2: P1: Support (LG CMCC vivo); P2: De-prioritize (xiaomi oppo); P3: Not consider (ZTE Apple Ericsson Huawei MTK); P3-1: Not consider however can be revisit in future upon RAN1/2 progress () - Other related proposals - P1: Whether LR based serving cell RRM relaxation case is valid is fully up to NW’s configuration, such as NW can enable/disable any RRM relaxation scenario based on the configured thresholds. (Ericsson, Nokia, CMCC)

In RAN4#111 meeting, RAN4 agreed on Case#1 (RRM offloading case) first and then in RAN4#112 meeting, RAN4 agreed on Case#3 (RRM relaxation case). Regarding whether to discuss other cases for LP-WUS, in our view the current agreed cases in RAN4, Case#1 and Case#3, are sufficient to consider both scenarios of MR RRM offloading and MR RRM relaxation of serving and neighbour cell measurements, which align with the WID objective in [2]. Therefore, RAN4 should focus on defining the core requirements for the current agreed cases, and if some issues are detected later to consider other cases, it can be revisited.

Proposal 1: No need to discuss further cases for LP-WUS in RAN4.

## Entry/Exit conditions

Following the progress in RAN4, LP-WUS capable UE may operate under three scenarios (see Figure 1):

- RRM offloading (Case#1): when MR is deactivated and LR is activated.

- RRM relaxation (Case#3): when both MR and LR are activated.

- Legacy UE: when MR is activated and LR is deactivated.

Figure 1: Different cases of LP-WUR UE

In Case#1, MR is deactivated and LR is activated. LR should perform wake-up signal monitoring and serving cell measurements in RRC IDLE/INACTIVE mode. All other tasks performed by MR (paging, serving and neighbour cell RRM measurements) are deactivated for power saving. Since in this case serving cell measurement is exclusively performed by LR, UE needs to rely on the LR’s RRM measurements to determine whether to stay in the current state or switch to another state. In this case, UE can either exit to legacy UE behaviour or enter Case#3 as shown in Figure 2.

Figure 3: UE at Case#3.

Figure 2: UE at Case#1.

In Case#3, both MR and LR are activated. LR performs wake-up signal monitoring and serving cell measurements in RRC IDLE/INACTIVE mode. MR performs relaxed serving and neighbour cell measurements in IDLE/INACTIVE mode. Since, in this case, serving cell measurements are performed by both MR and LR, RAN4 agreed not to consider combining the measurements across the two radios, i.e., MR and LR. When UE at Case#3, UE can either exit to legacy UE behaviour or enter Case#1 as shown in Figure 3. It is noted that, RAN2 agreed that exit condition is based on LR measurements only without MR measurements as captured in [3].

RAN2#127-bis agreement: - The LPWUS monitoring exit condition does not include MR measurements. - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). RAN2#128 agreements - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

Therefore, for both cases (Case#1 and Case#3) RAN4 can define same evaluation requirements for exit conditions based on LR measurements only. On the other hand, the entry conditions can be evaluated by MR against MR threshold for both Case#1 and Case#3, however, if LR threshold is also configured then the LR evaluation can also be considered. Note that the evaluation of MR is based on DRX cycle (similar to legacy serving cell measurements Nserv), while LR evaluation should be based on LO cycle regardless of whether LP-SS is configured or not (Note: DRX cycle and LO periodicity are the same based on RAN1 agreement).

Proposal 2: RAN4 to define evaluation requirements of entry/exit criteria for the following cases:

| Cases | Entry condition | Exit condition |
| --- | --- | --- |
| Case#1 (only LR) | - Evaluation based on MR using DRX cycle - Optional evaluation based on LR using LO cycle | - Evaluation based on LR using LO cycle |
| Case#3 (LR+MR) | - Evaluation based on MR using DRX cycle - Optional evaluation based on LR using LO cycle | - Evaluation based on LR using LO cycle |

Another issue needs to be discussed is that how to avoid the ping pong behaviour when transitioning between different cases. Activating and deactivating MR can consume more power and takes longer time compared to LR. Therefore, it is important to ensure UE can confidently switch to any case without having ping pong impact.

Proposal 3: Discuss how to avoid the ping pong behaviour when transitioning between different cases based on RRM measurements.

Since in Case#1 RRM measurements are fully offloaded to LR, UE need to be confident enough to enter Case#1 before deactivating MR. Therefore, we propose that RAN4 to study allowing the UE entering Case#3 first before entering Case#1 after a period of time (i.e., even when signal quality is higher than the threshold). This will help UE switching to Case#1 for RRM fully offloading more confidently after ensuring that signal quality condition does not change during Case#3 for a given period of time.

Proposal 4: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1.

Regarding the evaluation requirement, RAN4 last agreement:

Issue 1-2-4-2-3: On how to define LR evaluation requirements Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

RAN4 need to discuss the number of samples required to define the evaluation duration requirements for LR for both types OOK LR and OFDM LR. Given LR perform worse than MR, a higher number of samples is required to achieve reliable evaluation.

Proposal 5: For LR evaluation requirements and since OOK LR perform worse than MR, a higher number of samples than MR is required to achieve reliable evaluation.

RAN4 also need to discuss the MR evaluation requirements for RRM fully offloading to LR since RAN4 agreed to define the evaluation for both separately.

Proposal 6: RAN4 to define MR evaluation requirements for RRM fully offloading to LR.

## Others

Some other issues were also discussed in RAN4 as captured in the WF [1] below:

Issue 1-1-8: LP-WUR operating carrier frequency Issue 1-2-1: Accuracy requirements Issue 1-2-5: On LP-WUR cell selection evaluation requirements for case 1 Issue 1-4-1: LP-WUR at CONNECTED mode

For Issue 1-1-8, RAN4 received LS (R2-2501394) from RAN2 and according to RAN2 any potential overload issues related to operating carrier frequency could be addressed by current mechanism in spec. Therefore, in our understanding, previous RAN4 agreement of assuming both LR and MR are operating on the same carrier frequency as baseline is sufficient to define the RRM requirement for the LPWUS WI.

Proposal 7: On the LP-WUR operating carrier frequency, previous RAN4 agreement of assuming both LR and MR are operating on the same carrier frequency as baseline is sufficient to define the RRM requirement.

Also, regarding the new received LS from RAN1 (R1-2501624), RAN1 made the following agreement:

Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities

Based on above, RAN1 asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities. In our view, RAN4 can confirm RAN1 assumption.

Proposal 8: RAN4 to confirm to RAN1 that it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

For Issue 1-2-1, we support that not to capture dedicated accuracy requirement in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Proposal 9: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

For Issue 1-2-5, since in Case#1 only LR is activated in idle/inactive mode, RAN4 need to discuss whether to define cell selection requirements (evaluate S criteria) based on LR measurements. The issue will depend on the threshold configuration for the LR exit condition. If LR exit threshold is higher S criteria threshold then LR will exit Case#1 anyway even when the measurement is higher than S criteria threshold. Then MR can meet cell selection requirement. This also apply for Case 3 as well.

Proposal 10: No need to define the LR based cell selection criterion evaluation in Case#1 and Case#3.

For issue 1-4-1, we also agree that no RRM objectives is expected for connected mode according to the WID.

Proposal 11: No RRM objectives is expected for connected mode in this WI.

# Summary

In this contribution, the following conclusions are provided:

Proposal 1: No need to discuss further cases for LP-WUS in RAN4.

Proposal 2: RAN4 to define evaluation requirements of entry/exit criteria for the following cases:

| Cases | Entry condition | Exit condition |
| --- | --- | --- |
| Case#1 (only LR) | - Evaluation based on MR using DRX cycle - Optional evaluation based on LR using LO cycle | - Evaluation based on LR using LO cycle |
| Case#3 (LR+MR) | - Evaluation based on MR using DRX cycle - Optional evaluation based on LR using LO cycle | - Evaluation based on LR using LO cycle |

Proposal 3: Discuss how to avoid the ping pong behaviour when transitioning between different cases based on RRM measurements.

Proposal 4: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1.

Proposal 5: For LR evaluation requirements and since OOK LR perform worse than MR, a higher number of samples than MR is required to achieve reliable evaluation.

Proposal 6: RAN4 to define MR evaluation requirements for RRM fully offloading to LR.

Proposal 7: On the LP-WUR operating carrier frequency, previous RAN4 agreement of assuming both LR and MR are operating on the same carrier frequency as baseline is sufficient to define the RRM requirement.

Proposal 8: RAN4 to confirm to RAN1 that it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

Proposal 9: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Proposal 10: No need to define the LR based cell selection criterion evaluation in Case#1 and Case#3.

Proposal 11: No RRM objectives is expected for connected mode in this WI.

# References

- R4-2502600, WF on RRM requirements for NR_LPWUS, vivo.

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO.

- R2-2409212, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD, CATT