# 3GPP TSG-RAN WG4 Meeting #114bis	                 		                                   R4-2504910

# Wuhan, China, 7 – 11 April 2025

Agenda item:			7.27.3

Source:	China Telecom

Title:	WF on RRM requirements for NR_Mob_Ph4_Part2

Document for:	Approval

Notes

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other proposals can be proposed.

Topic #1: RRM discussion

General aspects and early synchronization

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement

RAN4#114 < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.

< Way forward > FFS the following options:

Conclude this issue in the next meeting.

  - Option 1: Define requirements for L1 measurement on deactivated Scell.

  - Option 2: Define requirements for UE based TA measurement.

  - Option 3: Not define requirement for any of them.

Conditional LTM delay

Issue 1-2-1: General aspects of Conditional LTM delay

< Agreement >

  - CHO requirements framework can be used as baseline for Conditional LTM.

Issue 1-2-2: The starting point of conditional LTM delay

RAN4#114 < Agreement > ad hoc - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM, [under assumption that UE will start evaluating the CLTM event after reception of the RRC for CLTM]. - Revisit if RAN2 has more progress.

< Agreement >

  - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

Issue 1-2-3: The components of conditional LTM delay

Issue 1-2-4: Tinterrupt

< Agreement >

  - Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay.

    - Note: The legacy value(s) in previous releases will be reused.

  - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt.

  - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay.

    - TEvent_DU can be updated based on RAN2 progress if needed.

  - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell.

    - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed.

      - Measurement latency for both serving and candidate cells need to be considered.

    - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO.

    - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19.

< Way forward > FFS

  - Tfirst-RS and TRS-proc are included in Tinterrupt

    - Reuse the values in Rel-18 LTM, and further discuss whether the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not.

Issue 1-2-5: TRRC

< Way forward > FFS the following proposals:

- Proposal 1: TRRC is the RRC procedure delay.

Issue 1-2-6: TEvent_DU

- The issue is merged into issue 1-2-3.

Issue 1-2-7: Tmeasure

- The issue is merged into issue 1-2-3.

Issue 1-2-8: TCLTM_execution

- The issue is merged into issue 1-2-3.

Issue 1-2-9: TLTM-RRC-processing

- The issue is merged into issue 1-2-3.

Issue 1-2-10: TLTM-processing

< Way forward > FFS the following proposal:

- Proposal 1: TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

    - Proposal 1a: TLTM-processing can be up to 20ms.

Issue 1-2-11: Tfirst-RS / TRS

Issue 1-2-12: TRS-proc

- The issues are merged into issue 1-2-3.

Issue 1-2-13: TLTM-IU

< Way forward > FFS the following proposal:

- Proposal 1: TLTM-IU is the interruption uncertainty during LTM cell switch.

Issue 1-2-14: Twait_RS

< Way forward > FFS the following proposal:

- Proposal 1: Twait_RS is the uncertainty in acquiring the first SSB for T/F tracking on the new cell.

Issue 1-2-15: Other

< Agreement >

- L1 measurement behaviour, which was introduced in R18, is not changed. CLTM requirements do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2.

Subsequent Conditional LTM

Issue 1-3-1: General aspects of Subsequent Conditional LTM delay

< Way forward > FFS the following options:

- Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

    - Proposal 1a: TRRC will not be considered in subsequent conditional LTM delay.

    - Proposal 1b: TEvent_DU needs to be updated.

    - Proposal 1c: Apart from the starting point of the delay.

- Proposal 2: The subsequent CLTM cell switch delay DCLTM-subsequent is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL message on the new target cell.

Issue 1-3-2: The starting point of Subsequent conditional LTM delay

< Agreement >

- The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

Issue 1-3-3: The components of subsequent conditional LTM delay

< Way forward > FFS the following proposals:

- Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. (Apple)

- Tmeasure is measurements time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- Proposal 2: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_interuption + Tfirst-RS + TRS-proc, where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU.

- Proposal 3: DCLTM-subsequent = TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt, where the TCLTM_interrupt = TCLTM-processing + TCLTM-IU.

Issue 1-3-4: Other

< Way forward > FFS the following proposal:

- Proposal 1: Define initial and subsequent CLTM cell switch delay requirements in the same section under section 6.3.

- CR skeleton

| Expected impact | Notes | New or impacted section | Companies |
| --- | --- | --- | --- |
| CLTM delay requirements and subsequent CLTM delay requirements |  | 6.x | China Telecom |