3GPP TSG-RAN WG4 Meeting # 114bis											R4-2504918

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.22.4

Source:	Samsung

Title:	Ad-hoc minutes for NR_MIMO_Ph5

Document for:	Approval

Introduction

This Ad-hoc minutes is to cover the discussion in [114bis][211] NR_MIMO_Ph5_Part1 and [114bis][212] NR_MIMO_Ph5_Part2 for the following issues:

45 min discussion for CJTC reporting and two TA and UL TCI state switching.

45 min disucssion for UEIBM

Topic #1: RRM requiements for MIMO_Ph5_Part2

- Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration?

- Proposals

  - Proposal 1 (Apple): Do not define core requirements for CJT calibration reporting enhancement for delay/frequency/phase offset reporting based on multiple samples.

  - Proposal 2 (Ericsson):

    - RAN4 to define the delay requirements of the CJTC frequency offset with 3 sample averaging.

    - RAN4 to define the delay requirements of the CJTC delay offset with 5 sample averaging.

  - Proposal 3 (MediaTek):

    - RAN4 to consider 3 samples to define RRM requirements for timing and frequency offsets between two TRPs.

    - RAN4 to adopt higher SNR side condition for the accuracy requirements of the delay and frequency offsets.

  - Proposal 4 (Nokia): RAN4 to define accuracy requirements for TO, FO, and PO CJT calibration reporting.

- Recommended WF

  - Down select to one of the following options:

    - Option 1: Not to define core requirements for CJT calibration reporting enhancement.

    - Option 2: Define core requirements for delay and frequency offset reporting and FFS number of samples.

For CJTC-P, it is another issue of Issue 1-3 in moderator summary.

Issue 1-3: How to define RRM core requirements for phase offset reporting based on the simulation results for CJT calibration?

Adhoc Discussion:

From the recommended WF from moderator’s, let’s collect the views from companies.

Samsung: we prefer option 2. No matter how many samples are needed, we can further discuss on the value of it, even if the number of samples can be one sample, we prefer to define core requirements for measurement period. Similar s RSSI in legacy, it is also based on one sample, but measurement period is defined base on one sample.

MTK: some companies prefer yes to define core. As moderator suggested, just down select from option 1 and option 2 here. We prefer option 2.

Intel: option 1 is difficult for us. We think the core req is important. For the measurement period and accuracy req. both of them are ensure from UE side.

E///: we can agree it option 2.

HW: we support to define the core requirements. Can further discuss about the how many samples for measurement periods.

CATT: we prefer the option 2 but we are not ok to preclude the CJTC-P. For the CJTC-P, we can further discuss but not accept to preclude CJTC-P right now.

Apple: our proposal is OP1. If it is just one sample, we don’t think core requirements is not needed. As long as the UE can pass the accuracy. We don’t have the core req for TDCP in Rel-18.

Samsung: TDCP is another different.

Apple: for Aperiodic reporting, our understanding is similar as aperiodic reporting in legacy, no measurement period is needed and applied.

Nokia: we prefer option2. If go option 1, even it is one-shot measurement, NW want to know how long will be used for the measurement period.

E///: M = 1 and M=3 are both applicable for Aperiodic report type for legacy.

Apple: as long as the UE can pass the accuracy requirements, no need for the measurement period.

Nokia: UE can decide whether to use the previous one for measurement occasion, if so, the measurement period is something related to what we discussed.

Intel: event-triggered in L3 measurement is one time measurement like what we defined in L3 measurement.

Samsung: another thing is for the scheduling restriction. Especially for CJTC-D, the out-of-range is indicated by UE, so the UE have to measure the CJTC measurement for more accurate performance.

The current status:

    - Option 1: (Apple)

      - Not to define core requirements for CJT calibration reporting enhancement.

    - Option 2: (Samsung, MTK, Intel, HW, Catt (not preclude for the CJTC-P), Nokia, E///)

      - Define core requirements for delay and frequency offset reporting and FFS number of samples.

    - Option 3: (QC)

      - Decide how many samples are needed, then come back to how/whether to define the measurement period.

Agreement:

- Issue 2-4: For Scenario#1 in FR2, whether to update the RRM requirements for 2TA enhancement for FR2?

In RAN4#114, RAN4 agreed to define the timing requirements for FR1, but FFS for FR2:

Agreement: For Scenario#1: RAN4 define the timing requirements for FR1, FFS on FR2. For Scenario#2, FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not). RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.” are currently only applied for Scenario #1. - Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s) - Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s). Note 1: the scenario 1 and 2 are based on RAN1 LS. Note 2: The above agreements apply when 2TA are configured.

- Proposals

  - Proposal 1 (Samsung, Nokia, MediaTek): RAN4 to define timing requirements for FR1 as well as FR2.

- Recommended WF

  - TBA

Adhoc Discussion:

Agreement:

RAN4 to define timing requirements for FR1 as well as FR2.

- Issue 2-5: Whether to define UL TCI state activation requirement based on SRS?

In RAN4#114, RAN4 WF was to down select one proposal from the following:

Way forward: Further discuss the two proposals and down-select one proposal in the next meeting: - Proposal 1 (Qualcomm, Huawei, MediaTek, Samsung, Apple, QC): No RRM impacts on TCI state switching requirement. - Proposal 3 (Xiaomi, Nokia): - Legacy DCI based UL TCI state activation requirement can be re-used for UL TRP. - RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline. - For DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. no need to define SRS based ULTCI state activation delay. - Further refine the wording for P3 in the WF, if needed

- Proposals

  - Proposal 1 (Qualcomm, Huawei, Samsung, Apple, MediaTek, Xiaomi [for Scenario#1]): No RRM impacts on TCI state switching requirement.

    - Proposal 1a (Qualcomm): RAN4 should not define RRM TCI state switching requirements based on SRS since no additional RRM capability in the UE is tested.

    - Proposal 1b (Samsung) If UL TCI states is associated to SRS: Follow same principle in Rel-17 which is no requirement for UE is requested to a separate UL TCI state.

    - Proposal 1c (MediaTek): If UL TCI states can be associated to SRS, then reuse the same principle in Rel-17 which is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS.

  - Proposal 2 (Ericsson): RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI state and the switch delay is 3ms.

  - Proposal 3 (ZTE): When applying R17/18 unified TCI state framework to support 2TA in asymmetric scenario,

    - If there is SSB received from the UL-only TRP, both existing R17/18 joint and UL TCI state could be used.

    - Otherwise, only the UL TCI state of existing R17/18 could be used for the UL-only TRP.

  - Proposal 4 (Nokia):

    - Update the unified UL TCI state switching delay requirements to be applicable when the target UL TCI state is associated with an uplink reference signal.

    - For the asymmetric DL/UL scenario, reuse Rel-17 and Rel-18 unified TCI single-DCI mTRP TCI state switching delay requirements for DL and UL TCI state switch.

    - RAN4 to define UL TCI state switch delay requirements for an uplink-only TRP with no distinction between known and unknown TCI states.

    - In the case of an uplink only TRP which does not transmit any downlink RS, the UL TCI state switch delay requirements shall apply when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS.

    - While defining TCI state switch delay requirements, RAN4 to consider the maintenance status of the PL-RS of the DL TRP associated to the target TCI state of the UL only TRP.

- Recommended WF

  - Check if P1 is agreeable.

Adhoc Discussion:

Agreement:

- Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

In RAN4#114, RAN4 agreed to update the RRM requirements for 2TA enhancement for Scenario#1:

Agreement: - For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state. - The above agreement can be revisited if not aligned with RAN1 further agreement.

- Proposals

  - Proposal 1 (Qualcomm): For scenario #1 gradual timing adjustment should be applied with the same value for both TAGs.

  - Proposal 2 (Ericsson): Gradual timing adjustment applies for each TAG and shall be met for each TAG.

  - Proposal 3 (Apple): The gradual timing adjustment requirements are only applicable to TRP with both UL and DL.

  - Proposal 4 (Nokia): With asymmetric DL sTRP/UL mTRP, specify conditions under which UE autonomous timing adjustment applies to just the DL/UL TRP or also to the UL-only TRP.

- Recommended WF

  - Note: proposals are not mutually exclusive.

  - Discuss whether gradual timing adjustment applies to both TAGs or not (as in P1 and P2)

  - Discuss whether gradual timing adjustment applies to only DL/UL TRP or also to the UL-only TRP (as in P3 and P4).

Adhoc Discussion:

Agreement:

Topic #2: RRM requirements for UE-initiated/event-driven beam management

- Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

[Agreement in RAN4#114]

Agreement:

The agreements under this issue are for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1.

TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS

  - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4

- Proposals

  - Proposal 1: (Qualcomm)

    - For the case M > 1 and P > 1, the measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam.

    - RAN4 should focus on the case of same periodicities of the current beam RS and the new beam RS(s) to define measurement reporting delay requirements.

  - Proposal 2: (Xiaomi, ZTE)

    - For event-2, apply the same measurement periodicities of current beam and new beam(s) to define the requirements.

  - Proposal 3: (Apple)

    - Measurement period for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam }
In FR1/ FR2 with resources not adjacent or overlapping: max{ Mc*N*Pc*TcurrBeam , MN*N*PN*TnewBeam }
In FR2 with resources adjacent or overlapping: max{ Mc*N*2*Pc*TcurrBeam , MN*N*2*PN*TnewBeam }

  - Proposal 4: (LGE)

    - M=1

    - TSSB = Max{TSSB_current, TSSB_new}

  - Proposal 5: (Samsung)

    - The evaluation periodicity, for event 2, the periodicity of current beam and new beam are the same. the evaluation periodicity can reuse the legacy L1-RSRP measurement period.Take SSB as example, the Tssb is the periodicity of the SSB-Index in indicated TCI state or QCLed with the indicated TCI state.

    - For M in L1_measurement periods, reuse the legacy value: SSB based: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. CSI-RS based: For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise; For aperiodic CSI-RS resources M=1

    - For event-2, P in L1_measurement periods, reuse the same definition in existing requirements.

  - Proposal 6: (Huawei)

    - prioritize the requirements for UEIBM for same periodicity between new beam(s) and current beam.

    - When timeRestrictionForChannelMeasurement is configured, M =1, and the event evaluation should be based on the most recent result from both new beam and current beam

    - When timeRestrictionForChannelMeasurement is not configured, whether filtering is assumed for both new beam and current beam shall be discussed.

  - Proposal 7: (Nokia)

    - If timeRestrictionForChannelMeasurement configuration is supported when UE is configured with UEIBM event-triggered reporting, RAN4 to reuse M=1 or M=3 for computation of L1 RSRP event triggered measurement period depending on whether timeRestrictionForChannelMeasurement is configured, similar to legacy.

    - Only define requirements considering same periodicity of current beam and new beam unless there are further RAN1 agreements on the support of different periodicities for current beam and new beam

- Recommended WF

  - For M L1_measurement periods:

    - Option 1: (Samsung, Nokia, MediaTek, Apple)

      - reuse the legacy definition and requirements

    - Option 1a (Apple): Consider Mc and Mn for current and new beam respectively.

    - Option 2: (LGE)

      - Only M = 1

    - Option 3: (Huawei)

      - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam.

      - If timeRestrictionForChannelMeasurement is not configured, to be discuss

  - For P (sharing factor) L1_measurement periods:

    - Option 1: (Apple)

      - Separate Pc and Pn for current beam and new beam

    - Option 2: (Samsung)

      - reuse the legacy definition and requirements

  - For T_RS (TSSB or TCSI-RS) in L1_measurement periods

    - T_RS (TSSB or TCSI-RS):

      - Option1: (Qualcomm, Xiaomi, Samsung, Huawei, Nokia, ZTE, MediaTek)

        - the value of current beam and new beams are the same.

      - Option 2: (LGE, MediaTek (if RAN1 further agree to support different periodicities))

        - Max{TSSB_current, TSSB_new}

      - Option 3: (Ericsson)

        - When the RS periodicity for current beam and new beam is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new.

Adhoc Discussion:

  - For M L1_measurement periods:

    - Option 1: (Samsung, Nokia, MediaTek, Apple)

      - reuse the legacy definition and requirements

    - Option 1a (Apple): Consider Mc and Mn for current and new beam respectively.

    - Option 2: (LGE)

      - Only M = 1

    - Option 3: (Huawei)

      - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam.

      - If timeRestrictionForChannelMeasurement is not configured, to be discuss

HW: for M=1, it is used “most recent”, it is the same as legacy. But for M=3, we want to further discuss whether it can be configured or not.

LGE: we just focus on timeRestrictionForChannelMeasurement is configured.

Samsung: we think both the timeRestrictionForChannelMeasurement is configured or not are allowed from RAN1. If timeRestrictionForChannelMeasurement is not configured, use 3 samples as legacy. In RAN1 draft CR, they just added the section in “Triggering/activation of CSI Reports”. But for legacy L1-RSRP to including “timeRestrictionForChannelMeasurement” or not, no any modification.

HW: to compare the 3 samples between the current beam and new beam, if they are a little timed separated, we want to further discuss how to perform the comparation between current beam and new beam.

QC: M=1 is more typical and useful one for the feature. But it depends on the NW configured. Also OK to no requirement if timeRestrictionForChannelMeasurement is not configured

MTK: for the timeRestrictionForChannelMeasurement is not configured,

Apple: technically we agree. The measurement period is a total thing, during the measurement period, the UE can measure which occasion by UE implementation.

HW: when perform the comparison, if the difference between the channel condition is changed.

  - For P (sharing factor) L1_measurement periods:

    - Option 1: (Apple)

      - Separate Pc and Pn for current beam and new beam

    - Option 2: (Samsung)

      - reuse the legacy definition and requirements

Moderator: our understanding of the definition is the same as legacy. The difference it only whether the values are diff or the same.

QC: only focus on the measurement of detecting beam.

HW: what is the detecting beam mean: only new beam or? It cannot say it is only for new beam

QC: not just for the new beam. It depends on which beam will trigger the event reporting.

HW: in test case, it is possible to define which one is triggering; but for core, we cannot define the core based on just one beam.

Apple: It depends on how to include it in the core requirements. If as legacy, RAN4 just define the total measurement period, but no further definition of UE shall measure on which occasion.

MTK: if assume ssb based L1-RSRP for current beam and the new beam, the collision is the same as both current beam and new beam. For CSI-RS, FFS.

Agreement:

  - For T_RS (TSSB or TCSI-RS) in L1_measurement periods

    - T_RS (TSSB or TCSI-RS):

        - the value of current beam and new beams are the same.

  - For M in L1_measurement periods:

    - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam

    - If timeRestrictionForChannelMeasurement is not configured,

      - Further discuss on:

        - Option1: Reuse the same values of legacy, 3 samples for the measurement period is base on 3sample. No further restriction of UE implantation of which one is used

        - Option 2: No requirements even the NW can configure it.

        - Option 3: FFS

  - For P (sharing factor) in L1_measurement periods:

    - If it is SSB based, the definition and value are the same as legacy.

      - Note: check with Apple whether it is agreeable.

    - If it is CSI-RS based, FFS

- Issue 1-2: Components of Event triggered measurement reporting delay?

- Proposals

  - Proposal 1: (Qualcomm)

    - Delay uncertainty till the first PUCCH can be sent after the event has been identified by the UE and the case of no UL resources should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting.

    - The UE shall send the beam report in the next available PUCCH resource to ensure that the reporting delay is kept as small as possible.

  - Proposal 2: (Xiaomi)

    - For L1 reporting delay, didn’t consider the impact of L1 result after L1 reporting is triggered.

    - Fine with both “The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel” and reuse L3 definition “The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering”. For Option 1, the definition of Tfirst UL channel will be the time interval from L1-RSRP RS which trigger L1 reporting to first PUCCH.

  - Proposal 3: (Apple, LGE, Samsung, Ericsson, ZTE, MediaTek)

    - Measurement period + time to PUCCH

    - Proposal 3a: (Apple)

      - Time to PUCCH can be defined as time to first PUCCH resource or periodicity of PUCCH resource for UEIBM - Tfirst-PUCCH or TPUCCH

      - max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam } + Tfirst-PUCCH or TPUCCH

    - Proposal 3b: (LGE, MediaTek)

      - UL_uncertainty is the first available UL channel after the measurement period

    - Proposal 3c: (Samsung)

      - Tfirst UL channel as from time point which the RS of L1-RSRP trigger L1 reporting to first PUCCH to the time point of the latest PUCCH occasion

    - Proposal 3d: (Ericsson)

      - TTime to First UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time. Where, Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1

  - Proposal 4: (CMCC)

    - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on.

  - Proposal 5: (Nokia)

    - The UE shall be able to send the PUCCH latest at the first PUCCH occasion that is available after the maximum L1 event evaluation delay (i.e. TL1-RSRP_event triggered_measurement_Period).

- Recommended WF

  - To decided inside there two options:

    - Option 1: (Apple, Xiaomi, LGE, Samsung, Ericsson, ZTE, MediaTek)

      - The event triggered measurement reporting delay is TL1-RSRP_measure + Tfirst UL channel

    - Option 2: (Xiaomi, CMCC)

      - The event triggered measurement reporting delay shall be less than TL1-RSRP_measure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering.

    - Option 3: (Qualcomm)

      - Tfirst UL channel should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting

  - To discuss whether to add the definition in core requirements:

    - The UE shall be able to send the PUCCH latest at the first PUCCH occasion that is available after the maximum L1 event evaluation delay (i.e. TL1-RSRP_event triggered_measurement_Period).

Adhoc Discussion:

Agreement:

- Issue 1-5: CSI-RS based L1-RSRP for inter-cell BM

- Proposals

  - Proposal 1: (Qualcomm, Nokia, MediaTek (descoped))

    - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

  - Proposal 2: (Apple)

    - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

    - RAN4 defines UEIBR reporting delay for CSI-RS based measurement without specifying intra-cell or inter-cell.

  - Proposal 3: (Samsung)

    - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

  - Proposal 4: (CMCC, Huawei, Ericsson)

    - Define CSI-RS based L1-RSRP measurement requirements for both intra-cell and inter-cell.

  - Proposal 5: (Ericsson)

    - Use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

  - Proposal 6: (ZTE)

    - needs to identify whether additional clarifications on the following points are necessary:

      - -  Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

      - -  The prerequisite on L3 measurement on NSC.

    - use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Recommended WF

  - To decide in these three options:

    - Option 1: Not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management

    - Option 2: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

    - Option 3:

      - To define different condition and delay for inter-cell CSI-RS measurement including:

        - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

        - The prerequisite on L3 measurement on NSC.

        - Similar sharing factor as that in SSB for the sharing of intra-cell and inter-cell

Moderator’s observation: although some companies mentioned CSI-RS L1 measurement to support for intra-cell and inter-cell (Apple, Huawei, CMCC, Ericsson, ZTE, Samsung), I don’t think companies are in the same page. ZTE and Ericsson mentioned to add new sharing factor [and known condition definition] clearly. From Apple, Huawei, [CMCC], my understanding is the same requirements are transparent and same for both intra-cell and inter-cell, no need to add new sharing factor [and known condition definition]. Please feel free to modify the recommended WF.

Adhoc Discussion:

Agreement:

- Issue 1-6: Whether to specify new unified TCI switching delay requirements?

- Proposals

  - Proposal 1: (Qualcomm, Apple, Samsung (for known case), Ericsson)

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management.

  - Proposal 2: (Xiaomi, Nokia, LGE)

    - Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

  - Proposal 3: (ZTE)

    - Based on RAN1 progress, for the TCI state switching triggered by UEIBR(may or may not receive TCI state switching command), UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

  - Proposal 4: (MediaTek)

    - RAN4 to consider skipping the fine timing sync (additional SSB reception) for known TCI state if RAN1 has further agreement on the reduction of additional SSB reception for TCI state of UE-initiated/event-driven beam. Otherwise, reuse the existing unified TCI state delay requirement.

- Recommended WF

  - Check whether to agree:

    - For MAC-CE based DL/UL TCI state switch delay

      - Option 1: (Qualcomm, Apple, Samsung (for known case), Ericsson)

        - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

      - Option 2: (ZTE)

        - Remove components of TL1-RSRP and Tfirst-SSB+TSSB-proc

      - Option 3: (MediaTek)

        - FFS on whether to remove Tfirst-SSB+TSSB-proc

      - Whether cover cases: target TCI state is known and unknown

    - For DCI based DL/UL TCI state switch delay

      - Option 1:

        - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

      - Option 2:

        - Whether to further reduce the delay, wait for further RAN1 progress

Moderator’s suggestion: For MAC-CE based, if target TCI state is unknown, the delay is added by TL1-RSRP, the TL1-RSRP is defined in legacy NW configured L1-RSRP reporting. Please the proponents of the case if target TCI state is unknow, what is the definition of TL1-RSRP in the delay.

Adhoc Discussion:

Agreement:

- Issue 1-10: Capabilities for Support of Event Triggering and Reporting Criteria?

- Proposals

  - Proposal 1: (Apple, Ericsson, Nokia)

    - Defined by RAN1. To be discussed later if any RAN4 specific UE capabilities need to be added.

  - Proposal 2: (Samsung)

    - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

  - Proposal 3: (CMCC, MediaTek)

    - Define capabilities for support of UE-initiated/event-driven beam reporting similar as L3

Adhoc Discussion:

Agreement: